3GPP TSG-SA5 Meeting #146 	S5-226132

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0214 | rev | - | Current version: | 18.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add stage 3 for data type AvailabilityStatus |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | AvailabilityStatus defined in ITU-T X.731 are not defined in TS28.623. AvailabilityStatus are necessary for network slice LCM, in the IOCs NetworkSliceController and NetworkSliceSubnetController defined in TS28.541. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 3 for data type AvailabilityStatus is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of shared objects as well as asynchronous or long running operations where the the consumer requests the allocation of an instance of an object by providing the requirements are not supported. Without a common/generic mechanism defined, solution to solve this issue in different use cases will not be uniform. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.541 CR 0800 TS28.531 CR 0145 TS28.622 CR 0179 TS28.623 CR 0201 |  |  |
| Other comments: |  | - Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/tree/TS28.623_Rel-18_CR0214_Add_stage_3_for_data_type_AvailabilityStatus - IOCs NetworkSliceController and NetworkSliceSubnetController are defined in TS 28.541 CR 0800 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## C.4.2a	OpenAPI document "TS28623_ComDefs.yaml"

openapi: 3.0.1

info:

title: Common Type Definitions

version: 17.1.0

description: >-

OAS 3.0.1 specification of common type definitions in the Generic NRM

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM; Common type definitions

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

Float:

type: number

format: float

DateTime:

type: string

format: date-time

Latitude:

type: number

format: float

minimum: -90

maximum: 90

Longitude:

type: number

format: float

minimum: -180

maximum: 180

Dn:

type: string

DnList:

type: array

items:

$ref: '#/components/schemas/Dn'

Mcc:

type: string

pattern: '^[0-9]{3}$'

Mnc:

type: string

pattern: '^[0-9]{2,3}$'

Nid:

type: string

pattern: '^[A-Fa-f0-9]{11}$'

PlmnId:

type: object

properties:

mcc:

$ref: '#/components/schemas/Mcc'

mnc:

$ref: '#/components/schemas/Mnc'

Tac:

type: string

pattern: '(^[A-Fa-f0-9]{4}$)|(^[A-Fa-f0-9]{6}$)'

UtraCellId:

type: integer

EutraCellId:

type: string

pattern: '^[A-Fa-f0-9]{7}$'

NrCellId:

type: string

pattern: '^[A-Fa-f0-9]{9}$'

TimeWindow:

type: object

properties:

startTime:

$ref: '#/components/schemas/DateTime'

endTime:

$ref: '#/components/schemas/DateTime'

GeoCoordinate:

type: object

properties:

latitude:

$ref: '#/components/schemas/Latitude'

longitude:

$ref: '#/components/schemas/Longitude'

ConvexGeoPolygon:

type: array

items:

$ref: '#/components/schemas/GeoCoordinate'

GeoAreaToCellMapping:

type: object

properties:

convexGeoPolygon:

$ref: '#/components/schemas/ConvexGeoPolygon'

associationThreshold:

type: integer

AreaOfInterest:

oneOf:

- $ref: '#/components/schemas/GeoAreaToCellMapping'

- type: array

items:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/Tai'

- type: array

items:

$ref: '#/components/schemas/NrCellId'

- type: array

items:

$ref: '#/components/schemas/EutraCellId'

- type: array

items:

$ref: '#/components/schemas/UtraCellId'

Fqdn:

type: string

Ipv4Addr:

type: string

pattern: '^(([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])\.){3}([0-9]|[1-9][0-9]|1[0-9][0-9]|2[0-4][0-9]|25[0-5])$'

example: '198.51.100.1'

Ipv6Addr:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))$'

example: '2001:db8:85a3::8a2e:370:7334'

Ipv6Prefix:

type: string

allOf:

- pattern: '^((:|(0?|([1-9a-f][0-9a-f]{0,3}))):)((0?|([1-9a-f][0-9a-f]{0,3})):){0,6}(:|(0?|([1-9a-f][0-9a-f]{0,3})))(\/(([0-9])|([0-9]{2})|(1[0-1][0-9])|(12[0-8])))$'

- pattern: '^((([^:]+:){7}([^:]+))|((([^:]+:)*[^:]+)?::(([^:]+:)*[^:]+)?))(\/.+)$'

example: '2001:db8:abcd:12::0/64'

IpAddr:

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

HostAddr:

#  This definition will be deprecated, when all occurances of HostAddr

#  are replaced by Host.

oneOf:

- $ref: '#/components/schemas/Ipv4Addr'

- $ref: '#/components/schemas/Ipv6Addr'

- $ref: '#/components/schemas/Fqdn'

Host:

oneOf:

- $ref: '#/components/schemas/IpAddr'

- $ref: '#/components/schemas/Fqdn'

Uri:

type: string

AdministrativeState:

type: string

enum:

- LOCKED

- UNLOCKED

OperationalState:

type: string

enum:

- ENABLED

- DISABLED

UsageState:

type: string

enum:

- IDEL

- ACTIVE

- BUSY

AvailabilityStatus:

type: string

enum:

- IN_TEST

- FAILED

- POWER_OFF

- OFF_LINE

- OFF_DUTY

- DEPENDENCY

- DEGRADED

- NOT_INSTALLED

- LOG_FULL

AttributeNameValuePairSet:

description: >-

The key of this map is the attribute name, and the value the attribute value.

type: object

minProperties: 1

additionalProperties:

nullable: true

AttributeValueChangeSet:

description: >-

The first array item contains the attribute name value pairs with the new values,

and the second array item the attribute name value pairs with the optional old values.

type: array

items:

$ref: '#/components/schemas/AttributeNameValuePairSet'

minItems: 1

maxItems: 2

Filter:

description: >-

The filter format shall be compliant to XPath 1.0.

type: string

SystemDN:

type: string

NotificationId:

type: integer

NotificationType:

oneOf:

- $ref: 'TS28532_FaultMnS.yaml#/components/schemas/AlarmNotificationTypes'

- $ref: 'TS28532_ProvMnS.yaml#/components/schemas/CmNotificationTypes'

- $ref: 'TS28532_PerfMnS.yaml#/components/schemas/PerfNotificationTypes'

- $ref: 'TS28532_HeartbeatNtf.yaml#/components/schemas/HeartbeatNotificationTypes'

- $ref: 'TS28532_FileDataReportingMnS.yaml#/components/schemas/FileNotificationTypes'

NotificationHeader:

type: object

properties:

href:

$ref: '#/components/schemas/Uri'

notificationId:

$ref: '#/components/schemas/NotificationId'

notificationType:

$ref: '#/components/schemas/NotificationType'

eventTime:

$ref: '#/components/schemas/DateTime'

systemDN:

$ref: '#/components/schemas/SystemDN'

required:

- href

- notificationId

- notificationType

- eventTime

- systemDN

ErrorResponse:

description: >-

Default schema for the response message body in case the request

is not successful.

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

End of change