3GPP TSG SA WG5 Meeting #146		S5-226408

# Toulouse (FR), 14-18 November 2022

# Source:	Telefónica S.A.

# Title:	Solution for intent-driven management to deliver a network slice

# Document for:	Approval

Agenda Item:	6.7.4.2

# 1	Decision/action requested

Discuss and approve on the proposal.

# 2	References

[1]	3GPP TR 28.836: “Study on intent-driven management for network slicing”.

[2]	3GPP TS 28.312: “Management and orchestration; Intent driven management services for mobile networks”.

[3]	3GPP TS 28.531: “Management and orchestration; Provisioning”.

[4]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and 3”.

# 3	Rationale

Clause 5.1.1 from 3GPP TS 28.836 [1] describes the use case of intent-driven management to deliver a network slice instance.

In 3GPP TS 28.312[2], clause 4.2.2, it is stated that intent-driven MnS producer have the following capabilities:

- Validate the intent

- Translate the received intent to executable actions as follows:

  - Performing service or network management tasks

  - Identifying, formulating and activating service or network management policies

- Evaluate the result/information about the intent fulfilment (e.g., the intent is initially satisfied or not) and intent assurance (e.g., the intent is continuously satisfied).

This contribution proposes a solution for “intent-driven management to deliver a network slice instance” use case [1], by making use of the capabilities listed above [2].

# 4				Detailed proposal

This document proposes the following changes in TR 28.836 [1].

1st Change

# 6	Potential solutions

## 6.x 	Solution for intent-driven management to deliver a network slice instance

### 6.x.1	Description

This potential solution describes the possible actions of an intent-driven MnS to deliver a network slice instance for the phase of the intent deployment. The intent driven MnS allows a consumer to declare the desired intent following information model defined in 3GPP TS 28.312 [2] and implements a number of intent lifecycle management functionalities to reach a successful deployment of the intent, given by the availability of a network slice instance capable to fulfil the intent expectation.

### 6.x.2	Details

Figure 6-x represents the system components for the intent-driven management procedure to deliver a network slice instance. In the proposed solution:

- The Intent Owner belongs to the CSC administrative domain, and acts as intent-driven MnS consumer.

- The Intent Handler belongs to the CSP administrative domain, and acts as intent-driven MnS producer.

- The intent issued by the Intent Owner and fulfilled by the Intent Handler belongs to the Intent-CSC category. For further details on the different intent categories, see 3GPP TS 28.312 [2], clause 4.1.2.

Figure 6-x: Potential solution for intent-driven management to deliver a network slice

The Intent Owner issues an intent to the Intent Handler. The intent is initially validated by the Intent Validation module, which verifies (i) the compliance of the intent format with the information model defined in 3GPP TS 28.312 [2]; and (ii) the consistency among the intent expectations that constitute the whole intent.

Once validated, the intent is sent over to the Intent Translation module, which is in charge of translating the intent semantics into a number of executable actions. In this case, the executable actions consist of provisioning and activation of one or more network slice instances, re-using one or more existing network slice instances, re-configuring one or more existing network slice instances or a combination of them. To trigger the resulting executable actions, the Intent Translation module consumes management services produced by the network slice provisioning service [3].

NOTE: In case of new or updated network slices, the ServiceProfile [4] is also elaborated by the Intent Translation module on the basis of the intent expectations.

Finally, after the enforcement of executable actions, there is a need to verify whether the intent is fulfilled or not. In this vein, the Intent Fulfilment Evaluation module is defined. This module makes use of information retrieved from network slice provisioning service, and computes a validation result. This result is included in the intent report which is sent back to the intent owner.

### 6.x.3	Evaluation

The proposed solution splits the capabilities of the intent-driven MnS producer (see 3GPP TS 28.312 [2], clause 4.2.2) in dedicated modules that interacts together to implement the whole action of the intent deployment applied to the delivery of a network slice instance.

The translation module may use a rule-based mechanism to translate the intent expectations into ServiceProfile [4] constructions. More elaborate solutions may involve AI/ML based mechanisms to improve the quality of the translation decisions.

The intent fulfilment evaluation module may use a rule-based mechanism to evaluate whether the expectations of the intent are satisfied or not. In case of failed validation, closed loop mechanisms potentially assisted by AI/ML based mechanisms may be adopted to suggest countermeasures that the intent deployment module may apply in the context of closed loop network automation.

End of change