# 3GPP TSG-SA5 Meeting #146e	S5-226412

# 14 Nov to 18 Nov 2022, E-meeting

# Source:	China Telecom

# Title:	Add the evaluation for KI#1

# Document for:	Approval

# Agenda Item:	6.7.6.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-040: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

In the previous meetings, the key issue # 1 for NWDAFFunction IOC enhancement to support multiple NWDAF deployment has been studied, and the potential solution has been approved.

In this contribution, we propose to evaluate the key issue #1.

4	Detailed proposal

Start of 1st Change

4.1.X	Evaluation

There is one potential solution for the NWDAFFunction IOC enhancement to support Multiple NWDAF Deployment.

In the case where multiple NWDAF instances are deployed, the NWDAFs can be deployed as different NWDAF instances with different capabilities. In the potential solution #1, the attribute nWDAFAggregationCapabilityId is defined to indicate whether the NWDAF has the analytics aggregation capability, and the attribute nWDAFMetadaProvCapabilityId is defined to indicate whether the NWDAF has the analytics metadata provisioning capability. This solution can be used to distinguish the NWDAF instances based on the NWDAF capability in multiple NWDAF deployment, so that the management can be performed accordingly.

Consequently, this potential solution can satisfy the requirements to enhance the NWDAFFunction IOC to support multiple NWDAF deployment. This solution is a feasible candidate and may move to the normative phase as solution for NWDAFFunction IOC enhancement to support multiple NWDAF deployment.

End of Modified Sections