# 3GPP TSG-SA5 Meeting #146e	S5-226414

# 14 Nov to 18 Nov 2022, E-meeting

# Source:	China Telecom

# Title:	Add the evaluation for KI#3

# Document for:	Approval

# Agenda Item:	6.7.6.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-040: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

In the previous meetings, the key issue #3 for the performance measurement of the NWDAF on the interaction aspect has been studied and some potential solutions have been approved.

In this contribution, we propose to add the evaluation for the key issue #3.

4	Detailed proposal

Start of 1st Change

### 4.3.X	Evaluation

There are five potential solutions for the performance measurement of the NWDAF on the interaction aspect.

In the potential solution #1, the performance measurements of NWDAF are defined as the number of subscriptions and/or requests received by NWDAF. In the potential solution #2, the performance measurements of NWDAF are defined as the number of notifications and/or responses generated by NWDAF.

In the potential solution #3, the performance measurements of Aggregator NWDAF are defined as the number of subscriptions and/or requests invoked by the Aggregator NWDAF to subscribe and/or request the analytics information for analytics aggregation. In the potential solution #4, the performance measurements of Aggregator NWDAF are defined as the number of notifications and/or responses received by Aggregator NWDAF to get analytics information for analytics aggregation.

The potential solution #1 and #2 can support the performance measurement of the NWDAF for performing the normal task (i.e., the analytics services). The potential solution #3 and #4 can support the performance measurement of the Aggregator NWDAF for performing the extra tasks (i.e., subscribe/request the analytics information from the other NWDAFs and aggregate the analytics output) compared to the normal task. These proposed performance measurements can indicate how busy the NWDAF is on analytics related services.

Consequently, these four potential solutions can satisfy the requirements to provide the performance measurement of the NWDAF on the interaction aspect. These four potential solutions are feasible candidates and may move to the normative phase as solutions for the performance measurement of the NWDAF performing the normal task and extra task.

Editor Notes: The Editor’s note in the potential solution #5 is not removed. So the potential solution #5 can be evaluated afther the Editor’s note removed.

End of Modified Sections