3GPP TSG-SA5 Meeting #146 	S5-226428

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Adding a solution using separate CHF defined max sessions

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 32.847.

# 2	References

[1]		3GPP TR 32.847: "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

Adding a solution using separate CHF defined max sessions.

# 4	Detailed proposal

First change

### 6.1.x	Solution#2.x New NSACF (CTF) – separate CHF configured max

#### 6.1.x.1	General Description

This solution uses a solution#1.1 and solution#1.3 based on Network Slice quota management between the CHF and the NSACF, session based charging and intermediate thresholds.

The general description is the same as clause 6.1.1.1, with the following differences:

- the "CHF max number of PDU sessions" correspond to the "max Nb Reg UEs" stored in the CCS

- the "NSACF max number of PDU sessions" correspond to the "max Nb Reg UEs" stored in the NSACF

- the "NS PDU sessions Quota limit" is always below the "NSACF max number of PDU sessions";

The NSACF is configured at provisioning of the network slice, with "NSACF max number of PDU sessions" and intermediate "number of PDU sessions" thresholds below the "NSACF max number of PDU sessions" for triggering the CHF.

The CHF is configured at provisioning of the network slice, with "CHF max number of PDU sessions" which is below or equal to the "NSACF max number of PDU sessions". When the "CHF max number of PDU sessions" is met the CHF will reject any request from NSACF increasing the number of PDU sessions.

NS_quota of "number of PDU sessions" for S-NSSAI is determined by CCS: set to the "number of PDU sessions", or to a lower value.

- during the charging session, the CHF may provide new intermediate thresholds which override existing ones, as described in solution #1.3, The thresholds for termination of solution #1.3 corresponds to "NS PDU sessions Quota limit".

#### 6.1.x.2	Architecture Description

Same architecture as clause 6.1.2.4.

#### 6.1.x.3	Flow description

The flow and steps are similar as in clause 6.1.3.5 apply with CCS control of "NS PDU sessions Quota limit".

End of changes