3GPP TSG-SA5 Meeting #146 	S5-226438

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Adding solution for SMS in clause 7.2

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

[2]		3GPP TS 23.540: "5G System: Technical realization of Service Based Short Message Service "

[3]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[4]	3GPP TS 32.240: "Charging management; Charging architecture and principles”.

# 3	Rationale

Adding solutions in clause 7.1 for VPLMN wholesale charging of HPLMN of SMS based on CDR generation in VPLMN. The current handling of SBI-based SMS is specified in TS 23.540 [2] and it reuses the SMS-SC as specified in TS 23.040 [3]. For MT SMS the SMS-SC interaction with the SMS-GMSC and for MO SMS it interacts with the SMS-IWMSC both still referring to the TS 23.040 [3] for the interaction. TS 32.240 [4] annex E,

# 4	Detailed proposal

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501:"System Architecture for the 5G System".

[3]	GSMA TD.201: "Common Billing and Charging Processes ".

[4]	3GPP TS 32.255:" Charging management;5G Data connectivity domain charging; stage 2".

[5]	3GPP TS 32.256:" Charging management; 5G connection and mobility domain charging; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[7]	GSMA WA.51: "5G SA Implementation Guidelines".

[8]	3GPP TS 32.274: " Charging management; Short Message Service (SMS) charging".

[9]	3GPP TS 32.256: "Charging management; 5G connection and mobility domain charging; Stage 2".

[10]		3GPP TS 23.540: "5G System: Technical realization of Service Based Short Message Service "

[11]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[12]	3GPP TS 32.240: "Charging management; Charging architecture and principles”.

Second change

#### 7.2.4.x	Solution #1.x: CDF and OCF communicating with SMS-SC for retail charging of SMS

##### 7.2.4.x.1	General

A possible solution for key issues #2f and #2g covering requirements REQ-CH_CVTOH-03, and REQ-CH_CVTOH-04, wholesale charging for SMS provided to the home MNO by the visited MNO. In this case the home MNO uses charging architecture defined in TS 32.240 [12] annex E, to generate charging information.

##### 7.2.4.x.2	Reference architecture

Figure 7.1.4.x.2-1: Roaming SMS scenario in service-based interface representation

Figure 7.1.4.x.2-2: Roaming SMS scenario in reference point representation

##### 7.1.4.x.3	Message flows

The flows for SMS charging would be the same as in TS 32.274 [8], that are based on the interactions specified in TS 23.540 [10] and TS23.040 [11].

End of changes