3GPP TSG-SA5 Meeting #146 	S5-226439

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Updating solutions 2.10 and 2.11

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

The solutions #2.10 #2.11 are only applicable for key issue #2e since the describe how the trigger could be handled for CHF to CHF interaction.

# 4	Detailed proposal

First change

#### 7.2.4.10	Solution #2.10: UE triggered V-CHF and H-CHF communication

##### 7.2.4.10.1	General

A possible solution for key issue #2e, would be to have the trigger for when the V-CHF should trigger a charging session with the H-CHF based on that the V-CHF gets a charging data request containing a SUPI that the V-CHF don’t have a session with a H-CHF setup.

##### 7.2.4.10.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.10.3	Message flows

In this case, the SMF, AMF, and SMSF establish the charging session and events with the V-CHF as specified in the TS 32.255 [4], TS 32.256 [5], and TS 32.274 [8]. The V-CHF would establish the charging session with the H-CHF based on the SUPI i.e., on a per UE basis.

Editor’s note: the message flows for the UE based solution are FFS

Second change

#### 7.2.4.11	Solution #2.11: Charging data request triggered V-CHF and H-CHF communication

##### 7.2.4.11.1	General

A possible solution for key issue #2e, the trigger for interaction for V-CHF communicating with H-CHF is based on the reception of a charging data request by the V-CHF.

##### 7.2.4.11.2	Reference architecture

The reference architecture would be the same as in solution #2.1 clause 7.2.4.1.

##### 7.2.4.11.3	Message flows

In this case V-CHF, as the NF consumer (CTF) towards the H-CHF, triggers a charging data request based on the reception of a charging data request from another NF consumer (CTF). The V-CHF (CTF) to may filter or convert the trigger from the V-CHF’s NF consumer (CTF) i.e., for 5G data connectivity charging the V-CHF may use either FBC or QBC triggers to trigger a charging data request towards the H-CHF, 5G connection and mobility charging a ECUR may be converted to either IEC or PEC, and Short Message Service (SMS) charging a ECUR may be converted to either IEC or PEC.

End of changes