3GPP TSG-SA5 Meeting #146 	S5-226447

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Discussion paper on comparison between double or single CHF

# Document for:	Endorsement

Agenda Item:	7.5.3

# 1	Decision/action requested

Agree detail proposal in section 4.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

There are basically three different potential architectures, where one CTF (AMF, SMF and SMSF) have for one chargeable event/session:

A.	two charging events/sessions, one to each of V-CHF and H-CHF.

B.	two charging events/sessions to V-CHF, the V-CHF forwards one of these to H-CHF

C.	one charging events/sessions to V-CHF, the V-CHF creates one new charging event/session to H-CHF

There is a fourth variant (D) which is a mix between B and C, where the CTF would only have one chargeable/event session towards the V-CHF but would sperate the information sent to V-CHF and the information to be forwarded to the H-CHF.

For potential architecture A the service used would be the Nchf_ConvergedCharging for both charging events/sessions, while for potential architectures B and C this doesn’t have to be the case. The comparison assumes that the Nchf_ConvergedCharging is used for all potential architectures i.e., between CTF and CHFs as well as between CHFs.

All potential architectures add latency, how much will to a large extent depend on the interaction with the HPLMN. Using the non-blocking mode could mitigate it to some extent but have other cons.

# 4	Detailed proposal

Comparison of the four different potential architectures, where one CTF (AMF, SMF and SMSF) have for one chargeable event/session:

A.	two charging events/sessions, one to each of V-CHF and H-CHF.

B.	two charging events/sessions to V-CHF, the V-CHF forwards one of these to H-CHF

C.	one charging events/sessions to V-CHF, the V-CHF creates one new charging event/session to H-CHF

D.	one chargeable/event session towards the V-CHF, but would sperate the information that should be sent H-CHF

Table 4-1: Comparison potential architectures

| Proposal | Pros | Cons |
| --- | --- | --- |
| A | - Already specified for some NFs - From H-CHF view like home routed roaming - From V-CHF view like home routed roaming | - Additional CTF handling - NF and vendor specific failure handling - No support of more than two CHFs, if required - Synchronization of rating groups |
| B | - Could reuse what is specified for A - From H-CHF view like home routed roaming - Allows the VPLMN to monitor what the HPLMN is receiving and requesting | - Additional CTF handling - NF and vendor specific failure handling - Synchronization of rating groups - Using V-CHF as a proxy/routing agent |
| C | - From H-CHF view like home routed roaming - From CTF view like home routed roaming (simplification) - V-CHF could be used to translate rating groups - Allows the VPLMN to monitor what the HPLMN is receiving and requesting - Allows the VPLMN to influence what the HPLMN is receiving and requesting - Supports more than two CHFs | - Additional speciation - Additional handling in V-CHF i.e., the CHF needs to be able to act as a Nchf consumer |
| D | See B, except for the possibility to reuse what is specified for A |  |