3GPP TSG-SA5 Meeting #146                              S5-226458

# Toulouse, France, 14 - 18 November 2022

# Source:	AsiaInfo

# Title:	pCR 28.834 Add potential solution for VNF package update of the cloud-native VNF

# Document for:	Approval

Agenda Item:	6.8.5.3

1	Decision/action requested

For approval

2	References

[1]	3GPP TR 28.834 v0.3.0 Study on Management of Cloud Native Virtualized Network Functions

3	Rationale

This contribution describes the potential solution which relate to use case #6 in [1].

4	Detailed proposal

The flowing changes are proposed to [1].

1nd change

5.6	Use case# 6: VNF package update of the cloud-native VNF

5.6.1	Description

As a pre-condition to this use case, the network is running normally. The 3GPP management system has subscribed to VNF instance lifecycle notifications from VNFM.

The use case begins when the 3GPP management system decides to update VNF package for a VNF instance.

The 3GPP management system requests VNFM to change current VNF package (as defined in clause 7.2.24 ETSI GS NFV-IFA 008 [8]).

VNFM inspects the current and target VNF Descriptor (as defined in clause 7.1.15 ETSI GS NFV-IFA 011[7]) to differentiate possible changing values that concern to some VNF component (e.g. VDU, internal VLD, etc.) or property (e.g. a Scaling Aspect, etc.).

The VNFM will change of both the VNF software version and the VNF virtualised resources , including terminating the virtualised resource instances running the current software version and instantiating new virtualised resource instances with the target VNF software version.

When the VNF software version and the VNF virtualised resources have been changed, VNFM sends a notification to the 3GPP management system to notify that the VNF package has been updated. This notification includes information on the changes to VNF package.

5.6.2		Requirements

REQ-CVNF_LCM_CON-1  The 3GPP management system shall be able to send a request for VNF package update of the cloud-native VNF to VNFM

REQ-CVNF_LCM_CON-2  The 3GPP management system shall be able to receive notification from VNFM about VNF package update of cloud-native VNF.

5.6.3		Potential solution

The 3GPP management system invokes the UpdateNsRequest operation (see clause 7.3.5 in ETSI GS NFV-IFA 013 [9]) to request NFVO via the Os-Ma-nfvo interface to update VNF package of the cloud-native VNF. When the NFVO receives the UpdateNsRequest for VNF package update of cloud-native VNF from the 3GPP management system, the NFVO sends the NS Lifecycle Change notification to the 3GPP management system indicating the result of VNF package update of cloud-native VNF(see clause 7.3.12 of ETSI GS NFV-IFA 013 [6]).

End of changes