3GPP TSG-SA5 Meeting #146 	S5- 226512

# Toulouse, France, 14 -18 November 2022

# Source:	China Unicom

# Title:	Add potential solution for issue on configuration of reliability in slice profiles

# Document for:	Approval

Agenda Item:	6.8.3

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.832 v0.3.0: “Management Aspects of URLLC”

3	Rationale

It was proposed to add potential solution for issue on configuration of reliability in slice profiles. A solution for configuration management related to URLLC is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains; Stage 1”

[3]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”

[4]	3GPP TS 38.211: “NR; Physical channels and modulation”

[5]	3GPP TS 38.212: “NR; Multiplexing and channel coding”

[6]	3GPP TS 38.213: “NR; Physical layer procedures for control”

[7]	3GPP TS 38.214: “NR; Physical layer procedures for data”

[8]	3GPP TS 38.300: “NR; NR and NG-RAN Overall Description; Stage2”

[9]	3GPP TS 38.321: “NR; Medium Access Control (MAC) protocol specification”

[10]	3GPP TS 38.323: “NR; Packet Data Convergence Protocol (PDCP) specification”

[11]	3GPP TR 38.824: “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”

[12]           3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3"

2nd Change

5.X	Issue #X:  Configiration of reliability in slice profiles

5.X.2	Potential solution

1. The ServiceProfile <<dataType>> defined in TS 28.541[12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | F | T | T |
| uLReliability | O | T | F | F | T |

2. The CNSliceSubnetProfile<<dataType>> defined in TS 28.541[12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | F | T | T |
| uLReliability | O | T | F | F | T |

3. The RANSliceSubnetProfile<<dataType>> defined in TS 28.541[12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | F | T | T |
| uLReliability | O | T | F | F | T |

4. The TopSliceSubnetProfile<<dataType>> defined in TS 28.541[12] shall add dLReliability and uLReliability attributes. The original reliability attribute shall be replaced by these two new attributes.

The dLReliability and uLReliability attributes shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dLReliability | O | T | F | T | T |
| uLReliability | O | T | F | F | T |

5. The dLReliability and uLReliability attributes shall be defined in TS 28.541[12] as follows.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| dLReliability | An attribute specifies in the context of network layer DL packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [28] and TS 22.104 [51]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| uLReliability | An attribute specifies in the context of network layer UL packet transmissions, percentage value of the amount of sent network layer packets successfully delivered to a given system entity within the time constraint required by the targeted service, divided by the total number of sent network layer packets, see TS 22.261 [28] and TS 22.104 [51]. | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |

End of Changes