3GPP TSG-SA5 Meeting #146 	S5- 226528

# Toulouse, France, 14 -18 November 2022

# Source:	China Unicom

# Title:	Add potential solution for issue on PLMN granularity requirement of performance measurements for MOCN

# Document for:	Approval

Agenda Item:	6.8.6

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.4.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

[2]      3GPP TS 28.552: "5G performance measurements".

3	Rationale

It was approved to add potential solution for issue on PLMN granularity requirement of performance measurements for MOCN. This contribution proposes to add PLMN-granularity for some performance measurements defined in TS 28.552[2].

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.X	Issue #X:  PLMN granularity requirement of performance measurements for MOCN

5.X.2	Potential solution

1. At NRCellCU, some performance measurements defined in TS 28.522 shall add PLMN granularity.

The PLMN granularity shall be added in the following performance measurements:

| Category | Measurement Name |
| --- | --- |
| RRC connection number | Max number of stored inactive RRC Connections |
| PDU Session Management | Number of PDU Sessions requested to setup |
|  | Number of PDU Sessions successfully setup |
|  | Number of PDU Sessions failed to setup |
| DRB related measurements | Mean number of DRBs being allocated |
|  | Peak number of DRBs being allocated |
| QoS flow setup | Number of QoS flow attempted to setup |
|  | Number of QoS flow successfully established |
|  | Number of QoS flow failed to setup |
| QoS flow modification | Number of QoS flows attempted to modify |
|  | Number of QoS flows successfully modified |
|  | Number of QoS flows failed to modify |
| QoS flow release | Number of released active QoS flows |
| 5QI 1 QoS Flow Duration Monitoring | Average Normally Released Call (5QI 1 QoS Flow) Duration |
|  | Average Abnormally Released Call (5QI 1 QoS Flow) Duration |
|  | Distribution of Normally Released Call (5QI 1 QoS Flow) Duration |
|  | Distribution of Abnormally Released Call (5QI 1 QoS Flow) Duration |

2. At NRCellDU, some performance measurements defined in TS 28.522 shall add PLMN granularity.

The PLMN granularity shall be added in the following performance measurements:

| Category | Measurement Name |
| --- | --- |
| Number of Active Ues | Max number of Active UEs in the UL per cell |
| Paging Measurement | Number of paging records received by the NRCellDU |
|  | Number of paging records discarded at the NRCellDU |

End of Change