3GPP TSG-SA5 Meeting #146 	S5-226567

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.530 | CR | 0054 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add network slice isolation use case and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSRULE |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | GSMA PRD NG.135 (v1.0, 21 July 2022) defines isolation for network network slicing (clause 3.1.7 of GSMA PRD NG.135). The existing 3GPP management system capabilities cannot support the use cases and requirements, which implied not only granular isolation requirements for network slice, but also flexible inclusion/exclusion rules between network slices (e.g. group network slices with same isolation requirement and security level together, and isolate the network slices in the group from network slices in other groups). This contribution introduces use case and requirements for network slice isolation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Network slice isolation use case and requirements added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network slice isolation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x (new), 5.1.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.541 CR 0799 |  |  |
| Other comments: |  | - Network slice isolation NRM is defined in CR0799 for TS 28.541 - Additional reference: Network slice isolation has been studied and documented in TR28.811 in the following clause: 5.3 Use case – Isolated deployment of multiple network slices of same customer 5.7 Use case – management data isolation for different network slice customers 6.2 Potential requirements for Network Slicing Management to support network slice isolation 6.6 Potential requirements for Network Slicing Management to support management data isolation for different NSCs 7.1 Possible solutions for network slice isolation 7.6 Possible solutions for management data isolation for different NSCs |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-226107 is a revision of S5-225156 - S5-226567 is a revision of S5-226107 |  |  |  |  |  |  |  |  |

1st Change

## 4.x	Network slice isolation

Isolation is one of the key expectations of network slicing. A NetworkSlice instance may be fully or partly, logically and/or physically, isolated from another NetworkSlice instance. Network slicing allows the concurrent execution of multiple network slice instances on top of a common infrastructure, satisfying their individual service SLS while guaranteeing their independence. The use of a single shared infrastructure for this makes isolation a key requirement in the support of network slicing. Isolation can be defined as the ability of a NSP to ensure that congestion, attacks and lifecycle-related events (e.g. scaling in/out) on one network slice instance does not negatively impact other existing network slice instances.

The isolation of slices includes isolation of management resource (e.g. data, functions or services) and managed resource (e.g. data, network functions, infrastructure resource, etc.) in multiple domains (e.g. RAN, Core, Transport, etc.). The resource of a slice can be dedicated for the slice or shared with other slices.

The NOP can define isolation groups (see IsolationGroup IOC defined in TS 28.541[6]) , with isolation policies. The isolation policies define the isolation rules for each management data (e.g. CM, PM, FM, MDT, QoE, trace data, etc) and managed resources (e.g. NF, virtual resource, virtual infrastructure etc).  The isolation group can be based on tenant, NSC, SST, etc.

The NOP can configure the isolation groups with isolation policies that defines the isolation requirements. NetworkSlice instance or NetworkSliceSubnet instances that share same isolation requirements are associated with the isolation group that defines the isolation rules.

### 4.x.2	Isolation of managed resources

Managed resource includes network function, virtual resources, physical resources, virtual infrastructure etc.

Network slices could be grouped together, and isolated from other group of the network slices. Therefor the isolation requirement may not only apply to a single network slice, but also a group of slices. E.g. in below picture, different type of Network Slices were separated to different isolation groups.

Figure 4.x.1-1 Isolation between Network Slices of different SSTs

In another more complicated example, the network slices of one tenant, especially for some sensitive services, could be physically isolated from slices of other tenants. In addition, inside one tenant, the slices of different SSTs could be further logically isolated with each other.

Figure 4.x.1-2 Isolation between Network Slices of different tenants and SSTs

### 4.x.1	Isolation of management data

Management data includes business related data, e.g., charging information, SLA, tenant profile, subscriber data, etc., and operation related data, e.g. CM, PM, FM data, log, trace, MDT, QoE, policy, analytics report, software, etc.

The data can be in different stages:

-	At rest, data that is housed on computer data storage (e.g. hard disk, CD, mobile, etc.) in any digital form (e.g. file, databases, data warehouses, etc.). e.g. management data stored in central or distributed database, etc. The data at rest could be "stolen" by illegal entities who can gain physical or digitally access to the device or data storage media. Isolating storage, access control and data encryption are always employed to protect data at rest.

-	In transit, data that is moving between nodes over public or private network. e.g. management data exchanged between customer tool and NSMF, between digital portal and NSMF/NSSMF/NFMF, between MnF and MnF/NF, etc. The data in transit could be leaked or tampered by any probe or tap over wire, or any network nodes in the flow, when it leaves the confines of protected enclaves, especially when it flows over public network. Security protocols, e.g. TLS, IPSec, etc., are always applied to isolate and protect data in transit.

-	In use: active data that is manipulated by an application or process, and is residing typically in computer random-access memory (RAM), CPU caches, or CPU registers. e.g. management data used in MDA, etc. Unlike data at rest or in transit, the data in use is always decrypted to be used by process in most of operation systems today, therefore it can be stolen through e.g. memory dump, side channel attack, etc. The data in use could be protect by isolating applications in different VM or container with trusted host operation system and hardware.

As data is critical asset of a network slice customer (NSC) for its business and operation, in GSMA GST, the data of one NSC is always required to isolate from other NSCs even for the least strict isolation level - service/tenant isolation.

Management data isolation is needed in all stages of the data during its lifecycle to satisfy SLS of network slice customer (NSC, or tenant), security policies of network slice provider (NSP) and regional/industry regulations. Different NCSs may have different requirements on data isolation and protection and a single NSC may also have different data isolation and protection requirements for different types or stages of data. E.g. A NSC from public safety may require dedicated MnF to collect and transmit its management data via IPSec based VPN, while a NSC as media content provider may accept shared MnF but separated MnS and transport to collect and transmit its PM/FM data. Another example, for a NSC of vehicle industry, it may require to have separate DB and protect anonymization for its MDT data, while just separate table for FM/PM data with availability and integrity protection.

2nd Change

### 5.1.x	Network slice isolation requirements

-	REQ-NSISOL-1 The 3GPP management system shall have the capability to support provisioning of isolation policies for management data and managed resource.

-	REQ-NSISOL-2 The 3GPP management system shall have the capability to support different isolation and protection requirements for different types of management data and managed resources.

-	REQ-NSISOL-3 The 3GPP management system shall have the capability to support isolation of managed resources for different network slice customers or different network slice groups.

-	REQ-NSISOL-4 The 3GPP management system shall have the capability to support management data isolation for different network slice customers or different network slice groups.

-	REQ-NSISOL-5 The 3GPP management system shall have the capability to support authorized network slice or network slice subnet related management service consumer to query the available isolation groups for the network slice or network slice subnet to be deployed.

-	REQ-NSISOL-6 The 3GPP management system shall have the capability to support management data and managed resource isolation in network slice management, network slice subnet management and network function management layers.

-	REQ-NSISOL-7 The 3GPP management system shall have the capability to support management data isolation when data in collection, in transmission, in rest and in use.

End of change