3GPP TSG SA WG5 Meeting #146	S5-226568

# Toulouse, France, 14-18 November 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0215 | rev |  | Current version: | 18.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Definition of parameters MDT Alignment Information and Available RAN Visible QoE Metrics (stage3, YANG) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eQoE |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The parameters were missing in SA5 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of parameters MDT Alignment Information and Available RAN Visible QoE Metrics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR description is not aligned with RAN specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | D.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR …CR … |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/461 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## D.2.14	module _3gpp-common-qmcjob.yang

<CODE BEGINS>

module _3gpp-common-qmcjob {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-qmcjob";

prefix "qmc3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Quality of Experience Measurement Collection Job handling";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2022-11-04 { reference “CR-0215”; }

revision 2022-08-17 { reference "CR-0185"; }

revision 2022-04-29 { reference "CR-0171"; }

revision 2022-03-22 { reference "draftCR"; }

revision 2022-01-06 { reference "CR-0142"; }

grouping QMCJobGrp {

leaf serviceType {

description "Specifies an end user service type for QoE measurements.";

mandatory true;

type enumeration {

enum DASH;

enum MTSI;

enum VR;

}

}

leaf areaScope {

type string;

description "Defines the area in terms of NR cells or Tracking Area

where the QMC shall take place.";

}

leaf qoECollectionEntityAddress {

type inet:ip-address;

mandatory true;

description "Specifies the address to which the QMC records shall be

transferred.  Ipv4 or Ipv6 address(es) may be used.";

}

list pLMNTarget {

max-elements 1;

description "Defines the PLMN for which sessions shall be selected in

the network request session in case of area based QMC when several

PLMNs are supported in the RAN (this means that shared cells and not

shared cells are allowed for the specified PLMN. Furthermore, several

PLMNs can be used for not shared RAN cases as well as for shared RAN

cases.). Only the sessions may be selected where the PLMN that the

UE reports as selected PLMN is the same as the PLMN Target.";

uses types3gpp:PLMNId;

key "mcc mnc";

}

leaf qoETarget {

type string;

description "Specifies the target object of the QMC in case of

signalling based QMC.  The qoETarget attribute shall be able

to carry IMSI or SUPI.";

mandatory true;

}

leaf qoEReference {

type string;

mandatory true;

description "Identifies the QoE measurement collection job in the

Managed Elements and in the measurement collection entity.  The QoE

reference shall be globally unique therefore it is composed as follows:

MCC+MNC+QMC ID, where the MCC and MNC are coming with the QMC

activation request from the management system to identify one PLMN

containing the management system, and QMC ID is a 3 byte Octet String.

The QMC ID is generated by the management system or the operator.";

}

list sliceScope {

description "Contains a list of S-NSSAIs (Single Network Slice Selection

Assistance Information).  A Network Slice end to end is identified by

S-NSSAI.";

key "sd sst";

uses types5g3gpp:SNssai;

}

leaf qMCConfigFile {

type string;

description "Provides a reference to a file including the parameters for

configuration of application layer measurements, known as Container

for Application Layer Measurement Configuration.";

mandatory true;

}

leaf mDTAlignmentInformation {

type uint64;

description "This parameter indicates the MDT measurements with which

alignment of QoE measurement is required. This parameter is optional

and is valid for NR only.";

}

leaf availableRANqoEMetrics {

type enumeration {

enum appLayerBufferLevelList;

enum playoutDelayForMedia;

enum Startup;

}

description "This parameter indicates available RAN visible QoE metrics

to the gNB. This parameter is optional and is valid for NR only.";

}

}

grouping QMCJobSubTree {

description "Helps with augmenting QMCJob into multple places.";

list QMCJob {

description "Represents the QMCJob IOC.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QMCJobGrp;

}

}

}

augment "/subnet3gpp:SubNetwork" {

uses QMCJobSubTree;

}

augment "/me3gpp:ManagedElement" {

uses QMCJobSubTree;

}

}

<CODE ENDS>