3GPP TSG-SA5 Meeting #146	S5-226572

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.404 | CR | 0011 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR 28.404 flow correction and clean up |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrected “Collecting QoE information from a specific end user service type” use case, editorial improvements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned introduction with TS-family for QoE, corrected the main use case, editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case description remains incorrect, a Reference remains incorrect, editorial errors remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction, 5.4.1, 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< unchanged sections skipped >>>

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management, as identified below:

TS 28.404:	"Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements";

TS 28.405:	"Quality of Experience (QoE) measurement collection; Control and configuration";

TS 28.406:	"Quality of Experience (QoE) measurement collection; Information definition and transport";

One main motivation of mobile network evolution is to improve the user experience, which is why the evaluation of the user experience  at the UE side is vital to network operators. This is especially true when the operators provide high bit rate real-time services like streaming services (typically video services), where even intermittent quality degradation is very annoying. Many of these streaming services are a significant part of the commercial traffic growth rate, therefore the focus is on the end users' experience .

Quality of Experience (QoE) information collection provides detailed information at call level on a number of UEs.

The capability to log information within a UE, and in particular the QoE of an end user service, initiated by an operator, provides the operator with QoE information. The collected information (specified in 3GPP TS 26.247 [2]) cannot be deduced from performance measurements in the mobile network.

The QoE information is information collected by the end user application in the UE.

The QoE information is collected by the management system for analysis and/or KPI calculations.

# 1	Scope

<<< unchanged sections skipped >>>

## 5.4	High-level use cases

### 5.4.1	Collecting QoE information from a specific end user service type

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Allow the operator to obtain QoE information for an end user service type based on a 5% of those sessions in a UE(s) in a specified area using a specific end user service type, in a specified collection centre. |  |
| Actors and roles | The operator which is the requester of the QoE information. |  |
| Telecom resources | The management system, the mobile network and the UE. |  |
| Assumptions | The application providing an end user service type is able to provide QoE information about its end user service performance. |  |
| Pre-conditions | Selected end users are using the specified end user service type. |  |
| Begins when | The management system receives a request from the operator. |  |
| Step 1 (M) | The management system receives a request from the operator and transfers it to the mobile network. |  |
| Step 2 (M) | The mobile network receives the request, starts a network request session and starts to check which connections fulfil the request. |  |
| Step3 (M) | When a connection is found that fulfils the request, the mobile network starts an UE request session and transfers the request to the UE. |  |
| Step 4 (M) | When the requested end user service type is started in the UE, it records the requested information for 5% of sessions of the requested service type and sends the recorded information to the specified collection centre. |  |
| Ends when | The network request session expires. |  |
| Exceptions |  |  |
| Post-conditions | The collected information is present in the specified collection centre. |  |
| Traceability | REQ-EUSPC-CON-1, REQ-EUSPC-CON-2 and REQ-EUSPC-CON-3 |  |
| NOTE Step 3 to 4 is repeated for all UEs that access the network in the specified area during the network request session. |  |  |

### 5.4.2	Void

### 5.4.3	Collecting QoE information from end user service type of specific streaming sources

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Allow the operator to obtain QoE information from end user service type from specific streaming sources. |  |
| Actors and roles | The operator which is the requester of the QoE information. |  |
| Telecom resources | The management system, the mobile network and the UE. |  |
| Assumptions | The application providing streaming video service is able to provide QoE information about its end user performance. |  |
| Pre-conditions | The selected end users are using the specified end user service type from the specified streaming sources. |  |
| Begins when | The operator requests a QoE information collection for a specific service type from specific streaming sources. |  |
| Step 1 (M) | When the management system receives a request from the operator, it transfers the request to the mobile network. |  |
| Step 2 (M) | When the mobile network receives the request it starts a network request session and starts to check which connections fulfil the request. |  |
| Step3 (M) | When a connection is found, the mobile network starts a UE request session and transfers the request to the UE. |  |
| Step 4 (M) | When the specified service type is started in the UE, the UE checks whether the streaming source is consistent with any of the streaming sources set by the request, and if yes, the UE collects relevant QoE information and send to the network. |  |
| Ends when | The network request session expires. |  |
| Exceptions |  |  |
| Post-conditions | The collected information is present in the specified collection center. |  |
| Traceability | REQ-EUSPC-CON-2, REQ-EUSPC-CON-3 and REQ-EUSPC-CON-5. |  |

### 5.4.4	Indication of QoE information collection

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Provide the OAM system with an indication that a recording session has started and subsequently allow the OAM system to modify the QoE measurement configuration e.g. the QoE configured area if the number of sessions are too small or too large. The indication may also be used to determine whether or not to terminate the QoE information collection if sufficient number of recording sessions have been started. |  |
| Actors and roles | The operator which is the requester of the QoE information. |  |
| Telecom resources | The management system and the RAN node. |  |
| Assumptions |  |  |
| Pre-conditions | Selected end users have been requested to provide QoE Information when the specified end user service type is used. |  |
| Begins when | The application layer sends AT command including streaming indication to access stratum. |  |
| Step 1 (M) | When the RAN node receives the streaming indication from the UE access stratum, the RAN node sends an indication to the triggering OAM system that a recording session has been started. |  |
| Ends when | The management system has received the indication that a recording session has been started. |  |
| Exceptions |  |  |
| Post-conditions | The OAM system is able to decide if the ongoing QoE measurement collection needs modification and if sufficient data has been obtained for analysis. The OAM system can use the indications to trigger evaluation of collected data. |  |
| Traceability | REQ-EUSPC-CON-6 |  |

### 5.4.5	Change collecting QoE information

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Allow the operator to change an ongoing QoE information collection job. |  |
| Actors and roles | The operator which is the requester of changing the QoE information collection job. |  |
| Telecom resources | The management system, the mobile network and the UE. |  |
| Assumptions | - |  |
| Pre-conditions | The QoE collection job is active. |  |
| Begins when | The operator has information that QoE information collection job needs to have more information as collected data will not be sufficient or that the collected information is enough. |  |
| Step 1 (M) | The management system receives a request from the operator to terminate the ongoing QoE information collection job. |  |
| Step 2 (M) | For all connections where the UE has been requested to start the QoE job, the mobile network transfer the termination request to the UE, which terminates the specified QoE collection information job. |  |
| Step 3 (M) | When all QoE collections are terminated in the UEs for the specified QoE information collection job, the mobile network finish to terminate the QoE information collection job. |  |
| Step 4 (O) | If collected information will not be sufficient, the operator initiates a new QoE information collection job. | Collecting QoE information from a specific end user service type, or Collecting QoE information from end user service type from a specific user. |
| Ends when | The network request session is terminated (when sufficient amount of data is collected), or when the new network request is started (when the collected data will not be sufficient). |  |
| Exceptions |  |  |
| Post-conditions | The network request session is terminated or the new network request is started. . |  |
| Traceability | REQ-EUSPC-CON-1 and REQ-EUSPC-CON-7 |  |

### 5.4.6	Void

<<< unchanged sections skipped >>>