3GPP TSG-SA5 Meeting #146 	S5-226624

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Management of MnFs solution 2

# Document for:	Approval

Agenda Item:	6.8.1.5

# 1	Decision/action requested

Agree an alternative solution to issue#8.

# 2	References

None.

# 3	Rationale

Both centralised and distributed automation functions needs top be managed. However, management of management shall be ovoided.

# 4	Detailed proposal

1st Modified Section

## 4.8	Issue #8: Use of Models in SBMA

### 4.8.1	Description

### 4.8.1.1	Issue 8-1

The following existing concepts are related to SBMA:

- TS 28.533 Management Function: A Management Function (MnF) is a logical entity playing the roles of MnS consumer and/or MnS producer. Management Function can be deployed as a separate entity or embedded in Network Function to provide MnS(s).

Figure 4.8-1: Examples of MnS deployment scenario

- TR 21.905 Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

- TS 28.533 / TS 23.501 Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

The following IOCs are specified to represent Network Element and Network Function:

- TS 28.622 ManagedElement IOC: This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[1] including virtualization or non-virtualization scenario.

- TS 28.622 ManagedFunction IOC: This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions, a managed function may contain other managed functions as specified for the specific subclass. This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI.

The following IOCs as specified in TS 28.622 are used to represent Management Function provided by MnS Producer:

- TS 28.622 MnsAgent IOC: The MnsAgent represents the MnS producers, incl. the supporting hardware and software, available for a certain management scope that is related to the object name-containing the MnS Agent.

- TS 28.622 ManagementNode IOC: This IOC represents a telecommunications management system (EM) within the TMN that contains functionality for managing a number of ManagedElements (MEs). (Note: This description is to be updated to replace use of EM)

#### 4.8.1.2	Issue 8-2

There are centralised and distributed automation functions for the same “thing”, e.g. Coverage and Capacity Optimisation exist both centrally and distributed. To be able to use just one of them, it shall be possible to turn on and off both the centralised and distributed function.

Some automation functions need to be able to turn on and off in e.g. a specified area (list of cells or list of TAs), or some other filtering criteria, where the filer criteria has to be done case by case.

### 4.8.2	Potential solutions

#### 4.8.2.1	Potential solution #8-1

This clause provides the description of using NRM to represent management of management function, network function, and network element.

Management Function could be deployed in following different deployment scenarios:

- The Management Function can be deployed in management system, including

  - The Management Function deployed in domain management system.

  - The Management Function deployed in cross domain management system.

- The Management Function deployed in Network Element.

From the three deployment scenarios of Management Functions above. Management Functions deployed in domain management system and Management Function deployed in Network Element need to be managed as managing network node in 3GPP management system. The way of managing the Management Function are modelled in corresponding management function IOC.

ManagementNode IOC represents a telecommunications management system within the TMN provided by vendor that contains management functionalities for managing a number of ManagedElements (MEs).

The Management Functions are managed by corresponding management function IOC as defined in TS 28.541, TS 28.104, TS 28.105, TS 28.536 and TS 28.312:

- IOC for Management of D-SON function:

- D-SON of ANR in TS 28.541 DANRManagementFunction IOC

- D-SON Energy Saving (ES) functions in TS 28.541 DESManagementFunction IOC

- D-SON RACH functions in TS 28.541 DRACHOptimizationFunction,

- D-SON MRO functions in TS 28.541 DMROFunction,

- D-SON PCI Configuration functions in TS 28.541 DPCIConfigurationFunction,

- D-SON LBO functions in TS 28.541 DLBOFunction,

- IOC for Management of management functions:

- EM centralized SON Energy Saving (ES) functions in TS 28.541 CESManagementFunction IOC

- EM centralized-SON PCI Configuration functions in TS 28.541 CPCIConfigurationFunction,

- EM centralized-SON Energy Saving (ES) functions in TS 28.541 CESManagementFunction,

- EM centralized-SON CCO functions in TS 28.541 CCOFunction

- MDAFunction in TS 28.104

- AIMLTrainingFunction in TS 28.105

- AssuranceClosedControlLoop in TS 28.536

Editor’s note: other management functions to be added with working progress. The criteria for identifying what kind of ManagementFunction should be managed is FFS.

The following table captures the relation between the concepts and related models:

| Concepts | Illustration of Related Management Models |
| --- | --- |
| TS 28.622 ManagementNode | A telecommunications management system within the TMN provided by vendor that contains management functionalities for managing a number of ManagedElements (MEs) are represented by ManagementNode IOC defined in TS 28.622, and a number of management functions. |
| TS 28.533 Management Function | The corresponding XXXManagementFunction IOC is used to support the managing of XXXMnF. |
| TR 21.905 Network Element | A network element is represented by ManagedElement IOC defined in TS 28.622. |
| TS 28.533 / TS 23.501 Network Function | A Network function is represented by ManagedFunction IOC defined in TS 28.622. |

It’s recommended the followingIOCs which could be used to represent MnFs to be managed. The modelling is not changed for existing functions to keep the backward compatibility.

| TS | IOC | Inheritant from | Contained by |
| --- | --- | --- | --- |
| TS 28.541 | - CPCIConfigurationFunction - CESManagementFunction - CCOFunction | Top | - ManagedEntity(SubNetwork/ManagedElement/NRCellDU/ManagementNode) - ManagedEntity(SubNetwork/ManagedElement/NRCellCU/ManagementNode) - SubNetwork/ManagementNode |
| TS 28.104 | MDAFunction | ManagedFunction | MDAEntity (SubNetwork/ManagedElement/ManagedFunction/ManagementNode) |
| TR 28.105 | AIMLTrainingFunction | ManagedFunction | ManagedEntity(Subnetwork/ManagedElement/ManagementFunction/ManagementNode) |
| TS 28.536 | AssuranceClosedControlLoop | Top | SubNetwork/ManagedElement/ManagementNode |

Editors’ Note: whether the MnFs to be managed IOCs are needed to be inherited from a same root IOC is FFS.

#### 4.8.2.2	Potential solution #8-2

This clause provides the description of using NRM to represent management of management function, just as functionality in Network Functions are modelled in the NRM. Also some existing management functions are modelled in the same way as functionality in Network Functions.

A management function is deployed in the management system.

As management of management is to be avoided, management functions need to be divided in two categories:

- “Direct management functions”, such as fault, configuration and performance functions.

- “Indirect management functions”, such as centralised (domain as well as cross domain) automation functions (analytics and assurance functions etc.).

“Indirect management functions” need to have standardised management for what characteristic shall have which target values to optimise on etc. They shaqll be able to be turned on and off.

“Direct management functions” do not need to have standardised management.

Existing managed functions or NFs are not to be remodelled.

and stage 3".

[49]	IETF RFC 8089: "The "file" URI Scheme".

End of Modified Sections