3GPP TSG-SA5 Meeting #146	S5-226635

Toulouse, France, 14 -18 November 2022

# Source:	China Unicom, HUAWEI

# Title:	Add solution for Energy Efficiency of a URLLC network slice based on reliability

# Document for:	Approval

Agenda Item:	6.9.2.1

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.913 v0.3.0: “Study on new aspects of Energy Efficiency (EE) for 5G Phase 2”

[2] 	3GPP TS 28.554 “Management and orchestration;5G end to end Key Performance Indicators (KPI)”

[3]            3GPP TR 28.832 v0.3.0: “Study on management aspects of URLLC”

3	Rationale

As presented in TR 28.913 [1], “Energy Efficiency KPI of URLLC Network Slice based on its Reliability” has been introduced as a key issue. It has been agreed that performance of a URLLC network slice can be its latency and reliability, while existing standardised solution (TS 28.554 [2], clause 6.7.2.3), determines Energy Efficiency (EE) for a URLLC slice only based on Latency performance, reliability performance is not considered. Reliability performance is also an integral part of URLLC slice by nature. Hence, it is important that a variant of Energy Efficiency KPI should also consider reliability performance of a URLLC slice for comprehensive assessment of EE.

This contribution is to propose a potential solution to add a network slice reliability factor to present variants of Energy Efficiency KPI of URLLC Network Slice in TS 28.554 [2].

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

4.6	Key Issue #6: Energy Efficiency KPI of URLLC Network Slice based on its Reliability

4.6.2	Potential solutions

4.6.2.X	Potential solution #X:

4.6.2.X.1	Introduction

In this potential solution #X, it is proposed to define two variants of Energy Efficiency KPI that consider reliability performance of a URLLC slice for comprehensive assessment of EE. Specifically, some of KPI and parameters refer to definitions in TS 28.554.

4.6.2.X.2	         Description

4.6.2.X.2.1	Based on both latency and reliability of the network slice

a) A KPI that shows the energy efficiency of network slices of type URLLC based on both latency and reliability. The Pns for a network slice of type URLLC is the inverse of the average end-to-end User Plane (UP) latency of the network slice multiplied by the end-to-end packet transmission reliability of the network slice [2]. In this KPI variant, latency and reliability are the two factors considered for evaluating the performance of network slice.

b)

PURLLC, Latency,Reliability=Network slice E2E reliabilityNetwork slice mean latency

,where

‘Network slice mean latency’ is defined according to clause 6.7.2.3.2 in TS 28.554

‘Network slice E2E reliability’ is defined as the end-to-end packet transmission reliability of the network slice, and where the end-to-end packet transmission reliability of the network slice from for one S-NSSAI is defined by:

Network slice E2E reliability=DLRelPSR_Uu*DLRelPSR_N3*ULRelPSR_Uu*ULRelPSR_N3

Where DLRelPSR_N3 and ULRelPSR_N3 are defined according to clause 6.8.1 in TS 28.554. Specifically, DLRelPSR_Uu and ULRelPSR_Uu for URLLC need further discussion according to Issue#3 in TR 28.832 [3].

c)

EEURLLC, Latency,Reliability=Network slice E2E  reliabilityNetwork slice mean latency*ECns

d) The KPI object is network slice.

4.6.2.X.2.2	Based on latency, Data Volume (DV) and reliability of the network slice

a) A KPI that shows the energy efficiency of network slices of type URLLC based on both latency, reliability and data volume. The Pns for a network slice of type URLLC is the sum of UL and DL traffic volumes at N3 or N9 interface(s) on a per S-NSSAI basis multiplied by the end-to-end packet transmission reliability of the network slice and the inverse of the end-to-end User Plane (UP) latency of the network slice.

b)

PURLLC,DV, Latency,Reliability=(ωn3*DVN3+ωn9*DVN9)*Network slice E2E  reliabilityNetwork slice mean latency

Where DVN3, DVN9 and relative parameters are defined according to clause 6.7.2.3.3 in TS 28.554.

c)

EEURLLC,DV, Latency,Reliability=(ωn3*DVN3+ωn9*DVN9)*Network slice E2E  reliabilityNetwork slice mean latency*ECNS

d) The KPI object is network slice.

End of changes