3GPP TSG-SA5 Meeting #146	S5-226669

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0016 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.405 clean up |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is inconsistency across TS-family of specifications for QoE and a confusing Reference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned Scope with TS-family for QoE, removed a duplicate Reference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency across TS-family of specifications for QoE remains, a confusing Reference remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4.6.1, 5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | d1: cover page corrections. Updated to show CR affects RAN & CN |  |  |  |  |  |  |  |  |

<<< unchanged sections skipped >>>

# 1	Scope

The present document addresses the mechanisms used for the function Quality of Experience (QoE) measurement collection in 3GPP networks . The measurements that are collected are DASH [6], MTSI [7] and Virtual Reality (VR) (see TS 26.118 [13]) measurements.

The function includes collecting QoE information from UEs frequenting a specified area or an individual UE for a specified end user service/end user service type. The document describes the activation and deactivation of a network request session, UE request session and recording session and also the reporting of recorded information [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.404: "Telecommunication management;Quality of Experience (QoE) measurement collection; Concepts, use cases and requirements".

[3]	3GPP TS 28.308: "Management of Quality of Experience (QoE) measurement collection Integration Reference Point (IRP); Information Service (IS)".

[4]	3GPP TS 25.331: "Radio Resource Control (RRC) protocol specification".

[5]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[6]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[7]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[8]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 36.413: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[10]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[11]	3GPP TS 38.331: " NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 38.413: " NG-RAN; NG Application Protocol (NGAP)".

[13]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

<<< unchanged sections skipped >>>

### 4.6.1	Activation of measurement collection for a UE in NR

#### 4.6.1.0	General

Activation of measurement collection for a UE can be done after UE is registered or before UE Registration procedure.

#### 4.6.1.1	Activation of QoE measurement task after completion of UE registration procedure

Figure 4.6.1.1-1 and the text below describe the activation of QoE measurement collection after completion of UE registration procedure.

Figure 4.6.1.1-1: QMC activation and reporting in NR after UE is registered

1. The MnS Consumer sends createMOI request for QMCJob to UDM that controls the impacted gNB(s), and includes the parameters: serviceType, sliceScope, qoECollectionEntityAddress, qoETarget, qoEReference and qMCConfigFile.

2. The UDM inserts subscriber related data and forwards it to the AMF.

3. The AMF forwards the configuration parameters serviceType, sliceSupportListQMC, measCollEntityIPAddress, qoEReference and containerForAppLayerMeasConfig in message UE Context Modification Request to the impacted gNB.

4. The gNB checks if the UE capability matches the criteria for serviceType in the QoE measurement configuration information.

5. If the UE has the wanted UE capability, the gNB starts a UE request session and stores the associated qoECollectionEntityAddress, sends the message RRCReconfiguration to the UE including serviceType, measConfigAppLayerId and measConfigAppLayerContainer.

NOTE: The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see [11].

6. The access stratum in the UE sends an unsolicited response to the application level including app-meas_service_type, meas_config_app_layer_id and app-meas_config_file. The unsolicited response is for the AT command +CAPPLEVMCNR which is sent from UE Application Level to UE Access Stratum during Registration procedure.

7. When the application for the specified serviceType starts, the QMC is initiated. To specify the session which is started, the application generates a recordingSessionId.

8. The application layer sends the AT command +CAPPLEVMRNR [7] including meas_config_app_layer_id and qoe_measurement_status that indicates that a session is started to the access stratum.

9. The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId and appLayerSessionStatus to the gNB.

10. When the QMC is completed or at the end of period for periodic report, the recorded information is collected in a QMC report, see [6], [7] or [13].

11. The application layer sends the AT command +CAPPLEVMRNR [7] including meas_config_app_layer_id, qoe_measurement_status indicating that session has ended and app-meas_report including recordingSessionId to the access stratum.

12. The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus and measReportAppLayerContainer including recordingSessionId to the gNB.

13. The gNB sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference and the RAN transparent container including the recordingSessionId and sliceScope, which contains only the S-NSSAI used. Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

#### 4.6.1.2	Activation of QoE measurement task before UE Registration procedure to the network

<<< unchanged sections skipped >>>

## 5.5	QMC configuration file (container) (M)

The QMC configuration file is a container that is specified in Annex L of TS 26.247 [6],  clause 16.5 of TS 26.114[7] or clause 9.4 of TS 26.118 [13].

The corresponding parameter name for this container in Radio Resource Control (RRC) Protocol specification is measConfigAppLayerContainer in case of  NR and LTE, see TS 38.331 [11], TS 25.331 [4] or "Container for application layer measurement configuration" in case of UTRAN, see TS 36.331 [8]

For NR, the QMC configuration file parameter in the management interface defines the location of the container..

## 5.6	QMC target (M)

<<< unchanged sections skipped >>>