3GPP TSG-SA5 Meeting #146 	S5-226697

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Correcting solutions 2.13 to 2.15

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Correcting references for solutions #2.13, #2.14, and #2.15.

# 4	Detailed proposal

First change

#### 7.2.4.13	Solution #2.13: V-CHF selected charging information for reporting to H-CHF

##### 7.2.4.13.1	General

A possible solution for key issue #2a and key issue#2b, covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02.the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session (From the standpoint of V-SMF, all the charging information for V-CHF and H-CHF are exchanged in one charging session between V-SMF and V-CHF), V-CHF is responsible for the charging reporting for H-CHF.

##### 7.2.4.13.2	Reference architecture

The reference architecture would be the same as figure 7.2.4.1.2-2.

##### 7.2.4.13.3	Message flows

During the PDU session establishment, V-SMF establishes the charging session with V-CHF, same with the figure 7.2.4.1.3-1 in solution #2.1.

V-CHF is responsible for distinguishing the charging information for H-CHF and V-CHF.

1.	The V-SMF sends Charging data request to V-CHF with the charging information;

2.	V-CHF determines the charging information for V-CHF and handled it.

3.	Based on the triggers on V-CHF, V-CHF constructs and reports the charging information to H-CHF;

4.	H-CHF sends the charging data response to V-CHF;

5.	V-CHF sends charging data response from H-CHF and/or sends the charging data response from V-CHF.

Second change

#### 7.2.4.14	Solution #2.14: V-SMF selected charging information for reporting to H-CHF

##### 7.2.4.14.1	General

A possible solution for key issue #2a and key issue#2b, covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02.the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session, V-SMF is responsible for the charging reporting for the V-CHF and H-CHF.

##### 7.2.4.14.2	Reference architecture

The reference architecture would be the same as figure 7.2.4.1.2-2.

##### 7.2.4.14.3	Message flows

During the PDU session establishment, V-SMF establishes the single charging session with V-CHF, same with the figure 7.2.4.1.3-1 in solution #2.1.

V-SMF is responsible for distinguishing the charging information for H-CHF and V-CHF. The V-CHF sends the charging information between V-SMF and H-CHF.

1.	The V-SMF sends charging data request to V-CHF with both the charging information for V-CHF and charging information for H-CHF with the indication;

2.	Based on the indication, V-CHF handles the charging information for V-CHF;

3.	Based on the indication, V-CHF sends charging information to H-CHF;

4.	H-CHF sends the charging data response to V-CHF;

5.	V-CHF sends charging data response from H-CHF and/or sends the charging data response from V-CHF its own.

Third change

#### 7.2.4.15	Solution #2.15: V-SMF setting charging information indication by information element group

##### 7.2.4.15.1	General

A possible solution for key issue #2a and key issue#2b, covering requirements REQ-CH_CVTOH-01, and REQ-CH_CVTOH-02.the charging information for 5G data connectivity provided to the CHF in the visited MNO and CHF in home MNO (via V-CHF) in single charging session, V-SMF is responsible for the charging reporting for the V-CHF and H-CHF, V-SMF sets an indication for a group of information elements for V-CHF, and sets another indication for the other group of information elements for H-CHF.

##### 7.2.4.15.2	Reference architecture

The reference architecture would be the same as figure 7.2.4.1.2-2.

##### 7.2.4.15.3	Message flows

In step 1 of message flow  figure 7.2.4.1.3-1, take the following charging information of FBC for example, the V-SMF set two indications for two groups of information elements, one indication is for V-CHF, the other indication is for H-CHF.

Table 7.2.4.b.3: Structure of Roaming QBC Information

Multiple Unit Usage (Group2 for V-CHF)

Rating Group

Used Unit Container

Target Indication: V-CHF

Multiple Unit Usage (Group2 for H-CHF)

Rating Group

Requested Unit

Used Unit Container

Target Indication: H-CHF

End of changes