3GPP TSG-SA5 Meeting #146 	S5-226707

Toulouse, France, 14 -18 November 2022

# Source:	Ericsson

# Title:	Addition of solution in clause 5.2 using separate CCS

# Document for:	Approval

Agenda Item:	7.5.4

# 1	Decision/action requested

Include the proposed changes in TR 28.828.

# 2	References

[1]		3GPP TR 28.828: "Study on charging aspects for enhanced support of non-public networks"

# 3	Rationale

Adding a solution for converged charging for number of UEs or PDU sessions using separate CCS

# 4	Detailed proposal

First change

#### 5.2.4.x	Solution #x: Converged charging for number of UEs or PDU sessions using separate CCS

##### 5.2.4.x.1	General

This solution addresses the key issues #2a, 2b and 2c covering potential requirements REQ-eNPN_CH_SNPN_AC-03, REQ-eNPN_CH_SNPN_AC-04  and REQ-eNPN_CH_SNPN_AC-05 based on distributed charging architecture split between two Converged Charging Systems (CCS) and exchanges between them.

Two types of subscribers are considered in the context of SNPN: the NPN-SC, and the individual end-user "UE" served under the NPN. Each type of subscriber is handled under a dedicated CCS: "NPN CCS" and "UE CCS" respectively.

UE CCS of the PLMN performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF/RF hosting individual UE account balance and subscribed services, as well as the rates for services.

NPN CCS of the PLMN performs charging functionalities for NPN-SC, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF/RF hosting individual NPN-SC account balance and subscribed services, as well as the rates for services.

How the account balance and subscribed services are provisioned (for both NPN-SC and individual UE) is unspecified and expected to be covered at BSS and OSS level.

A new reference point (ChX) is defined between CHF in NPN CCS and the CHF in UE CCS enabling association between individual UE charging and NPN-SC charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the NPN-SC CCS.

The new reference (ChX) may use the Nchf_ConvergedCharging service with extensions or use a completely new Nchf service.

For UE:

-	the 5G data connectivity charging from SMF is specified in 3GPP TS 32.255 [12]

-	the 5G connection and mobility charging from AMF is specified in 3GPP TS 32.256 [11]

and uses Nchf_ConvergedCharging service exposed by CHF in UE CCS.

##### 5.2.4.x.2	Architecture description

Figure 5.2.4.x.2-1: Distributed charging architecture in reference point representation.

Figure 5.2.4.x.2-2: Distributed charging architecture in service based representation.

##### 5.2.4.x.3	Procedures description

This would be the same as for the figure 6.1.4.4.3-1 for the number of PDU sessions and 6.1.4.4.3-2 for the number of UEs with the possibility of using Network Identifier and PLMN Id to identify the SNPN.

End of changes