3GPP TSG-SA5 Meeting #146 	S5-226789

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0803 | rev | 2 | Current version: | 18.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TS 28.530 defines that NetworkSlice instance represents service view of a network slice which exposes the root NetworkSliceSubnet instance. The non-root NetworkSliceSubnet instances are not directly associated with the NetworkSlice object instance. In Figure 6.2.1-1, the relation between NetworkSlice IOC and NetworkSliceSubnet IOC with 1:1 relationship implies that : - Every NetworkSlice MOI must be associated with one NetworkSliceSubnet MOI, and - Every NetworkSliceSubnet MOI must be associated with one NetworkSlice MOI. The NetworkSliceSubnet MOI which does not represent root/top NetworkSliceSubnet does not have a direct relation with NetworkSlice. Hence, the multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC represented Figure 6.2.1-1 in is incorrect. Further, NetworkSliceSubnets are associated to zero or one NetworkSlice - Zero: allows to have NetworkSliceSubnets associated to each other, without a NetworkSlice - One: Allows a NetworkSliceSubnet to be associated to exactly one NetworkSlice Any NetworkSlice is associated to exactly one NetworkSliceSubnet. Hence a correction is needed to the multiplicity of relationship between NetworkSlice IOC and NetworkSliceSubnet IOC represented Figure 6.2.1-1 needs to be corrected from 1:1 to 0..1:1 to allow the non-root NetworkSliceSubnet instances to exist without a direct relation with the NetworkSlice object instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multiplicity of relation between NetworkSlice IOC and NetworkSliceSubnet IOC is corrected from 1:1 to 0..1:1 Figure 6.2.1-1. Description for NetworkSliceSubnet IOC enhanced to describe the relationship between NetworkSlice instance, NetworkSliceSubnet instance representing top network slice subnet, NetworkSliceSubnet instance representing RAN network slice subnet and NetworkSliceSubnet instance representing Core network slice subnet. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect standards leads to confusion |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is a mirror of Rel-17 tDoc with reference S5-226112. Rel-16 related correction is in CR#0801 with tDoc S5-226111 and Rel-17 related correction is in CR#0802 with tDoc S5-226112. Since there are additional changes in Rel-17, the Rel-17 CR is a correction CR and not a mirror. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-225394 is a revision of S5-223451 - S5-226113 is a mirror of Rel-17 CR S5-226112. S5-226112 is a revision of S5-225394. Note: S5-225394 was submitted for Rel-18. Since the correction is applicable for Rel-16 and Rel-17 too, the correction is applied for Rel-16 Rel-16 in S5-226111 and for Rel-17 in S5-226112. - S5-226789 is a revision of S5-226113: Incorporation of review comments received. Note 4 added for Figure 6.2.1-1 to describe the the cardinality relation between NetworkSlice and NetworkSliceSubnet IOC. |  |  |  |  |  |  |  |  |

1st Change

### 6.2.1	Relationships

Figure 6.2.1-1: Network slice NRM fragment relationship

NOTE 1:	The <<OpenModelClass>> NetworkService and <<OpenModelClass>> VNF are defined in [40].

NOTE 2:	The target Network Service (NS) instance represents a group of VNFs and PNFs that are supporting the source network slice subnet instance.

NOTE 3:	The instance tree of this NRM fragment would not contain the instances of NetworkService and VNF. However, the NetworkSliceSubNet instances would have an attribute holding the identifiers of NetworkService instances and the ManagedFunction instance would have an attribute holding identifiers of VNF instances.

NOTE 4: NOTE 4: Any instance of the NetworkSliceSubnet IOC is associated to 0 to 1 instance of the NetworkSlice IOC:

- 0: applies to the top/root NetworkSliceSubnet IOC instance directly associated to a NetworkSlice IOC instance.

- 1: applies to all non-top/root NetworkSliceSubnet IOC instances, also known as constituent network slice subnets, not directly associated to a NetworkSlice IOC instance.

Any instance of the NetworkSlice IOC is associated to exactly one instance of NetworkSliceSubnet IOC (i.e. the top/root NetworkSliceSubnet IOC instance).

Figure 6.2.1-2: Transport EP NRM fragment relationship

Figure 6.2.1-3: containment relationship for network slice fragment

Figure 6.2.1-4: containment relationship for feasibility check and resource reservation NRM fragment

2nd Change

#### 6.3.2.1	Definition

This IOC represents the properties of a network slice subnet in a 5G network. For more information about the network slice subnet instance, see 3GPP TS 28.530 [70].

The NetworkSliceSubnet can be categorized by following types:

-	RANSliceSubnet represent the RAN network slice subnet in a 5G network, which is associated to one or multiple “RANSliceSubnetProfile”.

-	CNSliceSubnet represent the CN network slice subnet in a 5G network, which is associated to one or multiple “CNSliceSubnetProfile”.

-	TopSliceSubnet represent the top network slice subnet in a 5G network, which is associated to one or multiple “TopSliceSubnetProfile”.

The attribute epTransportRef is used to specify a list of EP_Transport instance as transport resources to be aggregated to a NetworkSliceSubnet instance. The MnS consumer determines the EP_Transport instance(s) to support EP_Application instances as part of the NetworkSliceSubnet instance and request the MnS producer to configure the attribute epTransportRef of the NetworkSliceSubnet.

The EP_Transport is name contained by SubNetwork, and an EP_Transport instance can be a new instance created for the EP_Application instances as part of NetworkSliceSubnet instance or an existing instance reused for EP_Application instance.

An instance of NetworkSliceSubnet that represents top network slice subnet shall be associated with one NetworkSlice instance. An instance of NetworkSliceSubnet that represents CN network slice subnet or RAN network slice subnet shall be associated with one instance NetworkSliceSubnet that represents top network slice subnet. An instance of NetworkSliceSubnet that represents CN network slice subnet or RAN network slice subnet may be associated with one or more constituent NetworkSliceSubnet instance(s) represented by attribute networkSliceSubnetRef.

End of change