# 3GPP TSG SA WG5 Meeting #146-e	                                                                                                                                                           S5-226814

# November 14 – November 18, 2022, e-Meeting

# Source:	Samsung

# Title:	Discussion Paper on GSMA NBI Requirements

# Document for:	Endorsement

Agenda Item:	6.5.3.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

# 3	Discussion

| Serial No. | Requirements | Description | Already Supported? | Comments/ Way forward |
| --- | --- | --- | --- | --- |
| 1 | The platform shall be able to support the following Application Provider requirements: |  |  |  |
| 1.1 | Footprint/coverage area selection | This is used to specify the target location for an application. | Supported | We already have requiredEASservingLocation as part of EASRequirement IOC. |
| 1.2 | Infrastructure resources | This included application resource requirements from the infrastructure including CPU, memory, storage, hypervisor, networking definition used by the application | Partial Supported | We already have virtualResource as part of EASRequirement IOC. We need to extend EASRequirements to include hypervisor and networking requirements. |
| 1.3 | Other optional requirements from the infrastructure | This includes other requirements from the infrastructure including use of GPUs, use of FPGAs, accelerator support: SRIOV, DPDK, performance optimisation capabilities: NUMA, CPU Pinning, use of dedicated core etc. | Not supported | Extensions required for EASRequirements IOC. |
| 1.4 | Requirements from edge cloud | This includes QoS requirement for the EDN. | Partial Supported | QoS requirements are considered in EAS deployment procedure but it is missing as part of EASRequirements IOC. Extensions required for EASRequirements IOC. |
| 1.5 | Type of application instantiation | This include static, dynamic and capacity based application instantiation. Static: the application shall be deployed in several edge sites based on Application Provider's requirements and the operator's deployment criteria. The application shall be deployed upfront (independently of the UC's request). Dynamic: when a UC requests an application, the application shall be deployed in the selected edge location (triggered by UNI request(s)). Based on capacity: criteria to define if there needs to be an instance per user or one instance per specific number of users. | Partial Supported | Static EAS instantiation is supported where EASRequirements tells what is required. Dynamic EAS instantiation is supported based on EAS discovery failure event. EAS instantiation based on UC request should be considered implicitly covered as an unsuccessful UC request would result in EAS discovery failure measurement production. Dynamic instantiation based on user capacity is out-of-scope of SA5. |
| 1.6 | Policy control concerning support of stateful and stateless applications. | The Application Provider shall be able to indicate that: - Its Edge Application cannot be moved from one edge compute resource to another - Its Edge Application can be moved from one edge compute resource to another, without any notification - Its Edge Application can be moved from one edge compute resource to another with prior notification. | Not supported | This sort of information need to be captured in the edge NRM. Extensions required for EASRequirements/EASFunction IOC. |
| 1.7 | Session Continuity | Session Continuity sensitivity indicating the edge application's capabilities to support application session relocations across Cloudlets | Supported | EASRequirements already has attribute serviceContinuity indicating if the EAS supports service continuity which requires ACR from source EAS to target EAS. |
| 2. | The OP shall offer following management capabilities to Application Providers |  |  |  |
| 2.1 | The OP shall enable application developers to request Edge Cloud in an Availability Zone (within the OP and federated OPs) | This allows AP to request for deployment of Edge Cloud in an Availability Zone | Not Supported | This requires an NBI functionality using which AP can request edge services in a particular Availability Zone (geographical location). The relation between requiredEASservingLocation and Availability Zone is not clear. |
|  |  | This allows AP to request for deployment of Edge Cloud in an Availability Zone: - On a basis where the application developer reserves resources (on a relatively long-lasting basis) ahead of their usage. - On a basis where resources are allocated as the application instance needs them (“reservationless” or “dynamic”) and the application developer selects the degree of scaling it requires (for example, number of sessions). - On a basis where resources are isolated from those used by other application developers. - An application developer may provide the OP with information about its estimated workload to help the OP optimise the deployment of Edge Application(s). The resources to be reserved includes Infrastructure resources (CPU, memory, storage, networking), use of GPUs and hardware accelerators. | Not Supported | The resource reservation functionality does not exist in 28.538. |
| 2.2 | An OP shall offer a range of quality policies so that an Application Provider can choose the performance that their application requires. | These policies are defined based on objectively measured end-to-end parameters that include performance aspects of both the network and the Cloudlet, such as latency, jitter and packet loss (measured as average statistics). | Not Supported | This requires an NBI functionality where the list of available QoS is provided to the AP so that it can make choice of the location where to deploy the application. |
| 2.3 | QoS query | The NBI shall enable a request-response mechanism through which the Application Provider can state a geographical point where a typical user could be and get informed of the mean latency performance expected. | Not Supported | This requires an NBI functionality where the available QoS at a particular location is provided to the AP. |
| 2.4 | Edge Cloud capabilities | The OP shall describe the capabilities of the Edge Cloud including the geographical zones where it is provided and the type and “granularity” of edge cloud and network service (typically generic Compute, memory, storage, and specialised compute, such as GPU and future resource types). | Partially Supported | Considering EdgeCloud = EDN, the EDN IOC already contain eDNServiceArea. Service details can be added in the EDN IOC. This can be done by referring to related specifications in ETSI NFV. |
| 2.5 | Application LCM | The OP shall offer a structured workflow for application deployment and instantiation: CRUD functions. | Supported |  |
|  |  | The OP shall allow a developer to specify that its Edge Applications should be restricted to a particular geographical zone. This restriction would ensure compliance with the applicable data privacy laws. | Supported |  |
| 2.6 | Performance Assurance | The OP shall provide an Application Developer with telemetry information concerning the performance of the Edge Cloud service, including fault reporting. | Supported |  |
|  |  | The OP shall offer functionality that supports the application developer to manage its application instances. For example, to monitor operational performance, get diagnostic logs and help with debugging. | Supported |  |
| 2.8 | Application Resource Catalogue | The OP shall offer Application Providers a registry to store their application images and update or delete them. The registry may be centralised or distributed, depending upon the Application Provider’s needs to reduce boot time and recovery. | Not supported | The functionality of Application Catalogue/Registry is missing in 28.538. 28.538 may refer to the related definitions from ETSI NFV. |

# 4	Proposals

WG is requested to endorse the text in column named “Comments/ Way forward”.

End of modified section