3GPP TSG-SA5 Meeting #146 	S5-226827

Toulouse, France, 14 -18 November 2022							revision of S5-226582

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0832 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Specify network slicing provisioning rule requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSRULE |  |  |  |  |  |  | Date: |  | 2022-10-26 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Specify Network slicing provisioning rule requirements allowing the network slice or network slice subnet MnS consumer to specify additional requirements influencing MnS producer decisions about NetworkSlice or NetworkSliceSubnet sharing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5.1.1 and 5.1.2 Update use case for network slice allocation and network slice subnet allocation to include the selectively-shared option 5.2.1 and 5.2.2 Update provisioning requirements for network slice and network slice subnet 6.5.1.1 and 6.5.2.1 Update operation descriptions for network slice provisioning and network slice subnet provisioning to include the selectively-shared option. This stage 2 update is dependent on slice NRM updates in the related contribution S5-226XXX. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Insufficient capabilities to guide the producer in provisioning of network slice and network slice subnet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2, 5.2.1, 5.2.2, 6.5.1.1, 6.5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR 28.541 CR S5-226XXX |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR .. |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st Change

### 5.1.1	Network slice instance allocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To satisfy request for allocation of a network slice instance with certain characteristics, by creation of new or using existing network slice instance; the request includes the network slice related requirements. |  |
| Actors and Roles | A network slice provisioniong management service consumer. |  |
| Telecom resources | Network slice instance Network slice subnet instance Transport network A network slice provisioning management service provider. A network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice provisioning management service provider receives the request for allocation of the network slice instance with certain characteristics; the request contains network slice related requirements and the information indicating whether the requested network slice requirements could be allocated to existing NSIs given by constraints associated with one or more sharing groups. If sharing is allowed, the consumer may specify an additional constraint restricting sharing to other allocations which by the consumer have been associated with a specific sharing group. Further constraints on isolation may also be given for resources and management data. The network slice provisioning management service provider have the capability to translate the network slice related requirements (e.g. SLA information from GSMA GST) and use it as input to the servie profile [6], the service profile can also be translated to the corresponding requirements for dedicated domains and NSSIs. |  |
| Step 1 (M) | If the requested NSI can be shared and if an existing NSI can be used, the network slice provisioning management service provider decides to use the existing NSI. Modification of the existing NSI may be needed to satisfy the network slice instance related requirements. Use case is completed go to “Step 8". Otherwise, the network slice provisioning management service provider triggers to create a new NSI, for which the following steps 2 – 8 are needed. |  |
| Step 2 (M) | The network slice provisioning management service provider decides on the constituent NSSIs and the topology of the NSI to be created using the information from service profile [6]. For the constituent NSSIs, the network slice provisioning management service provider derives network slice subnet related requirements from the network slice related requirements. If reconfiguration of the transport network is needed, the network slice provisioning management service provider derives transport network related requirements (e.g. latency, bandwidth) from the network slice related requirements. |  |
| Step 3 (M) | For the required NSSI(s), the network slice provisioning management service provider sends network slice subnet related requirements to the network slice subnet provisioning management service provider to request allocation of the required NSSI(s). | Network slice subnet instance allocation use case |
| Step 4 (M) | The network slice provisioning management service provider receives the information of the allocated NSSI(s) (e.g. the management identifier of NSSI, service access point information of NSSI, external connection point information of NSSI) from NSSMF. |  |
| Step 5 (M) | The network slice provisioning management service provider, via the network slice subnet provisioning management service provider, sends the transport network related requirements (e.g. external connection point, latency and bandwidth) to the TN Manager. The TN manager reconfigures the TN accordingly and responds to the network slice provisioning management service provider via the network slice subnet provisioning management service provider. |  |
| Step 6 (M) | The network slice provisioning management service provider receives the response from TN Manager via the network slice subnet provisioning management service provider. |  |
| Step 7 (M) | The network slice provisioning management service provider associates the NSSI(s) with the corresponding NSI (e.g. allocation of the management identifier of NSI and mapping the management identifier of NSI with the received management Identifier of NSSI(s)) and triggers to establish the links between the service access points of the NSSI(s). |  |
| Step 8 (M) | The network slice provisioning management service provider notifies the network slice instance information of NSI (e.g., the management identifier of NSI). |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | An NSI is ready to satisfy the network slice related requirements. |  |
| Traceability | REQ-PRO_NSSI-FUN-1, REQ-PRO_NSI-FUN-3, REQ-PRO_NSI-FUN-X. |  |

### 5.1.2	Network slice subnet instance allocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Create a new network slice subnet instance or use an existing network slice subnet instance to satisfy the network slice subnet related requirements; provide the provisioning service consumer with identity of the NFVO which the consumer can use for further access to the information of the involved VNFs, PNFs and NSs. |  |
| Actors and Roles | A network slice subnet provisioning management service consumer. |  |
| Telecom resources | Network Slice Subnet instance Network Service instance A network slice subnet provisioning management service provider. The operator deployed NFVO to manage the lifecycle of VNFs and interconnection between the VNFs and PNFs in terms of the NS instances. |  |
| Assumptions | Network slice subnet instance may include network functions which are virtualized. |  |
| Pre-conditions | VNF Packages for virtualized network functions to be included in the network slice subnet instance have been already on-boarded. |  |
| Begins when | The network slice provisioning management service provider sends to the network slice subnet provisioning management service provider a request for a NSSI to be associated with the NSI; the request contains network slice subnet related requirements including the SliceProfile [6]. The network slice subnet provisioning management service provider receives request for a network slice subnet instance. The request contains network slice subnet related requirements and the information indicating whether or not the requested NSSI could be shared with profiles from other allocation requests. If sharing is allowed, the consumer may specify an additional constraint restricting sharing to other allocation requests which by the consumer have been associated with a specific sharing group. The request may include guidance for use of particular NFVO(s) when VNFs and PNFs in certain part of the network are involved. The request may also include query of the identity of the NFVO to be used. |  |
| Step 1 (M) | Based on the network slice subnet related requirements received, the network slice subnet provisioning management service provider decides to create a new NSSI or use an existing NSSI. |  |
| Step 2 (M) | If an existing network slice subnet instance is decided to be used, the network slice subnet provisioning management service provider may trigger to modify the existing network slice subnet instance to satisfy the network slice subnet related requirements. Go to “Step 8”. Otherwise, the network slice subnet provisioning management service provider triggers to create a new NSSI, the following steps are needed. |  |
| Step 3 (O) | If the required NSSI contains constituent NSSI(s) managed by other network slice subnet provisioning management service provider (s), the first network slice subnet provisioning management service provider derives the requirements for the constituent NSSI(s) and sends those requirements to the corresponding network slice subnet provisioning management service provider (s) which manages the constituent NSSI(s). The first network slice subnet provisioning management service provider receives the constituent NSSI information from the other network slice subnet provisioning management service provider (s) and associates the constituent NSSI(s) with the required NSSI. |  |
| Step 4 (M) | Based on the network slice subnet related requirements received and SliceProfile [6], the network slice subnet provisioning management service provider decides that to satisfy the NSSI requirements, the part of the network controlled by certain NFVO should be involved. The network slice subnet provisioning management service provider determines the NS related requirements (i.e. information about the target NSD and additional parameterization for the specific NS to instantiate, see clause 7.3.3 in ETSI GS NFV-IFA013 [3]). |  |
| Step 5 (M) | Based on the NS related requirements, the network slice subnet provisioning management service provider triggers corresponding NS instantiation request to NFVO via Os-Ma-nfvo interface as described in clause 6.4.3 in TS 28.525 [2], and the NFVO performs NS instantiation. (see note) | TS 28.525 [2] Clause 6.4.3 NS instance use cases |
| Step 6 (M) | The network slice subnet provisioning management service provider associates the NS instance with corresponding network slice subnet instance (e.g. allocation of the management identifier of NSSI and mapping with the corresponding identifiers). |  |
| Step 7 (M) | The network slice subnet provisioning management service provider is using the NF provisioning service to configure the NSSI constituents. In case of RAN NSSI, the configuration contains RRM policy information for individual Radio cells. In the cells shared by multiple NSSIs such policy includes guidance for split of Radio resources between the NSSIs. | NF provisioning service |
| Step 8 (M) | The network slice subnet provisioning management service provider notifies the provisioning service consumer with the NSSI information (e.g. the management identifier of NSSI) and the NFVO identity when relevant. The network slice provisioning management service provider associates the NSSI with the NSI. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | A NSSI is ready to satisfy the network slice subnet related requirements. |  |
| Traceability | REQ-PRO_NSSI-FUN-2, REQ-PRO_NSSI-FUN-3, REQ-PRO_NSSI-FUN-4, REQ-PRO_NSSI-FUN-5, REQ-PRO_NSSI-FUN-6, REQ-PRO_NSSI-FUN-14, REQ-PRO_NSSI-FUN-Y. |  |
| NOTE: According to the TS 28.525 [2], for the PNFs, NS instantiation includes only establishment of interconnection with other NFs. |  |  |

Start of 2nd Change

### 5.2.1	Requirements for network slice provisioning service

REQ-PRO_NSI-FUN-1	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request a network slice instance.

REQ-PRO_NSI-FUN-2	The network slice provisioning service provider shall have the capability allowing its authorized consumer to send the network slice related requirements.

NOTE 1: The network slice related requirements include requirements such as area traffic capacity, charging, coverage area, isolation, end-to-end latency, mobility, overall user density, priority, service availability, service reliability, UE speed; see TS 22.261 [5] where these parameters are defined for end user services.

NOTE 2:The network slice related requirements also include requirements derived from the Generic network Slice Template (GST) defined by GSMA in [9].

NOTE 3: The SLA requirements can be translated to network service profile which can be used to decides on the constituent NSSIs and the topology of the NSI.

NOTE 4: The ServiceProfile can be translated to corresponding requirements for dedicated domains/NSSI.

REQ-PRO_NSI-FUN-3	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the deallocation of a network slice instance.

REQ-PRO_NSI–FUN-4	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request activation of a network slice instance.

REQ-PRO_NSI–FUN-5	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice instance.

REQ-PRO_NSI-FUN-6	The network slice provisioning service provider shall have the capability allowing its authorized consumer to request the modification of a network slice instance.

REQ-PRO_NSI-FUN-7	The network slice provisioning service provider shall have the capability allowing its consumer to obtain the network slice management data.

REQ-PRO_NSI-FUN-8	The network slice provisioning service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning the requested network slice instance  at a particular point of time.

REQ-PRO_NSI-FUN-9	The network slice management service provider shall have the capability allowing its authorized consumer to request the capacity planning of a network slice instance.

REQ-PRO_NSI-FUN-10	The network slice management service provider shall have the capability allowing its authorized consumer to modify the network slice related requirements.

REQ-PRO_NSI-FUN-X	The network slice provisioning service provider shall have the capability allowing its authorized consumer to include requirements related to sharing of network slice instances, and isolation of resources and management data in a network slice allocation request

NOTE 1: The network slice related requirements may include requirements on whether or not a dedicated network slice instance has to be used to satisfy the allocation request

NOTE 2: The network slice related requirements may include an optional restriction on sharing, based on instance sharing groups, where allocation requests for the same sharing group can be allocated to a common network slice instance

NOTE 3: The requirements issued by the authorized consumer can guide the provisioning service provider. However, even if allowed based on consumer input, it is the service provider that decides whether or not to share an instance between different allocation requests.

### 5.2.2	Requirements for network slice subnet provisioning service

REQ-PRO_NSSI-FUN-1	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request a network slice subnet instance.

REQ-PRO_NSSI-FUN-2	The network slice subnet provisioning service provider shall have the capability of interaction with NFVO via the NS lifecycle management interface.

REQ-PRO_NSSI-FUN-3	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to send network slice subnet related requirements.

REQ-PRO_NSSI-FUN-4	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request to create a new NSSI or use an existing NSSI based on the network slice subnet related requirements.

REQ-PRO_NSSI-FUN-5	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request to associate the NS instance with corresponding NSSI.

REQ-PRO_NSSI-FUN-6	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request the configuration of the RAN NSSI constituents with the RRM policy information for simultaneous support of multiple NSIs.

REQ-PRO_NSSI-FUN-7	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain network slice subnet instance information.

REQ-PRO_NSSI-FUN-8	The network slice subnet provisioning service provider shall have the capability of allowing its authorized consumer to request the deallocation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-9	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request activation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-10	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request deactivation of a network slice subnet instance.

REQ-PRO_NSSI-FUN-11	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to request modification of a network slice subnet instance.

REQ-PRO_NSSI-FUN-12	The network slice subnet provisioning service provider shall have the capability allowing its consumer to obtain information regarding available network slice subnet resources.

REQ-PRO_NSSI-FUN-13	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to obtain the feasibility of provisioning a network slice subnet instance at a particular point of time.

REQ-PRO_NSSI-FUN-14	The network slice subnet provisioning service provider shall have the capability to satisfy the request to consume the NF provisioning service.

REQ-PRO_NSSI-FUN-15	The network slice subnet provisioning service provider shall have the capability to expose limited management capability to its consumer according to mutual agreement.

REQ-PRO_NSSI-FUN-16	The network slice subnet provisioning service provider shall have the capability allowing its consumer to provide slice specific operation information for the (re)configuration to a NSSI.

REQ-PRO_NSSI-FUN-x	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to assign priority of a network slice subnet.

REQ-PRO_NSSI-FUN-Y	The network slice subnet provisioning service provider shall have the capability allowing its authorized consumer to include requirements related to sharing of network slice subnet instances, and isolation of resources and management data sharing.in a network slice subnet allocation request

NOTE 1: The network slice subnet related requirements may include requirements on whether or not a dedicated network slice subnet instance has to be used to satisfy the allocation request

NOTE 2: The network slice subnet related requirements may include an optional restriction on sharing, based on instance sharing groups, where allocation requests for the same sharing group can be allocated to a common network slice subnet instance

NOTE 3: The requirements issued by the authorized consumer can guide the provisioning service provider. However, even if allowed based on consumer input, it is the service provider that decides whether or not to share an instance between different allocation requests.

Start of 3rd Change

## 6.5.	Operations of provisioning

### 6.5.1	AllocateNsi operation

#### 6.5.1.1	Description

This operation is invoked by network slice provisioning MnS consumer to request the provider to allocate a network slice instance to satisfy network slice related requirements. The provider may create a new NSI or using existing NSI to satisfy the request. The requirements in the request are compared/matched against the actual capabilitites of all candidate NSIs by the provider. If an existing NSI can be found e.g. with the right coverage and with good enough latency and with compatible fulfilment rules, it is eligible for allocation. If such an existing NSI can not be found then a new NSI is created with capabilities to host the service, provided that required NSSIs can be created.

#### 6.5.1.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. |

#### 6.5.1.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the ServiceProfile [6] which has been allocated and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| networkSliceDN | M | The DN of NetworkSlice MOI uniquely identifying the network slice instance. |  |

### 6.5.2	AllocateNssi operation

#### 6.5.2.1	Description

This operation is invoked by network slice subnet provisioning MnS consumer to request the provider to allocate a network slice subnet instance to satisfy the network slice subnet related requirements. The provider may create a new NSSI or using existing NSSI to satisfy the request. The requirements in the request are compared/matched against the actual capabilitites of all candidate NSSIs by the provider. If an existing NSSI can be found e.g. with the right coverage and with good enough latency and with compatible fulfilment rules, it is eligible for allocation. If such an existing NSSI can not be found,  then a new NSSI is created with capabilities to host the service, if enough resoures are available.

#### 6.5.2.2	Input parameters

| Parameter Name | Support Qualifier | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter specifies the network slice subnet related requirements defined in SliceProfile in Clause 6.3.4 in TS 28.541 [6]. |

#### 6.5.2.3	Output parameters

| Parameter name | Support Qualifier | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the SliceProfile [6] which has been allocated and the actual value assigned to each. |
| status | M | ENUM (OperationSucceeded, OperationFailed) | An operation may fail because of a specified or unspecified reason. |
| networkSliceSubnetDN | M | The DN of NetworkSliceSubnet MOI uniquely identifying the network slice subnet instance. |  |

End of Changes