# 3GPP TSG-SA5 Meeting #146e	S5-226832

# 14 Nov to 18 Nov 2022, E-meeting                                                  revision of S5-226412

# Source:	China Telecom

# Title:	Add the evaluation for KI#1

# Document for:	Approval

# Agenda Item:	6.7.6.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-040: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

In the previous meetings, the key issue # 1 for NWDAFFunction IOC enhancement to support multiple NWDAF deployment has been studied, and the potential solution has been approved.

In this contribution, we propose to evaluate the key issue #1.

4	Detailed proposal

Start of 1st Change

4.1.2	Potential solutions

4.1.2.1	Potential solution #1: the attribute for NWDAF capability

4.1.2.1.1	Introduction

This solution is provided on how to enhance the NWDAFFunction IOC to support reflecting the differences on the NWDAF capability aspect in multiple NWDAF deployment. The proposed attribute indicating the different NWDAF capability is defined in the NWDAFFunction IOC.

The proposed solution can be a potential solution for the NWDAFFunction IOC enhancement to support multiple NWDAF deployment.

4.1.2.1.2	Description

These proposed attributes indicating the "analytics aggregation capability" and "analytics metadata provisioning capability" of NWDAF are defined in NWDAFFunction IOC as follows:

Attribute name: analyticsAggregation;

Documentation: It represents the "analytics aggregation capability" identifier, which indicates whether the NWDAF has the "analytics aggregation capability".

Allowed Values: The allowed value can be "0"and "1", where "1" indicates the NWDAF has the "analytics aggregation capability" and "0" indicates the NWDAF does not support the "analytics aggregation capability".

Attribute name: analyticsMetadataProvisioning;

Documentation: It represents the "analytics metadata provisioning capability" identifier, which indicates whether the NWDAF has the "analytics metadata provisioning capability".

Allowed Values: The allowed value can be "0" and "1", where "1" indicates the NWDAF has the "analytics metadata provisioning capability" and "0" indicates the NWDAF does not support the "analytics metadata provisioning capability".

Start of 2nd Change

4.1.X	Evaluation

There is one potential solution for the NWDAFFunction IOC enhancement to support Multiple NWDAF Deployment.

In the case where multiple NWDAF instances are deployed, the NWDAFs can be deployed as different NWDAF instances with different capabilities. In the potential solution #1, the attribute  analyticsAggregation is defined to indicate whether the NWDAF has the analytics aggregation capability, and the attribute analyticsMetadataProvisioning is defined to indicate whether the NWDAF has the analytics metadata provisioning capability. This solution can be used to distinguish the NWDAF instances based on the NWDAF capability in multiple NWDAF deployment, so that the management can be performed accordingly.

Consequently, this potential solution can satisfy the requirements to enhance the NWDAFFunction IOC to support multiple NWDAF deployment. This solution is a feasible candidate as input to the normative phase on the NWDAFFunction IOC enhancement to support multiple NWDAF deployment.

End of Modified Sections