# 3GPP TSG-SA5 Meeting #	S5-226834

# revision of S5-226151

# August 2022, E-meeting

# Source:	Nokia

# Title:	pCR TR28.864 Potential Solution on NWDAF Analytics-related Timing

# Document for:	Approval

# Agenda Item:	6.7.6.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.864-020 “Study on Enhancement of the management aspects related to NWDAF”.

# 3	Rationale

In SA5#145, three requirements for NWDAF related timing were objected with the concern that they were contradictory to the NWDAF specifications. Some of the comments regarding REQ 3 and were not totally correct in the sense that the specs do not state that NWDAF rejects misconfigured request and does not deliver analytics for cases where the computation is delayed. It is, however, the case that the NWDAF specifications do not state that NWDAF has to reject or cannot deliver the analytics if the deadline is reached. Yes, specific implementations may handle it as such, but the specifications do not mandate the NWDAF to not deliver the analytics out of time or to reject immediately a misconfigured request. Relatedly, capturing these events may be useful for network management. A solution is neded that supports all these events.

This pCR is to extend the requirements and add a potential solution on Performance Management of NWDAF Analytics-related Timing.

# 4	Detailed proposal

Start of modification

## 4.7	Key Issue #7: Performance Management of the NWDAF Analytics-related Timing

### 4.7.1	Description

One area for NWDAF performance management is to study NWDAF’s performance in case of errors due to timer expiration. The optional time "Time when analytics information is needed" could be specified during the analytics subscription/request by the consumer. It indicates to the NWDAF the latest time at which the analytics consumer expects to receive the requested analytics data. Meanwhile, the NWDAF has a delay per Analytics ID, called "Supported Analytics Delay", that is specified in the NWDAF profile and indicates the amount of time per per Analytics ID that is expected to lapse between the instant that the NWDAF receives a request for analytics and the time point at which the NWDAF delivers the corresponding analytics report

Ideally, the time "Time when analytics information is needed" should not be set to a value lower than "Supported Analytics Delay". Otherwise, if the time "Time when analytics information is needed" is reached but the analytics information is not ready, the UNSATISFIED_REQUESTED_ANALYTICS_TIME error message might be returned, while if the time "Time when analytics information is needed" is set to a value lower than "Supported Analytics Delay", it is expected that the NWDAF might not be able to treat the Analytics ID on time.

On the other hand, the specifications indicate that NWDAF should trigger ML model training, but the time required for that operation (which can be significantly long under certain circumstances) is not considered in the "Supported Analytics Delay" attribute. In addition, the performance management of NWDAF related to errors generated by expiration of "Time when analytics information is needed" is missing. Thus, there is the need of introducing performance measurement and KPI for performance management of the NWDAF based on this aspect.

The 3GPP management system should introduces the enablers to manage NWDAF performance based on expiration of "Time when analytics information is needed" specified by analytics consumer in the analytics subscription/request. In particular, the 3GPP management system should introduce a mechanism to keep track of analytics production failures due to infringement of timing attributes specified in the analytics subscription/request.

### 4.7.2	Potential requirements

REQ-NWDAF_TIMER-1 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which all the times are appropriately configured, which is the case when the "time within which Analytics is delivered to the consumer" is lower than "Supported Analytics Delay" and the "Supported Analytics Delay" is lower than  the "Time when Analytics information is needed".

REQ-NWDAF_TIMER-2 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which the analytics is delivered after the supported delay but before the deadline, which is the case when the "Supported Analytics Delay" is lower than "time within which Analytics is delivered to the consumer" and "time within which Analytics is delivered to the consumer" is lower than "Time when Analytics information is needed".

REQ-NWDAF_TIMER-3 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which analytics computation is too late. which is the case when "Supported Analytics Delay" is lower than "time within which Analytics is delivered to the consumer".

REQ-NWDAF_TIMER-4 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which the consumer request is misconfigured, which is the case when the "time within which the Analytics is delivered to the consumer", is lower than “Time when Analytics information is needed” but “Time when Analytics information is needed” is itself lower than  the “Supported Analytics Delay

REQ-NWDAF_TIMER-5 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which the analytics is delivered before the supported delay but not by the deadline, analytics deadline is misconfigured configured, which is the case when the “the time when Analytics is delivered to the consumer" is lower than “Supported Analytics Delay" but not lower than “Time when Analytics information is needed”.

REQ-NWDAF_TIMER-6 the 3GPP management system shall have a capability to allow an authorized consumer to configure the NWDAF to track the number of events/scenarios for which the analytics is delivered after both the supported delay and the deadline, but the deadline is misconfigured, which is the case when the “the time when Analytics is delivered to the consumer" is lower “Time when Analytics information is needed” and lower than “Supported Analytics Delay" but , “Supported Analytics Delay" is greater than  the “Time when Analytics information is needed”.

### 4.N.3	Possible solution

Let denote with  the “Supported Analytics Delay”, with TA the “Time when Analytics information is needed” and with TD  the time when the Analytics is delivered to the consumer. Then the following measurements and or counters for NWDAF timing performance may be introduced in one of the specifications TS 28.552, TS 28.554, TS 23.501, TS 23.288:

- NbrAnalSuccNWDAFTimeAn (TD≤DS≤TA): This measurement provides the number of analytics produced in time when “Time when analytics information is needed” timer is correctly sets higher than “Supported Analytics Delay”.

- NbrAnalWarnSuccNWDAFTimeAn (DS<TD≤TA ): This measurement provides the number of analytics produced before “Time when analytics information is needed” timer expiration but with a longer time than “Supported Analytics Delay”.

- NbrAnalFailNWDAFTimeAn( TA expiration : ): This measurement provides the number of analytics failures due to “Time when analytics information is needed” timer expiration.

- NbrAnalSuccNWDAFSuppAnDelay (number of success when TD≤TA< DS ): This measurement provides the number of analytics successes when “Time when analytics information is needed” timer has been set to a value lower than “Supported Analytics Delay”.

- NbrAnalFailNWDAFSuppAnDelay (number of failure when  TA≤DS): This measurement provides the number of analytics failures due to “Time when analytics information is needed” timer expiration when the timer has been set to a value lower than “Supported Analytics Delay”. In this case, the NWDAF produced the analytics with a time lower than “Supported Analytics Delay”.

- NbrAnalFailNWDAFProactTimeAn (number of failure when TA≤DS and NWDAf rejected the request): This measurement provides the number of analytics failures due to proactively reject of analytics request/subscription in case NWDAF predicts that an analytics request/subscription cannot be satisfied within “Time when analytics information is needed” time with high probability.

End of modifications