# 3GPP TSG-SA5 Meeting #146e	S5-226835

# 14 Nov to 18 Nov 2022, E-meeting                                                  revision of S5-226414

# Source:	China Telecom

# Title:	Add the evaluation for KI#3

# Document for:	Approval

# Agenda Item:	6.7.6.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-040: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

In the previous meetings, the key issue #3 for the performance measurement of the NWDAF on the interaction aspect has been studied and some potential solutions have been approved.

In this contribution, we propose to add the evaluation for the key issue #3.

4	Detailed proposal

Start of 1st Change

## 4.3	Key Issue #3: Performance Measurement of the NWDAF on the Interaction Aspect

### 4.3.1	Description

The NWDAF was firstly introduced in Rel-15 and it follows the SBA concept. The interaction between the producer and consumer of the services provided by NWDAF are defined in TS 23.288 [2].

As the requirement on network automation kept increasing, the NWDAF is expected to provide a variety of services to the other NFs and play a very important role in the future. Moreover, in the scenario where multiple NWDAFs are deployed, the NWDAF is also able to act as an Aggregator NWDAF to interact with one to multiple other NWDAFs.

From the perspective of management, it is necessary for the operator to monitor the performance of NWDAF based on the interaction between the producer and consumer of the services provided by the NWDAF. The performance measurements of NWDAF on the interaction aspect can be used to estimate how busy the NWDAF is on specific services. The interaction may include the requests, subscriptions, responses and notifications received and/or generated by NWDAF.

However, in the management domain, the performance measurement of NWDAF on this aspect is missing.

In this key issue, the potential solution(s) is provided on the metrics and KPIs for performance measurement of the NWDAF based on the interaction between the producer and consumer of the analytics services provided by the NWDAF.

Start of 2nd Change

### 4.3.X	Evaluation

In order to measure how busy the NWDAF is on specific services, there are multiple potential solutions proposed for the performance measurement of the NWDAF on the interaction aspect.

In the potential solution #1, the performance measurements of NWDAF on the interaction aspect are provided as the number of subscriptions and/or requests received by NWDAF. In the potential solution #2, the performance measurements of NWDAF on the interaction aspect are provided as the number of notifications and/or responses generated by NWDAF.

In the potential solution #3, the performance measurements of Aggregator NWDAF on the interaction aspect are provided as the number of subscriptions and/or requests invoked by the Aggregator NWDAF to subscribe and/or request the analytics information for analytics aggregation. In the potential solution #4, the performance measurements of Aggregator NWDAF on the interaction aspect are provided as the number of notifications and/or responses received by Aggregator NWDAF to get analytics information for analytics aggregation.

The potential solution #1 and #2 can support the performance measurement of the NWDAF on the interaction aspect for performing the normal task (i.e., the analytics services). The potential solution #3 and #4 can support the performance measurement of the Aggregator NWDAF on the interaction aspect for performing the extra tasks (i.e., subscribe/request the analytics information from the other NWDAFs and aggregate the analytics output) compared to the normal task. These proposed performance measurements on the interaction aspect can indicate how busy the NWDAF is on analytics related services.

Consequently, the potential solutions #1, #2, #3, #4 can provide the performance measurement of the NWDAF on the interaction aspect. These potential solutions are feasible candidates as input to the normative phase on the performance measurement of the NWDAF performing the normal task and extra task.

Editor Notes: The Editor’s note in the potential solution #5 is not removed. So the potential solution #5 can be evaluated after the Editor’s note removed.

End of Modified Sections