# 3GPP TSG-SA5 Meeting #146e	S5-226837

# 14 Nov to 18 Nov 2022, E-meeting                                                  revision of S5-226416

# Source:	China Telecom

# Title:	Add the evaluation for KI#5

# Document for:	Approval

# Agenda Item:	6.7.6.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-040: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

In the previous meetings, the key issue #5 for performance measurement on data collection exposed by NWDAF has been studied, and the potential solution has been approved.

In this contribution, we propose to add the evaluation for the key issue #5.

4	Detailed proposal

Start of 1st Change

4.5	Key Issue #5: Performance Measurement on Data Collection Exposed by NWDAF

4.5.1      Description

Data collection play an important role for the NWDAF providing analytic services and model training services.

In the Rel-17, the enhanced data collection feature is introduced, with which the data collection may be performed via Nnwdaf DataManagement services provided by NWDAF.

The Nnwdaf_DataManagement services enable the consumer to subscribe/unsubscribe, be notified about data exposed by NWDAF, or fetch the subscribed data. The consumer of the Nnwdaf_DataManagement services can be the other NWDAF instances.

The operator may be interested in the how often the Nnwdaf_DataManagement services are requested or how many data are provided via Nnwdaf_DataManagement services.

On one hand, the service provider NWDAF will prepare the data before they are provided to consumer, and since the preparation includes both collection and processing of the data from the other data sources and historical data having been collected by NWDAF, this measurement will indicate how busy the NWDAF is on data collection related services which can be taken as a reflection of the overhead variation of NWDAF. This measurement will also lead to adjusting the network resource allocated to the service provider NWDAF instance to support data exchange with other consumers. And on some other cases, this measurement may indicate that deploying a dedicated DCCF is necessary.

On the other hand, as the enhanced data collection feature is introduced for Rel-17 NWDAF to increases data collection efficiency, this measurement will be a complimentary for the performance measurement of the NWDAFs which does not support DataManagement services. This measurement will also provide information on how many data collection is performed with the more effective manner, which can contribute the potential reduction of the CAPEX.

In this key issue, the potential solutions are provided on the performance measurement about the Nnwdaf_DataManagement services provided by NWDAF.

Start of 2nd Change

4.5.X	Evaluation

In order to measure how busy the NWDAF is on data collection related services, there are multiple potential solutions for the performance measurement on data collection exposed by NWDAF. The data collection may be performed via Nnwdaf DataManagement services provided by NWDAF.

In the potential solution #1, the performance measurements on data collection exposed by NWDAF are provided as the number of Nnwdaf_DataManagement service subscriptions and/or requests received by NWDAF. In the potential solution #2, the performance measurements on data collection exposed by NWDAF are provided as the number of Nnwdaf_DataManagement service notifications and/or responses generated by NWDAF.

These two proposed performance measurements can indicate how often the Nnwdaf_DataManagement services are requested and how busy the NWDAF is on data collection related services.

Consequently, these two potential solutions are feasible candidates as input to the normative phase on the performance measurement on data collection exposed by NWDAF.

End of Modified Sections