3GPP TSG-SA5 Meeting #146 	S5-226844

# Toulouse, France, 14 -18 November 2022											Revision of S5-226467

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0026 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the Triggers for the NEF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1_NEFCH |  |  |  |  |  | Date: |  |  | 2022-11-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The corresponding applicable triggers in the NEF for exposure function northbound APIs converged charging is not specifed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new clause for the applicable Triggers in the NEF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NEF converged charging is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1.x(New), 6.2a.1.2.1, 6.2a.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.4.1.x	Applicable Triggers in the NEF

5.4.1.x.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. identifier of the AF).

Each trigger condition (i.e. chargeable event) defined for the NEF converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.4.1.x.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the NEF when charging is active for the corresponding NEF functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from NEF towards the CHF.

Table 5.4.1.x.1.1: Default Trigger conditions in NEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| API Invocation | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| API Invocation Response | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| API Notification | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| API Notification to NF | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| API Notification acknowledgement | - | Immediate | Not Applicable | Not Applicable | ECUR: Charging Data Request [Termination] |

Next change

6.2a.1.2.1	Charging Data Request message

Table 6.2a.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for NEF converged charging.

Table 6.2a.1.2.1.1: Charging Data Request message contents

| Information Element |  | Category |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| Session Identifier |  | OC |  | Described in TS 32.290 [57] |  |
| Subscriber Identifier |  | OM |  | Described in TS 32.290 [57], and holds the identifier of the AF |  |
| NF Consumer Identification |  | M |  | Described in TS 32.290 [57] |  |
| Invocation Timestamp |  | M |  | Described in TS 32.290 [57] |  |
| Invocation Sequence Number |  | M |  | Described in TS 32.290 [57] |  |
| Retransmission Indicator |  | - |  | This field is not applicable. |  |
| One-time Event |  | OC |  | This field indicates, if included, that this is a one-time event and that there will be no update or termination. |  |
| One-time Event Type |  | OC |  | Described in TS 32.290 [57]. |  |
| Notify URI |  | OC |  | This field contains URI to which notifications are sent by the CHF. The latest received value shall always be used at notifications. |  |
| Triggers |  | OC |  | This field is described in TS 32.290 [57] and holds the NEF specific triggers described in clause 5.4.1.x. |  |
| Multiple Unit Usage |  | OC |  | This field contains the parameters for the quota management request and/or usage reporting. |  |
| Rating Group |  | M |  | Described in TS 32.290 [57] |  |
| Requested Unit |  | OC |  | Described in TS 32.290 [57] |  |
| Used Unit Container |  | OC |  | Described in TS 32.290 [57] |  |
| NEF API Charging Information |  | OM |  | This field holds the NEF API specific information described in clause 6.3.1.4 |  |

Next change

6.2a.3.2	NEF charging CHF CDR data

If enabled, CHF CDRs for NEF charging shall be produced for NEF chargeable events.

The fields of NEF charging CHF CDR are specified in table 6.2a.3.2.1.

Table 6.2a.3.2.1: NEF charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in TS 32.298 [3] |
| Recording Network Function ID | OM | Described in TS 32.298 [3] |
| Subscriber Identifier | M | This field contains the identification of the API user (e.g. SCS Identifier) |
| NF Consumer Information | OC | This field holds the information of the NEF that used the charging service |
| Triggers | OC | This field is described in TS 32.298 [3] and holds the NEF specific triggers described in clause 5.4.1.2. |
| List of Multiple Unit Usage | OM | Described in TS 32.298 [3] |
| Record Opening Time | OC | Described in TS 32.298 [3] |
| Duration | M | Described in TS 32.298 [3] |
| Record Sequence Number | OC | Described in TS 32.298 [3] |
| Cause for Record Closing | M | Described in TS 32.298 [3] |
| Local Record Sequence Number | OM | Described in TS 32.298 [3] |
| Record Extensions | OC | Described in TS 32.298 [3] |
| NEF API Charging Information | OM | This field holds the NEF specific information defined in clause 6.3.1.4 |

End of change