3GPP TSG-SA5 Meeting #146 	S5-226848

Toulouse, France, 14 -18 November 2022											(was S5-226353)

# Source:	MATRIXX Software, Ericsson

# Title:	pCR TR 32.847 Clarify solution #1.6 description

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This pCR is to Clarify solution #1.6 description

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to clarify solution #1.6 description to highlight the NS quota management functionality on top of solution#1.3.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

#### 6.1.6.1	General Description

This solution combines solution#1.3 and parts of solution#1.1 based on Network Slice quota management between the CHF and the NSACF, session based charging and intermediate thresholds.

The general description is the same as clause 6.1.1.1, with the following differences:

- the "CHF max Nb of Reg UEs" correspond to the "max Nb Reg UEs" stored in the CCS;

- the "NSACF max Nb of Reg UEs" correspond to the "max Nb Reg UEs" stored in the NSAC;

- the "NS Reg UEs Quota limit" is always below the "NSACF max Nb of Reg UEs”;

The NSACF is configured at provisioning of the network slice, with “NSACF max Nb of Reg UEs" and at least one intermediate "Nb of Reg UEs" threshold below the "max Nb of Reg UEs" for triggering the CHF.

The CHF is configured at provisioning of the network slice, with "CHF max Nb Reg UEs" which is below or equal to the "NSACF max number of PDU sessions". When the "CHF max Nb Reg UEs" is met the CHF will reject any request from NSACF increasing the Nb Reg UEs.

During the charging session, the CHF may provide new intermediate thresholds which override existing ones, as described in solution #1.3, associated with trigger mechanism in clause 6.1.3.3.

In this solution #1.6, the NSACF may request for NS_quota of "Nb of Reg UEs" to be granted for the S-NSSAI:

- upon crossed threshold triggering initial;

- upon exceeding NS_quota previously supplied by CHF;

NS_quota supplied by CHF may be set to the "NS Reg UEs Quota limit", or to a lower value.

Next change

#### 6.1.6.3	Flow description

The flow and steps are similar as in clause 6.1.3.5 apply with CCS control of NS-quota and "NS Reg UEs Quota limit" as described in figure 6.1.6.3-1

Figure 6.1.6.3-1: NSACF (CTF) – CCS control of NS_quota

Step 0ch: "NS Reg UEs Quota limit" under the "max Nb of Reg UEs" for S-NSSAI, determined by CCS,.

Step 1: NSACF configured with "max Nb of Reg UEs" and a set of intermediate thresholds per S-NSSAI.

Steps 2 to 2ch-a: same as figure 6.1.3.5-1

2ch-b: The NSACF invokes the Nchf_NSConvergedCharging service initial and request for NS_quota of "Nb of Reg UEs" to be granted for the S-NSSAI.

2ch-c. CHF Account and rating control, CHF determines NS_quota to be granted, based on "NS Reg UEs Quota limit".

2ch-d. CHF provides response to NSACF, with possible NS_quota.

2ch-e. NS_quota as new limit for NSACF.

Steps 3 to 3ch-d: same as figure 6.1.3.5-1

4. NS admission control procedure per 3GPP TS 23.502 [11].

4ch-a. NS_quota of "Nb of Reg UEs" will be exceeded with addition of a new UE.

4ch-b: The NSACF invokes the Nchf_NSConvergedCharging service update and request for more NS_quota.

4ch-c. CHF Account and rating control, additional NS_quota for 'Nb of Reg UEs" may be granted  or not by CHF.

4ch-d. CHF provides response to NSACF, with possible new NS_quota, or rejection of additional UE.

4ch-e. if supplied the new NS_quota applies, otherwise (i.e. rejection of additional UE) the previous NS_quota applies.

5. NS admission control procedure per 3GPP TS 23.502 [11].

End of changes