| 3GPP TS 28.317 V0.2.0 (2022-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Self-configuration of Radio Access Network Entities; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Concept and Background	7

4.1	Background	7

4.2            Self-configuration	7

4.3	Self-configuration management	7

5	Management capabilities	7

5.1	Self-configuration management	7

5.1.1	Use cases	7

5.1.2	Requirements	8

6	Stage 2 definition	9

7	Stage 3 definition	9

8	Procedure for Self-establishment	9

Annex <X> (informative): Change history	10

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document describes the functionality for RAN NE self-configuration and ARCF data handling to enable RAN NE self-establishment, as well as requirements need to be met to support this functionality. The concepts, use cases, requirements, procedure and management service definition for RAN NE self-configuration management and ARCF data handling of RAN NEs are specified in this document.

The NE within virtualization is not in the scope of this document.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.314 Management and orchestration; Plug and Connect; Concepts and requirements

…

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

RAN NE Self-Configuration: The process which brings a RAN network element into service requiring minimal human operator intervention or none at all.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

RANSC	RAN NE self-configuration

- Concept and Background

4.1	Background

Establishment of a new RAN NE in network by an autonomous way can greatly improve the efficiency of RAN deployment. It referes to the procedure of a new RAN NE automatically establishing when it is powered up and connects to the IP network, which includes:

-	Plug and connect to management system

-	Self-Configuration

Plug and connect to management system has been specified in TS 28.314 [X]. This document mainly focuses on Self-Configuration.

Editor’s Note: there might be some external/additional data is needed for Self configuration, which is FFS.

4.2 Self-configuration

RAN NE can be taken to a state ready to carry traffic using self-configuration in an automated manner. Self-configuration may include following processes: generate the RAN NE configuration data, download and activate software, download and active configuration data, self-test and update network resource model, etc.

4.3	Self-configuration management

Self-configuration management capability is needed to monitor the self-configuration process and provide the operator with this information (e.g. progress information). In addition, it allows the operator to control the execution of the self-configuration process.

Figure 4.3 Example of deployment scenario for Self-configuration management

- Management capabilities

5.1	Self-configuration management

5.1.1	Use cases

Self-configuration management capability is performed by RANSC MnS producer. RANSC MnS producer monitors the self-configuration process and provides the RANSC MnS consumer with this information (e.g. progress information). In addition, RANSC MnS producer also allows the RANSC MnS consumer to control the execution of the self-configuration process.

The RANSC MnS consumer can request RANSC MnS producer to create and activate a self-configuration process for certain RAN NE in the case of the RANSC MnS consumer trigger the self-configuration process. Besides, RANSC MnS producer also can create and activate a self-configuration process triggered by itself based on the self-configuration management profile (representing the decision of RANSC MnS consumer) configured by RANSC MnS consumer.

As the self-configuration process is complex and time-consuming, the RANSC MnS consumer needs to obtain the progress of the self-configuration process. Self-configuration process includes several steps (each step can represent one or several of activities) according to the self-configuration capabilities. The authorized RANSC MnS consumer may want to be informed the important events for step transition (e.g. start to execute a new step) and abnormal events during the self-configuration process. The RANSC MnS consumer also wants to be informed the reasons when abnormal event (e.g. failure) occurred.

The RANSC MnS consumer may want to request to set the stop point(s) for certain step of the self-configuration process based on self-configuration capability. When a stop point is reached, the self-configuration process is paused and the RANSC MnS consumer is informed with the pause information (incl. corresponding step information). When the RANSC MnS consumer sends a resume request, the self-configuration process will continue to execute to the next step.

During the self-configuration, the RANSC MnS consumers can send request to RANSC MnS producer to query the list of ongoing self-configuration process or the progress for certain self-configuration process.

When the last step of the self-configuration process is completed, RANSC MnS producer needs to send the result of this process to the RANSC MnS consumers. The authorized RANSC MnS consumers can terminate an ongoing self-configuration process for failure analysis and solving. When the last step of the self-configuration process is completed successfully, the RANSC MnS producer can delete the self-configuration process automatically.

5.1.2	Requirements

REQ-RANSC_Mgmt-1:  The RANSC MnS producer shall have the capability to allow the authorized MnS consumers to request to create and activate a self-configuration process.

REQ-RANSC_Mgmt-2:  The RANSC MnS producer shall have the capability to allow the authorized MnS consumers to request to query the list of ongoing Self-configuration processes.

REQ-RANSC_Mgmt-3：The RANSC MnS producer shall have the capability to report the step information of a self-configuration process to the authorized MnS consumers.

REQ-RANSC_CMgmt-4:  The RANSC MnS producer shall have the capability to report abnormal information to the authorized MnS consumers when detected.

REQ-RANSC_Mgmt-5:  The RANSC MnS producer shall have the capability to inform the authorized MnS consumers the result (success or failure) of the self-configuration process when the process is finished.

REQ-RANSC_Mgmt-6:  The RANSC MnS producer shall have the capability to allow the authorized MnS consumers to request to query the progress (e.g. step information) of the self-configuration process when needed.

REQ-RANSC_Mgmt-7：The RANSC MnS producer shall have the capability to allow the authorized MnS consumers to request to terminate an ongoing self-configuration process.

REQ-RANSC_Mgmt-8:  The RANSC MnS producer shall have the capability to inform the authorized MnS consumers the information that it has deleted the self-configuration process automatically.

Editor’s Note: The term “RANSC MnS producer/consumer” need to be revisited later”

- Stage 2 definition

- Stage 3 definition

- Procedure for Self-establishment

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222393 | - | - | - | Initial skeleton | 0.0.0 |
| 2022-04 | SA5#142e | S5-222634 S5-222605 |  |  |  | 1. Add structure 2. Add scope | 0.1.0 |
| 2022-11 | SA5#146 | S5-226807 S5-226811 |  |  |  | 1.pCR 28.317 Add Concept for RANSC 2. PCR 28.317 Usecase and requirement for Self-configuration control and monitor | 0.2.0 |