| 3GPP TR 28.925 V0.9.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on enhancement of service based management architecture (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Intpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Issue investigations and potential issue solutions	8

4.1	Issue #1:  Scope of specifications	8

4.1.1	Description	8

4.1.2	Potential solutions	9

4.1.2.1	Solution #1-1 scope of 28.533 [2]	9

4.1.2.2	Solution #1-2 new SBMA TS for Generic NRM	9

4.1.2.4	Solution #1-4 new SBMA TS for Generic RAN NRM	9

4.2	Issue #2: Content errors	10

4.2.1	Description	10

4.2.2	Potential solutions	10

4.2.2.1	Solution #2-1 3GPP specific information in TSs classified as generic	10

4.2.2.2	Solution #2-2 for stage 1 content in stage 2 TS 28.533 [2]	10

4.3	Issue #3: Reference errors	10

4.3.1	Description	10

4.3.2	Potential solutions	11

4.3.2.1	Solution #3-1 Make not used references void	11

4.4	Issue #4: SBMA supporting manangement of 5G SA and NSA scenarios	11

4.4.1	Description	11

4.4.1.1	Analysis of the existing specification capabilities	11

4.4.1.2	Management support for NG-RAN Overall Architecture	11

4.4.1.3	Management support for EN-DC Overall Architecture	12

4.4.2	Potential solutions #4-1	14

4.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs	14

4.5.1	Description	14

4.5.2	Potential solutions	14

4.5.2.1	ETSI ISG ZSM	14

4.5.2.1.1	Introduction	14

4.5.2.1.2	Potential solutions #5-1	15

4.6	Issue #6: Software management feature in SBMA for 5G	15

4.6.1	Description	15

4.6.2	Potential solutions	16

4.7	Issue #7: Inventory missing in 5G	16

4.7.1	Description	16

4.7.2	Potential solutions	16

4.8	Issue #8: Use of Models in SBMA	16

4.8.1	Description	16

4.8.2	Potential solutions	17

4.9 	Issue #9: Improvement on the management function description	19

4.9.1	Description	19

4.9.2	Potential solutions	20

4.10	Issue #10: Use of 32.401 in SBMA	20

4.10.1	Description	20

4.10.2	Potential solutions	20

4.11	Issue #11: Use of 32.404 in SBMA	20

4.11.1	Description	20

4.11.2	Potential solutions	21

4.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101	21

4.12.1	Description	21

4.12.2	Potential solutions	22

4.13	Issue #13:  Analysis on IRP specification needs to support SBMA	22

4.13.1	Description	22

4.13.2	Potential solutions	23

4.14	Issue #14: Providing a usage guide for SBMA series of specifications	26

4.14.1	Description	26

4.14.2	Potential solutions	26

5	Conclusion and Recommendation	26

5.1	Issue #1 Scope of specifications	26

5.1.1	Solution #1-1 scope of 28.533 [2]	26

5.1.2	Solution #1-2 new SBMA TS for Generic NRM	26

5.1.3	Solution #1-3 scope of referred TSs in 28.533 [2].	26

5.1.4	Solution #1-4 new SBMA TS for Generic RAN NRM	26

5.2	Issue #2 Content errors	26

5.3	Issue #3 Reference errors	26

5.3.1	Solution #3-1 Make not used references void	26

5.4	Issue #4 SBMA supporting management of 5G SA and NSA scenarios	27

5.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs	27

5.6	Issue #6: Software management feature in SBMA for 5G	27

5.7	Issue #7 Inventory missing in 5G	27

5.8	Issue #8: Use of Models in SBMA	27

5.9 	Issue #9: Improvement on the management function description	27

5.10	Issue #10: Use of 32.401 in SBMA	27

5.11	Issue #11: Use of 32.404 in SBMA	27

5.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101	28

5.13	Issue #13:  Analysis on IRP specification needs to support SBMA	28

5.14	Issue #14: Providing overview for SBMA series of specifications	28

Annex A (informative): Change history	29

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This report is to study on enhancement of service based management architecture.

# 1	Scope

The present document studies on the potential enhancement of service based management architecture based on the existing 5G service-based management architecture.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[4]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[5]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[6]	ETSI GS ZSM 002: “Zero-touch network and Service Management (ZSM); Reference Architecture”.

[7]	3GPP TS 28.510: "Configuration Management (CM) for mobile networks that include virtualized network functions; Requirements".

[8]	3GPP TS 28.511: "Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[9]	3GPP TS 28.512: "Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 2".

[10]	3GPP TS 28.513: "Configuration Management (CM) for mobile networks that include virtualized network functions; Stage 3".

[11]	3GPP TS 28.515: "Fault Management (FM) for mobile networks that include virtualized network functions; Requirements".

[12]	3GPP TS 28.516: "Fault Management (FM) for mobile networks that include virtualized network functions; Procedures".

[13]	3GPP TS 28.517: "Fault Management (FM) for mobile networks that include virtualized network functions; Stage 2".

[14]	3GPP TS 28.518: "Fault Management (FM) for mobile networks that include virtualized network functions; Stage 3".

[15]	3GPP TS 28.520: "Performance Management (PM) for mobile networks that include virtualized network functions; Requirements".

[16]	3GPP TS 28.521: "Performance Management (PM) for mobile networks that include virtualized network functions; Procedures".

[17]	3GPP TS 28.522: "Performance Management (PM) for mobile networks that include virtualized network functions; Stage 2".

[18]	3GPP TS 28.523: "Performance Management (PM) for mobile networks that include virtualized network functions; Stage 3".

[19]	3GPP TS 28.525: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[20]	3GPP TS 28.526: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.527: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 2".

[22]	3GPP TS 28.528: "Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Stage 3".

[23]	3GPP TS 28.622: "Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[24]	3GPP TS 32.103: "Integration Reference Point (IRP) overview and usage guide".

[25]	3GPP TS 28.537: "Management capabilities ".

[26]	3GPP TS 32.425: "Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN".

[27]	3GPP TS 28.552: "5G performance measurements".

[28]	3GPP TS 28.500, "Concepts, use cases and requirements".

[29]	3GPP TS 32.111-2: "Telecommunication management; Fault Management; Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[30]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM Information Service (IS)".

[31]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[32]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[33]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[34]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[35]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[36]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[37]	3GPP TS 32.156 "Telecommunication management; Fixed Mobile Convergence (FMC); Model repertoire".

[38]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[39]	3GPP TS 32.404: "Telecommunication management; Performance Management (PM); Performance measurements; Definitions and template".

[40]	3GPP TS 25.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[41]	3GPP TS 25.433 "UTRAN Iub interface Node B Application Part (NBAP) signalling".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSS	Business Support System

MnS	Management Service

PM	Performance Management

SBMA	Service Based Management Architecture

WI	Work Item

# 4	Issue investigations and potential issue solutions

## 4.1	Issue #1:  Scope of specifications

### 4.1.1	Description

The scopes of specifications are not clear.

TS 28.533 [2] (the TS is only valid for 5G and exists in Rel-15,16 and 17):

Scope:

There is no limitation in the scope that the TS is only valid for 5G (or rather not valid for 2G, 3G and legacy 4G.

There are no borders for where the SBMA is valid, which gives some problems when dealing with external entities governed by other fora, e.g. interfaces with MANO, towards verticals, other operators etc. All that control is left to access control, which is not (yet) specified. There is a link with the studies FS_MNSAC and FS_NSCE.

References:

28.510 [7], 28.511 [8], 28.512 [9], 28.513 [10], 28.515 [11], 28.516 [12], 28.517 [13], 28.518 [14], 28.520 [15], 28.521 [16], 28.522 [17], 28.523 [18], 28.525 [19], 28.526 [20], 28.527 [21] and 28.528 [22] are referred, but those TSs are not valid for 5G, They are used in Annex A.4, which is informative.

TS: 28.622 [23] (The TS is valid for 3G, 4G and 5G and exists in Rel-15,16 and 17):

This TS is made for IRP architecture, but is applied also for Service Based Management Architecture, SBMA.

Introduction:

It is stated that the TS is part of a set that is used for management and orchestration of 5G.

Scope:

SBMA is not included according to scope. However, it is nothing is written whether the IRP architecture is valid for 5G. The scope does not say anything about this TS being valid for 5G (which the Introduction does).

The information that the TS is an IRP TS, with the information above in Introduction and Scope a logical conclusion is that IRPs are allowed to be used for 5G. However, the evolution of most IRP TSs has not been in par with the SBMA TSs.

TS: 28.662 [30] (The TS is valid for 3G, 4G and 5G and exists in Rel-15, 16 and 17):

This TS is made for IRP architecture, but is applied also for Service Based Management Architecture, SBMA.

This NRM would be beneficial to be included in SBMA, but an update is needed.

### 4.1.2	Potential solutions

#### 4.1.2.1	Solution #1-1 scope of 28.533 [2]

a.	Introduce the limitation that the TS is not valid for 2G, 3G and 4G in the Scope clause.
b.	Introduce text that 28.533 [2] is valid for SBMA.
c.	Introduce text that access rights for interfaces are outside the scope for this TS (this is be replaced with a TS number when such a TS is published). 
Also to be discussed whether access rights per external (and internal) interfaces are to be documented in some way.

Editor's Note: The internal and external access rights are connected to architectural discussions.

#### 4.1.2.2	Solution #1-2 new SBMA TS for Generic NRM

a.	Replace 28.622 [23] with a new 5G TS for SBMA

b.	Keep 28.622 [23] to be valid for IRP methodology. Move all information about SBMA to a new SBMA TS, i.e. endorse the contribution S5-213040.

Editor's Note: Discuss and agree on whether it is allowed to use IRP methodology in 5G. The result shall be specified in the Scope of IRP and SBMA TSs (e.g. in IRP TSs it is specified that they are valid for 2G, 3G and 4G, while SBMA TSs it is specified that they are valid for 5G. Or it is specified in IRP TSs that they are valid for 2G, 3G, 4G and 5G, while SBMA TSs is specifying that they are valid for 5G).

#### 4.1.2.3	Solution #1-3 scope of referred TSs in 28.533 [2]

The TSs 28.510 [7], 28.511 [8], 28.512 [9], 28.513 [10], 28.515 [11], 28.516 [12], 28.517 [13], 28.518 [14], 28.520 [15], 28.521 [16], 28.522 [17], 28.523 [18], 28.525 [19], 28.526 [20], 28.527 [21] and 28.528 [22] are present in an informative annex so they can be removed.

#### 4.1.2.4	Solution #1-4 new SBMA TS for Generic RAN NRM

Replace 28.662 [30] with a new 5G TS for SBMA with information only applicable to SBMA. New information might be introduced. IRP applicable information is kept in 28.662, which would be valid for older systems using IRP the architecture.

## 4.2	Issue #2: Content errors

### 4.2.1	Description

TS 28.533 [2]:

Content error:

TS 32.103 [24] claims that 28.533 [2] is a "generic" TS (in contrast to "5G specific"), yet clause 4.6 is using 5G entities, clause 5.2 and 5.4 are specifying 3GPP management system interactions with NFV MANO and NWDAF.
Clause 4.2.2: MnS component type A is said to be generic wrt operations and notifications. But is really PM notifications etc. generic?

Clause 6 contains use cases and requirements, which should not be included in a stage 2 TS.

28.533 [2] clams to be the architecture TS for SBMA, but it does not have any boundaries for where it is valid (e.g. is service management and BSS included?).

Implementation errors:

Clause 6: The requirement tag REQ-MnSD-FUN-X has not been given a number.

### 4.2.2	Potential solutions

#### 4.2.2.1	Solution #2-1 3GPP specific information in TSs classified as generic

28.533 [2] clause 4.2.2: Discuss whether operations and notifications used in 5G that are specific for a certain functionality is included in MnS component A which is stated to be generic. The TS is to be changed accordingly to the agreement.

Atl.1	28.533 [2] should be generic, so the not generic content should be deleted, 
Alt.2	28.533 [2] should be 5G specific, so all information about generic and 5G specific TSs should be removed.

#### 4.2.2.2	Solution #2-2 for stage 1 content in stage 2 TS 28.533 [2]

Move clause 6 in 28.533 [2] to 28.537 [25].

Change REQ-MnSD-FUN-X to REQ-MnSD-FUN-1 (but not in 28.533 [2]).

## 4.3	Issue #3: Reference errors

### 4.3.1	Description

TS 28.533 [2]:

32.101 [3] is referenced, but that TS is not valid for 5G. But the reference is only present in clause 2.

32.425 [26] is referenced, but that TS is not valid for 5G (also 28.552 [27] refers to 32.425 [26])

28.500 [28] is referenced, but that TS is not valid for 5G.

28.622 [23] is referenced, but that is an IRP TS (but used for 5G).

TS: 28.622 [23]:

32.111-2 [29] is referenced, but the reference is only present in Clause 2.

32.662 [30] is referenced, but the reference is only present in Clause 2.

### 4.3.2	Potential solutions

#### 4.3.2.1	Solution #3-1 Make not used references void

  - Make reference to 32.101 [3] void in 28.533 [2].

  - Make the references to 32.111-2 [29] and 32.662 [30] void in 28.622 [23].

## 4.4	Issue #4: SBMA supporting manangement of 5G SA and NSA scenarios

### 4.4.1	Description

#### 4.4.1.1	Analysis of the existing specification capabilities

Service based management architecture (SBMA) and corresponding MnS are introduced in 5G management architecture in TS 28.533 [2], and a different management reference model (i.e. IRP) had been used to manage the network before 5G era in 3GPP TS32.101 [3].

The analysis is based on the following understanding of the existing specification capabilities:

- The management mechanism of LTE supports interface IRP and NRM IRP models.

- The management mechanism of 5G supports MnS which includes MnS component A (Operation/Notification), MnS component B (NRM models) and MnS component C (Alarm/Performance information).

- LTE NRM (with enhancement of YAML or YANG solution set) can be used as MnS component type B and work together with MnS component type A.

#### 4.4.1.2	Management support for NG-RAN Overall Architecture

As description in 3GPP TS 38.300 [4], an NG-RAN node is either a gNB or an ng-eNB which are interconnected with each other by means of the Xn interface and connected with 5GC by means of the NG interface, more specifically to the AMF by means of the NG-C interface and to the UPF by means of the NG-U interface. The NG-RAN architecture is introduced in 3GPP TS 38.300 [4], as follows.

Figure 4.4.1.2-1: NG-RAN Overall Architecture

NOTE: The ng-eNB node provides E-UTRA user plane and control plane protocol terminations towards the UE, and connects via the NG interface to the 5GC.

In order to provide management support for NG-RAN, the 3GPP management system needs to support the management for gNB, ng-eNB and 5GC. There are potential 2 management options to support, as follows.

NG-RAN management Option#1

Figure 4.4.1.2-2: NG-RAN management Option#1

In the NG-RAN management Option#1:

- The gNB management domain provides MnS (including MnS component type A, B and C) for the management of gNB.

- The ng-eNB management domain provides IRP (including interface IRP and NRM IRP) for the management of ng-eNB.

- The 5GC management domain provides MnS (including MnS component type A, B and C) for the management of 5GC.

NG-RAN management Option#2

Figure 4.4.1.2-3: NG-RAN management Option#2

In the NG-RAN management Option#2:

- The gNB management domain provides MnS(including MnS component type A, B and C) for the management of gNB

- The ng-eNB management domain provides MnS ((including MnS component type A, B and C) for the management of ng-eNB. In this case, ng-eNB NRM used as MnS component type B, which means the YAML/YANG solution set for ng-eNB needs to be provided.

- The 5GC management domain provides MnS (including MnS component type A, B and C) for the management of 5GC.

#### 4.4.1.3	Management support for EN-DC Overall Architecture

NG-RAN supports Multi-Radio Dual Connectivity (MR-DC) operation whereby a UE in RRC_CONNECTED is connected to two different nodes, one providing NR access and the other one providing either E-UTRA or NR access. One node acts as the MN and the other as the SN. The MN and SN are connected via a network interface and at least the MN is connected to the core network (e.g. EPC).

The following figure illustrates the MR-DC with EPC (i.e. EN-DC) architecture in TS 37.340 [5].

Figure 4.4.1.2-4: EN-DC Overall Architecture

NOTE: the en-gNB node provides NR user plane and control plane protocol terminations towards the UE, and acts as Secondary Node in EN-DC.

In order to provide management support for EN-DC, 3GPP management system needs to provide the management for en-gNB, eNB and EPC. There are also potential 2 management options to support, as follows.

EN-DC management Option#1

Figure 4.4.1.2-5: EN-DC management option#1

In EN-DC management option#1,

- The en-gNB management domain provides MnS (including component type A, B and C) for management of en-gNB.

- The eNB management domain provides IRP (including interface IRP and NRM IRP) for the management of eNB.

- The EPC management domain provides IRP (including interface IRP and NRM IRP) for the management of EPC.

Figure 4.4.1.2-6: EN-DC management Option#2

EN-DC management Option#2

In EN-DC management option#2,

- The en-gNB management domain provides MnS (including component type A, B and C) for management of en-gNB.

- The eNB management domain provides MnS (including component type A, B and C) for management of eNB. In this case, eNB NRM used as MnS component type B, which means the YAML/YANG solution set for eNB NRM needs to be provided.

- The EPC management domain provides MnS (including component type A, B and C) for management of EPC. In this case, EPC NRM used as MnS component type B, which means the YAML/YANG solution set for EPC NRM needs to be provided.

### 4.4.2	Potential solutions #4-1

The above potential 4 management mechanism options described in clause 4.4.1 could be classified as two management options, as follows.

Figure 4.4.2-1: Management Option A and B for 5G SA and NSA

Option#A (including above NG-RAN management Option#1 and EN-DC management Option#1)

- The legacy nodes (e.g. eNB, ng-eNB and EPC) management domain provides IRP (including interface IRP and NRM IRP) for the management of legacy nodes.

- The 5G nodes (e.g. gNB, en-gNB and 5GC) provides MnS (including MnS component type A, B and C) for the management of 5G nodes.

Option#B (including above NG-RAN management Option#2 and EN-DC management Option#2)

- The legacy nodes (e.g. eNB, ng-eNB and EPC) management domain provides MnS (including MnS component type A, B and C) for the management of legacy node. In this case, legacy node NRM used as MnS component type B, which means the YAML/YANG solution set for legacy node NRM needs to be provided.

- The 5G nodes (e.g. gNB, en-gNB and 5GC) provides MnS (including MnS component type A, B and C) for the management of 5G nodes.

## 4.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs

### 4.5.1	Description

There are some relevant works on the architectures and frameworks for automation of management and orchestration in other Standards Developing Organizations (SDOs) in industry. This clause provides a brief overview on the works in the related SDOs.

### 4.5.2	Potential solutions

#### 4.5.2.1	ETSI ISG ZSM

##### 4.5.2.1.1	Introduction

The goal of ETSI ZSM is to enable zero-touch automated network and service management in a multivendor environment.

As documented in ETSI GS ZSM 002 [6], it provides the ZSM framework reference architecture as shown in figure 4.5.2.1.1-1 and distributed management and data services, organized into management domains and integrated via an integration fabric. The integration fabric is used to enable management service consumption, communication, and integration with 3rd party management systems. The cross-domain data service allows data sharing across domains. All management services provide a set of capabilities for their consumption.

Figure 4.5.2.1.1-1: ZSM framework reference architecture

In figure 4.5.2.1.1-1, every management domain, as well as the E2E service management domain, provides a set of ZSM service capabilities by management functions that expose and/or consume set of service end-points. The cross-domain integration fabric facilitates providing capabilities and accessing endpoints cross-domain. Some services are only provided and consumed locally inside the management domain.

##### 4.5.2.1.2	Potential solutions #5-1

3GPP management service deployment framework based on ZSM framework reference architecture is captured in clause 5.3 of TS 28.533 [2]. 3GPP SBMA is aligned with ZSM framework reference architecture.

## 4.6	Issue #6: Software management feature in SBMA for 5G

### 4.6.1	Description

3GPP TS 32.531[31], TS 32.532[32], TS 32.533[33] define the requirements, Information Solution and CORBA solution set for software management which includes Automated software management and non-automated software management. 3GPP TS 28.525 [19] and 28.526[20] describes the management aspects for VNF application software (i.e. 3GPP network function software), which includes reusing the software Management IRP [32] for the interaction between NM and EM. The management of software for VNF itself is not defined in 3GPP, which is in the scope of ETSI NFV.  Software Management IRP [32] is applicable for the deployments using IRP Framework, which is not applicable for the deployments using SBMA.

The 5G management is using the SBMA, so corresponding software management solution is missing.

### 4.6.2	Potential solutions

Introduce a software management feature in SBMA which can be applicable for 5G and future network is needed. The management of software for VNF itself is not included, which is defined by ETSI NFV.

## 4.7	Issue #7: Inventory missing in 5G

### 4.7.1	Description

LTE has specifications about inventory: 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] that define the requirements, Network Resource Model Information Service (IS) and Solution Set. 5G does not have anything similar, even though 5G also consist of software, hardware, network functions and licences.

The attribute table is not follow the latest template documented in TS 32.156 [37], for example, the legal value for “isReadable”, “isWritable”,”isInvarient” and “isNotifyable” should be True or False.

### 4.7.2	Potential solutions

Replace 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] with a new 5G TSs for SBMA.

- Provide the YANG and YAML solution sets for inventory management NRM in SBMA.

## 4.8	Issue #8: Use of Models in SBMA

### 4.8.1	Description

The following existing concepts are related to SBMA:

- TS 28.533 Management Function: A Management Function (MnF) is a logical entity playing the roles of MnS consumer and/or MnS producer. Management Function can be deployed as a separate entity or embedded in Network Function to provide MnS(s).

Figure 4.8-1: Examples of MnS deployment scenario

- TR 21.905 Network Element: A discrete telecommunications entity which can be managed over a specific interface e.g. the RNC.

- TS 28.533 / TS 23.501 Network Function: A 3GPP adopted or 3GPP defined processing function in a network, which has defined functional behaviour and 3GPP defined interfaces.

The following IOCs are specified to represent Network Element and Network Function:

- TS 28.622 ManagedElement IOC: This IOC represents telecommunications equipment or TMN entities within the telecommunications network providing support and/or service to the subscriber. A ManagedElement IOC is used to represent a Network Element defined in TS 32.101[1] including virtualization or non-virtualization scenario.

- TS 28.622 ManagedFunction IOC: This IOC is provided for sub-classing only. It provides attribute(s) that are common to functional IOCs. Note that a ManagedElement may contain several managed functions, a managed function may contain other managed functions as specified for the specific subclass. This IOC can represent a telecommunication function either realized by software running on dedicated hardware or realized by software running on NFVI.

The following IOCs as specified in TS 28.622 are used to represent Management Function provided by MnS Producer:

- TS 28.622 MnsAgent IOC: The MnsAgent represents the MnS producers, incl. the supporting hardware and software, available for a certain management scope that is related to the object name-containing the MnS Agent.

- TS 28.622 ManagementNode IOC: This IOC represents a telecommunications management system (EM) within the TMN that contains functionality for managing a number of ManagedElements (MEs). (Note: This description is to be updated to replace use of EM)

### 4.8.2	Potential solutions

This clause provides the description of using NRM to represent management of management function, network function, and network element.

Management Function could be deployed in following different deployment scenarios:

- The Management Function can be deployed in management system, including

  - The Management Function deployed in domain management system.

  - The Management Function deployed in cross domain management system.

- The Management Function deployed in Network Element.

From the three deployment scenarios of Management Functions above. Management Functions deployed in domain management system and Management Function deployed in Network Element need to be managed as managing network node in 3GPP management system. The way of managing the Management Function are modelled in corresponding management function IOC.

ManagementNode IOC represents a telecommunications management system within the TMN provided by vendor that contains management functionalities for managing a number of ManagedElements (MEs).

The Management Functions are managed by corresponding management function IOC as defined in TS 28.541, TS 28.104, TS 28.105, TS 28.536 and TS 28.312:

- IOC for Management of D-SON function:

- D-SON of ANR in TS 28.541 DANRManagementFunction IOC

- D-SON Energy Saving (ES) functions in TS 28.541 DESManagementFunction IOC

- D-SON RACH functions in TS 28.541 DRACHOptimizationFunction,

- D-SON MRO functions in TS 28.541 DMROFunction,

- D-SON PCI Configuration functions in TS 28.541 DPCIConfigurationFunction,

- D-SON LBO functions in TS 28.541 DLBOFunction,

- IOC for Management of management functions:

- EM centralized SON Energy Saving (ES) functions in TS 28.541 CESManagementFunction IOC

- EM centralized-SON PCI Configuration functions in TS 28.541 CPCIConfigurationFunction,

- EM centralized-SON Energy Saving (ES) functions in TS 28.541 CESManagementFunction,

- EM centralized-SON CCO functions in TS 28.541 CCOFunction

- MDAFunction in TS 28.104

- AIMLTrainingFunction in TS 28.105

- AssuranceClosedControlLoop in TS 28.536

Editor’s note: other management functions to be added with working progress. The criteria for identifying what kind of ManagementFunction should be managed is FFS.

The following table captures the relation between the concepts and related models:

| Concepts | Illustration of Related Management Models |
| --- | --- |
| TS 28.622 ManagementNode | A telecommunications management system within the TMN provided by vendor that contains management functionalities for managing a number of ManagedElements (MEs) are represented by ManagementNode IOC defined in TS 28.622, and a number of management functions. |
| TS 28.533 Management Function | The corresponding XXXManagementFunction IOC is used to support the managing of XXXMnF. |
| TR 21.905 Network Element | A network element is represented by ManagedElement IOC defined in TS 28.622. |
| TS 28.533 / TS 23.501 Network Function | A Network function is represented by ManagedFunction IOC defined in TS 28.622. |

It’s recommended the followingIOCs which could be used to represent MnFs to be managed. The modelling is not changed for existing functions to keep the backward compatibility.

| TS | IOC | Inheritant from | Contained by |
| --- | --- | --- | --- |
| TS 28.541 | - CPCIConfigurationFunction - CESManagementFunction - CCOFunction | Top | - ManagedEntity(SubNetwork/ManagedElement/NRCellDU/ManagementNode) - ManagedEntity(SubNetwork/ManagedElement/NRCellCU/ManagementNode) - SubNetwork/ManagementNode |
| TS 28.104 | MDAFunction | ManagedFunction | MDAEntity (SubNetwork/ManagedElement/ManagedFunction/ManagementNode) |
| TR 28.105 | AIMLTrainingFunction | ManagedFunction | ManagedEntity(Subnetwork/ManagedElement/ManagementFunction/ManagementNode) |
| TS 28.536 | AssuranceClosedControlLoop | Top | SubNetwork/ManagedElement/ManagementNode |

Editors’ Note: whether the MnFs to be managed IOCs are needed to be inherited from a same root IOC is FFS.

## 4.9 	Issue #9: Improvement on the management function description

### 4.9.1	Description

The following information is documented in TS 32.101 [3] to describe the TMN architecture aspects and the management functions.

Analysis:

- TS 32.101 [3] defines the management principles and high-level requirements for the management of PLMNs. Some descriptions in corresponding area in clause 7 of TS 32.101 [3] can be improved and used for SBMA.

2. The following management features are captured in annex of TS 28.533 [2], as follows.

Figure 4.9.1-1: Overview of 5G management specifications

The related management features description in TS 28.533 [2] are limited compared with the management functions description in TS 32.101 [3].

### 4.9.2	Potential solutions

Elaborate the management functional overviews in TS 28.533 [2] with taking into account of the management function description information in TS 32.101 [3] clause 7.

## 4.10	Issue #10: Use of 32.401 in SBMA

### 4.10.1	Description

Concepts and definitions in 32.401 [38] are used in 28.552 [27], Therefore 32.401 [38] is referred to in 28.552 [27].

32.401 [38] is written for an IRP architecture, for which it is still needed. To include SBMA in 32.401 [38] will be almost duplicating the specification.

### 4.10.2	Potential solutions

It is proposed to make

- a new TS for Performance Management (PM); Concept and requirements or

- to include changes into 28.550 [28].

in a new work item in Rel-18.

## 4.11	Issue #11: Use of 32.404 in SBMA

### 4.11.1	Description

Major issues found:

- Concepts, definitions and the template in 32.404 [39] are used in 28.552, therefore 32.404 [39] is referred to in 28.552 [27].

- Some terms (e.g. IRP and NE) used are not valid in SBMA.

- Some normative references are only to IRP TSs, e.g. references to 25.331 [40] and 25.433 [41].

- The PM LCM in Annex B.5 is different than the CM LCM. It should be investigated whether they should be aligned/changed.

### 4.11.2	Potential solutions

It is proposed to update TS 32.404 [39] in a new work item in Rel-18.

## 4.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101

### 4.12.1	Description

In TS 32.101[3], Clause 5.1 illustrates the management reference model which shows the Operations Systems interfacing with other systems. An Operations System supports management interfaces to other systems.

Figure 1: Management reference model

A number of management interfaces in a PLMN are identified in figure 1, namely:

1)	between the Network Elements (NEs) and the Element Manager (EM) of a single PLMN Organisation;

2)	between the Element Manager (EM) and the Network Manager (NM) of a single PLMN Organisation;

NOTE:	In certain cases, the Element Manager functionality may reside in the NE in which case this interface is directly from NE to Network Manager). These management interfaces are given the reference name Itf-N and are the primary target for standardization.

3)	between the Network Managers and the Enterprise Systems of a single PLMN Organisation;

4)	between the Network Managers (NMs) of a single PLMN Organisation;

4a) between the Domain Managers (DMs) of a single PLMN Organisation.

5)	between Enterprise Systems & Network Managers of different PLMN Organisations;

5a) between the Domain Managers (DMs) of different PLMN Organisations.

6)	between Network Elements (NEs).

7)	between the Network Management Layer Service (NMLS) and the Network Manager (NM).

IRPs may be implemented at interfaces 2, 3, 4, 5 and 7.

TS 28.533[2] introduces the Service Based Management Architecture (SBMA). The fundamental building block of the Service Based Management Architecture (SBMA) is the Management Service (MnS). A MnS is a set of offered capabilities for management and orchestration of network and services. An MnS producer offers its services via a standardized service interface composed of individually specified MnS components (MnS component type A, B, C).

Analysis:

- In TS 32.101, there is clearly showing the entities and the corresponding interfaces in the management reference model.

- In TS 28.533, the interaction of paradigm of MnS producer and MnS Consumer is defined without indicating the entities.

- The entities in TS 32.101 can be illustrated using the MnS consumer and MnS producer according to the way of using interface IRPs.

### 4.12.2	Potential solutions

There are the following aspects to be considered in the solution of using MnS in management reference model in TS 32.101.

- The management services may be used to support the interoperability between different entities in TS 32.101 (e.g. Enterprise Systems, NM, DM, EM etc.) in the context of 5G management.

- It’s not possible that one set of management services could apply for the interoperability between different entities. Different set of management services may be used for the interoperabilites between different entities. For example, the set of management services supporting interoperability between EM and NM may be different from the set of management services for interoperability between NM and enterprise systems.

## 4.13	Issue #13:  Analysis on IRP specification needs to support SBMA

### 4.13.1	Description

The overview of interface IRP and NRM IRP are illustrated in clause 5 and clause 6 in TS 32.103[24] as follows:

Figure 4.13-1: 3GPP Interface IRPs

Figure 4.13-2: 3GPP NRM IRPs

Some interface IPR specifications (e.g. Basic CM IRP, Alarm IRP) have been developed to corresponding SBMA specifications (e.g. provisiong MnS, fault supervision MnS), some NRM IRP specifications (e.g. Generic NRM IRP) are reused for SBMA, there are still some IRP specifications haven't produced corresponding SBMA specifications.

### 4.13.2	Potential solutions

Following table 4.13.2-1 provides the overview analysis on 3GPP Interface IRPs and the related SBMA specifications.

| 3GPP Interface IRP specifications |  | Whether correponding SBMA specification is needed | Status |
| --- | --- | --- | --- |
| Generic/Common Interface IRPs | Name convention for Managed Objects (32.300) | Yes | The name convention is used in SBMA. |
|  | Notification IRP (32.301/2/6) | Yes | SBMA specification is already defined (TS 28.532) |
|  | Generic IRP (32.31x) | Not needed as it’s management of interface IRPs |  |
| FM related interface IRPs | Alarm IRP (32.111-x) | Yes | SBMA specification is already defined (TS 28.532) |
|  | Advanced Alarm Management IRP (32.12x) | Yes | Aready covered by FS_FSEV SID |
|  | Test management IRP (32.32x) | TBD |  |
| CM related interface IRPs | Basic CM IRP (32.60x) | Yes | SBMA specification is already defined (TS 28.532) |
|  | Bulk CM IRP (32.61x) | TBD | YANG (RFC 7950) and Netconf (RFC6241) allows updating a single boolean attribute/leaf or the whole configuration. Individual and bulk are not handled separately. |
|  | Kernel CM IRP (32.66x) | TBD | YANG (RFC 7950) and Netconf (RFC6241) allows updating a single boolean attribute/leaf or the whole configuration. Individual and bulk are not handled separately. Part of operations defined in Kernel CM IRP have already a SBMA specification, e.g. notifyMOICreation, notifyMOIDeletion, etc. (cf. TS 28.532). |
| PM file format/collection & Trace related Interface IRPs | Performance Management IRP (32.41x) | Yes | SBMA specification is already defined (TS 28.532 and TS 28.550) |
|  | Performance Measurement collection (32.43x) | Yes | SBMA specification is already defined (TS 28.532 and TS 28.550) |
|  | Trace Management IRP (32.42x, 32.44x) | Yes | Already reused for SBMA |
|  | Notification Log IRP (32.33x) | Yes | Aready covered by FS_eSBMAe SID (see FS_eSBMAe_WoP#7) |
| Supporting interface IRPs | File Transfer IRP (32.34x) | Yes | SBMA specification is already defined (TS 28.537 and TS 28.622) |
|  | Communication Surveillance IRP (32.35x) | Yes | SBMA specification is already defined (TS 28.537,TS 28.622, TS 28.532) |
|  | Entry Point IRP (32.36x) | Yes | SBMA specification is already defined (5GDMS, TS 28.537, TS 28.622) |
| Other interface IRPs | Partial Suspension of Itf-N IRP (32.38x) | TBD |  |
|  | Delta synchronization IRP (32.39x) | TBD |  |
| SON related Interface IRPs | Self-Configuration IRP (32.50x) | Yes | SBMA specification for PnC is already defined (TS 28.314/5/6) SBMA specification for SBMA is covered by RANSC WI |
|  | Software Management IRP (32.53x) | Yes | Covered by Issue #6 in clause 4.6 |

Following table 4.13.2-2 give the overview analysis on  3GPP NRM IRPs to derive which IRP spepcifications needs to be convert to SBMA specifications in the future.

| 3GPP NRM IRP specifications |  | Whether correponding SBMA specification is needed | Status |
| --- | --- | --- | --- |
| FMC model | Umbrella Information Model (28.620) | Yes | Reused for SBMA |
| Common NRM IRP | Generic NRM IRP (28.621/2/3) | Yes | Covered by Issue #4 in clasue 4.1. |
| 3GPP Access Network (AN) related NRM IRPs | Generic RAN NRM IRP (28.661/2/3) | Yes | Covered by Issue #4 in clasue 4.3 |
|  | GERAN NRM IRP (28.654/5/6) | Not applicable |  |
|  | UTRAN NRM IRP (28.651/2/3) | Not applicable |  |
|  | E-UTRAN NRM IRP (28.657/8/9) | Yes | Covered by Issue #4 in clasue 4.3 |
|  | Generic RAN NRM IRP 28.661/2/3) | Yes | There is a dependency from 28.541 to 28.662 |
| 3GPP Core Network (CN)/IMS related NRM IRPs | CN NRM IRP (28.701/2/3) | Not applicable |  |
|  | CN NRM IRP (28.704/5/6) | Not applicable |  |
|  | EPC NRM IRP (28.707/8/9) | Not applicable |  |
| 3GPP H(e)NB related NRM IRPs | HNS NRM IRP (28.671/2/3) | Not applicable |  |
|  | HeNS NRM IRP (28.674/5/6) | Not applicable |  |
| Service management related NRM IRPs | SuM NRM IRP (28.751/2/3) | TBD | Not used for 5G??? |
| Supporting NRM IRPs | Inventory Management NRM IRP (28.631/2/3) | Yes | Covered by Issue #7 in clasue 4.7 and5.4. They are already included in (informative) annex E in TS 28.533 about 5G specifications though still lacking SBMA-compatible solution set |
|  | Transport Network NRM IRP (28.731/2/3) | TBD | ATM is not used in 5G??? Is this correct??? |
|  | Signalling Transport NW IF NRM IRP (28.734/5/6) | TBD | CCS-7 is not used in 5G??? Is this correct??? |
|  | State management data definition IRP (28.624/5&) | Yes | Most of the relevant contents from 28.625 have been copied into 28.622/28.541. |
| Self-Organizing Networks NRM IRPs | SON Policy NRM IRP (28.627/8/9) | Not applicable |  |
| Non-3GPP access interworking NRM IRPs | EPC and non-3GPP access interworking system Network Resource Model (NRM) IRP (28.611/2/6) | Not applicable |  |
| Wireless Local Area Network NRM IRP | WLAN NRM IRP (28.681/2/3) | TBD |  |

## 4.14	Issue #14: Providing a usage guide for SBMA series of specifications

### 4.14.1	Description

TS 32.103 has captured the Telecommunication management; Integration Reference Point (IRP) overview and usage guide for managing of 2G, UMTS and LTE network. The usage guide in TS 32.103 is very useful for external readers to get an overview of how IRP series of specifications.

### 4.14.2	Potential solutions

It is recommended to create a new specification to capture the overview and usage guide information for SBMA series of specifications.

# 5	Conclusion and Recommendation

## 5.1	Issue #1 Scope of specifications

## 5.1.1	Solution #1-1 scope of 28.533 [2]

It is recommended to not pursue the solution#1-1.

## 5.1.2	Solution #1-2 new SBMA TS for Generic NRM

Replace 28.622 [23] with a new 5G TS for SBMA, including stage 1, 2 and 3.

## 5.1.3	Solution #1-3 scope of referred TSs in 28.533 [2].

It is recommended to not pursue the solution#1-3.

## 5.1.4	Solution #1-4 new SBMA TS for Generic RAN NRM

Replace 28.662 [30] with a new 5G TS for SBMA, including stage 1, 2 and 3.

## 5.2	Issue #2 Content errors

## 5.3	Issue #3 Reference errors

## 5.3.1	Solution #3-1 Make not used references void

It is recommended to:

- Make the reference to 32.101 [3] void in 28.533 [2].

as a maintenance CR.

## 5.4	Issue #4 SBMA supporting management of 5G SA and NSA scenarios

Two management options are identified for 5G SA and NSA in section 4.4.2.

It is recommended that the NG-RAN management Option#2 is to be supported by specifying YANG and YAML solution sets in a new TS. It may be an own stage 3 TS or included in the stage 2 TS.

## 5.4	Issue #7 Inventory missing in 5G

Replace 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] with one or several new 5G TSs for SBMA for stage 1, 2 and 3.

Propose a Work Item to include inventory NRM in SBMA.

## 5.5	Issue #5: SBMA supporting management architecture and frameworks in other SDOs

3GPP SBMA is aligned with ZSM framework reference architecture, and related description is captured in TS 28.533[2]. No further modification is needed to existing specifications.

## 5.6	Issue #6: Software management feature in SBMA for 5G

## 5.7	Issue #7 Inventory missing in 5G

Replace 3GPP TS 28.631 [34], 3GPP TS 28.632 [35] and 3GPP TS 28.633 [36] with one or several new 5G TSs for SBMA for stage 1, 2 and 3.

Propose a Work Item to include inventory NRM in SBMA.

## 5.8	Issue #8: Use of Models in SBMA

It’s recommended to add IOC summary in TS 28.533 Annex E (informative): 5G specifications overview to provide the overview of IOC information which currently captured in multiple specifications. The following grouping could be considered as starting inputs:

- IOCs representing network resources as defined in SA2 and RAN3 (e.g. Network Element, Network Function)

- IOCs representing Management of D-SON functions as shown in section 4.8.2.

- IOCs representing Management of management functions as shown in section 4.8.2.

## 5.9 	Issue #9: Improvement on the management function description

## 5.10	Issue #10: Use of 32.401 in SBMA

Replace 32.401 [38] with a new 5G TS for SBMA.

## 5.11	Issue #11: Use of 32.404 in SBMA

Update 32.404 [39] to include SBMA.

## 5.12 	Issue#12: illustration of using MnS in management reference model in TS 32.101

## 5.13	Issue #13:  Analysis on IRP specification needs to support SBMA

## 5.14	Issue #14: Providing overview for SBMA series of specifications

SBMA related management features are currently documented in multiple specifications. An overview including the high level summary description and dependency relationship between multiple specifications could help readers to get full picture of series of SBMA specifications.

It is recommended to create a new specification to capture the overview for SBMA series of specifications.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04-2021 | SA5#137e | S5-213042 |  |  |  | Initial skeleton | 0.0.0 |
| 04-2021 | SA5#137e | S5-213043 |  |  |  | Correction on title | 0.0.1 |
| 05-2021 | SA5#137e | S5-213572 S5-213573 S5-213574 |  |  |  | Update approved tdocs in SA5#137e | 0.1.0 |
| 09-2021 | SA5#138e | S5-214451 S5-214649 |  |  |  | First conclusions for 28.533 First conclusion for NSA | 0.2.0 |
| 10-2021 | SA5#139e | S5-215617 S5-215618 |  |  |  | Update approved tdocs in SA5#139e: Add key issue on software management feature in SBMA for 5G Inventory for 5G | 0.3.0 |
| 11-2021 | SA5#140e | S5-216235 S5-216594 |  |  |  | Add potential solution and conclusion for Issue#5 Add key issue on modelling of MnF | 0.4.0 |
| 04-2022 | SA5#142e | S5-222629 S5-222630 |  |  |  | Add issue on the applicable content from TS 32.101 section 7 Proposal to update 32.401 and 32.404 for SBMA | 0.5.0 |
| 05-2022 | SA5#143e | S5-223579 S5-223581 |  |  |  | Add issue on illustration of using MnS in management reference model in TS 32.101. Add issue on analysis on IRP specifications needs to support SBMA. | 0.6.0 |
| 07-2022 | SA5#144e | S5-224160 |  |  |  | Add conclusion for overview and usage guide for SBMA specifications | 0.7.0 |
| 07-2022 | SA5#145e | S5-225799 S5-225800 S5-225670 |  |  |  | Update of the TR 28.925, due to the study being moved to Rel-18 and new proposals pCR TR28.925 Add conclusion for issue #14 pCR TR28.925 Add description on MnFs to be managed | 0.8.0 |
| 11-2022 | SA5#146 | S5-226236 S5-226963 |  |  |  | rapportuer reorganizing modification for section 5 Add conclusion for issue 4.8 | 0.9.0 |