3GPP TSG SA WG5 Meeting #146		S5-226917

# Toulouse (FR), 14-18 November 2022													Revision of S5-226450

# Source:	Telefónica S.A.

# Title:	pCR 28.908 Clarifying simultaneous and separate execution of training and inference phases

# Document for:	Approval

Agenda Item:	6.7.5.1

# 1	Decision/action requested

Discuss and approve on the proposal.

# 2	References

[1]	3GPP TR 28.908: “Study on Artificial Intelligence / Machine Learning (AI/ML) management”.

[2]	3GPP TS 28.105: “5G: Management and orchestration; Artificial Intelligence/Machine Learning (ML) management”, version 17.1.1, Release 17.

# 3	Rationale

The phases pictured in the Figure 4.2.1-1 are represented in the logical order, which does not always mean they need to execute in sequence at operation time (i.e., for a stage to get started, it is required that the previous one gets ended). The execution of all these phases at operation time is highly impacted by the selected learning method.

In a telco-grade context, it is worth noting that in some learning methods, training and inference can happen at the same time (e.g., in reinforcement learning models), while in other models, training and inference are clearly separated (e.g., in Supervised Learning models).

This contribution aims at clarifying the above ideas.

# 4				Detailed proposal

This document proposes the following changes in TR 28.908 [1].

1st Change

## AI/ML workflow for 5GS

### 4.2.1	AI/ML operational workflow

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN and management system), and the generic workflow of the operational steps in the lifecycle of an AI/ML entity, is depicted in the figure 4.2.1-1.

Figure 4.2.1-1: AI/ML operational workflow

The workflow involves 3 main phases; the training, deployment and inference phase,  including the main operational tasks for each phase. These are briefly described below:

Training phase:

-	AI/ML Training: Learning by the Machine from the training data to generate the (new or updated) AI/ML entity (see TS 28.105 [4]) that could be used for inference. The AI/ML Training may also include the validation of the generated AI/ML entity to evaluate the performance variance of the AI/ML entity when performing on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is not acceptable), the AI/ML entity needs to be re-trained. This is the initial step of the workflow. The AI/ML Training MnS is specified in TS 28.105 [4].

-	AI/ML Testing: Testing of the validated AI/ML entity with testing data to evaluate the performance of the trained AI/ML entity for selection for inference. When the performance of the trained AI/ML entity meets the expectations on both training data and validation data, the AI/ML entity is finally tested to evaluate the performance on testing data. If the testing result meets the expectation, the AI/ML entity may be counted as a candidate for use towards the intended use case or task, otherwise the AI/ML entity may need to be further (re)trained. In some cases, the AI/ML entity may need to be verified which is the special case of testing to check whether it works in the AI-enabled function or the target node. In other cases, the verification step may be skipped, for instance in case the input and  output data, data types and formats, have been unchanged from the last AI/ML entity.

Deployment phase:

-	AI/ML Deployment: Deployment of the trained and tested AI/ML entity to the target inference function which will use the subject AI/ML entity for inference.

NOTE: 	The deployment phase may not be needed in some cases, for example when the training function and inference function are in the same entity.

Inference phase:

-	AI/ML Inference: Performing inference using the AI/ML entity by the inference function.

In telco-grade environments, it is worth noting that the selected learning method (see examples of learning methods  captured in Table 4.1-1 in [2]) can influence on how AI/ML operational workflow executes. In some cases (e.g., when using supervised learning methods), the inference phase can not start until training phase gets ended. In other cases (e.g., when using reinforcement learning methods), the inference phase can start while training phase is still in progress.

End of change