3GPP TSG-SA5 Meeting #146 	S5-226946

# Toulouse, France, 14 -18 November 2022

# Source:	Huawei, CMCC

# Title:	Discussion paper on concepts in FSEV study

# Document for:	Approval

Agenda Item:	6.7.7.1

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	S5-225661: "draft TR 28.830 Fault supervision evolution"; v0.3.0

[3]	TS 32.111-1: "Telecommunication management; Fault Management; Part 1: 3G fault management requirements (Release 17)”

[4]	TS 28.532: “Management and orchestration; Generic management services; (Release 17)”

[5]	S5-225459rev2: "pCR 28.830 Add alarm related definition"; Nokia, Nokia Shangai Bell

[6]	TS 28.104: "Management Data Analytics (MDA)"

[7]	S5-226633: "Rel-18 pCR 28.830 Add alarm related definitions"

[8]	S5-226630: "Rel-18 Input to DraftCR 28.532 Add alarm prediction solution"

[9]		ITU-T Recommendation X.733 (1992) | ISO/IEC 10164-4: 1992, Information technology – Open Systems Interconnection – Systems Management – Alarm reporting function.

3	Rationale

This tdoc discusses some basic concepts related to FSEV, and seeks consensus on the conceptual requirements in FSEV study [2]. The most relevant concepts are event, alarm and fault. Definitions in 3GPP scope are referenced.

Event: Network occurrence which has significance for the management of an NE. Events do not have state [3].

Event notification: Notification used to inform the recipient about the occurrence of an event [3].

Alarm: An alarm signifies an undesired condition of a resource (e.g. network element, link) for which an operator action is required [3].

Alarm notification: Notification used to inform the recipient about the occurrence of an alarm [3].

Fault: A deviation of a system from normal operation, which may result in the loss of operational capabilities of the element or the loss of redundancy in case of a redundant configuration [3].

ADAC Faults: Faults that are "Automatically Detected and Automatically Cleared" by the system when they occur and when they are repaired [3].

ADMC Faults: Faults that are Automatically Detected by the system when they occur and Manually Cleared by the operator when they are repaired [3].

Discussion: Event represents anything that occurs, for example a configuration change, a threshold crossing, an error or a failure. Events do not have state [5]. It is a neutral concept. Alarm indicates undesired condition of a resource. Fault indicates abnormal operation or capability in the network. From fault management perspective, the alarm notification is the explicit information for the MnS consumer to request for operator actions. It may be caused by “alarm” or “fault”.

Proposal 1: Currently the definition of ADAC Faults and ADMC Faults have not been included in the TR and it is proposed to include them to reflect current status. Existing definitions can be reused.

Service failure dection: This is not a terminology or definition currently. The following are described in [9] regarding threshold based warning:

Early detection of faults before significant effects have been felt by the user is a desirable requirement of communicating systems. Degradation of service may be detected by monitoring of error rates. Threshold mechanisms on counters and gauges are a method of detecting such trends and providing a warning to managers when the rate becomes high.

Service failure prediction: This is not a terminology or definition currently. In [6], the requirement REQ-FAILURE_PRED_MDA-03 is as follows: MDA capability for failure prediction shall be able to provide the analytics output including predictions of potential service failures, as well as the possible recommendation actions to prevent failures.

Alarm prediction: This is not a terminology or definition currently. In [8], a solution for the above requirement is added. The proposed alarm prediction items were included in the alarmInformationList etc.

Disccussion: From exiting specifications and requirements, it could be considered that “prediction” is a kind of capability which may need to be introduced.

Proposal 2: It is proposed to introduce the definition of “Service failure prediction” and the predicted results may be included in “MDA report” or alarmInformationList. The following is a potential definition for it.

Service failure prediction: Capability for providing the analytics output including predictions of potential service failures as well as the possible recommendation actions to prevent failures.

4	Detailed proposal

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		ITU-T Recommendation X.731 (1992) | ISO/IEC 10164-2: 1992, Information technology – Open Systems Interconnection – Systems Management – State management function.

[3]		TS 28.625: "State Management Data Definitions"

[4]		ITU-T Recommendation X.733 (1992) | ISO/IEC 10164-4: 1992, Information technology – Open Systems Interconnection – Systems Management – Alarm reporting function.

[5]		TS 28.532: "Generic management services"

[6]		ITU-T Recommendation X.739 (1993), Information technology – Open Systems Interconnection – Systems Management – Metric Objects and attributes.

[7]		ITU-T Recommendation E.880 (1993): Telephone network and ISDN Quality of service, network management and traffic engineering. Field data collection and evaluation on the performance of equipment, networks and services

[8]		TS 28.552: "5G performance measurements"

[9]		TS 28.554: "5G end to end Key Performance Indicators (KPI)"

[10]	TS 28.104: "Management Data Analytics (MDA)"

2nd Change

5.1	Key Issue 1: Missing definitions

5.1.1	Description

TS 28.532, clause 11.2, which defines the Fault Supervision MnS, does not provide the necessary definitions and descriptions required to understand the current state of art as to alarm management. This is because much of the material specified and available for the IRP Framework was not moved to SBMA.

5.1.2	Potential solutions

TS 28.532, clause 11.2 should provide all necessary definitions. This clause provides an (non exhaustive) overview on available definitions.

Event:

SA5 IRP: Network occurrence which has significance for the management of an NE. Events do not have state.

Event notification:

SA5 IRP: Notification used to inform the recipient about the occurrence of an event.

Note TS 28.532 specifies the "notifyEvent" notification, whose definition is: "This notification notifies the MnS consumer, who has a subscription receiving this type of notification, that certain network events have occurred with potential service impact, for example, system restart and system redundancy shift (backup)."

Error:

ITU-T X.733: A deviation of a system from normal operation.

Merriam-Webster: A deficiency or imperfection in structure or function.

RFC 8632: Term not used, or definition not provided.

SA5 IRP: Term not used, or definition not provided.

Failure:

Merriam-Webster: A state of inability to perform a normal function.

Fault:

ITU-T X.733: The physical or algorithmic cause of a malfunction. Faults manifest themselves as errors.

Merriam-Webster: A physical or intellectual imperfection or impairment.

RFC 8632: A fault is the underlying cause of an undesired behavior. There is no trivial one-to-one mapping between faults and alarms. One fault may result in several alarms in case the system lacks root-cause and correlation capabilities. An alarm might not have an underlying fault as a cause. For example, imagine a bad Mean Opinion Score (MOS) alarm from a Voice over IP (VOIP) probe and the cause being non-optimal QoS configuration.

SA5 IRP: A deviation of a system from normal operation, which may result in the loss of operational capabilities of the element or the loss of redundancy in case of a redundant configuration.

ADAC Faults: Faults that are "Automatically Detected and Automatically Cleared" by the system when they occur and when they are repaired.

ADMC Faults: Faults that are Automatically Detected by the system when they occur and Manually Cleared by the operator when they are repaired.

Discussion: The SA5 definition for fault deviates from the other definitions. The other definitions distinguish clearly between the real reason or underlying cause for a malfunction (which they call fault) and the malfunction itself (which they call error). Malfunction means a deviation of a system from normal operation.

SA5 needs to discuss the old definition of the term fault.

Alarm:

ITU-T X.733: A notification, of the form defined by this function, of a specific event. An alarm may or may not represent an error.

Merriam-Webster: A signal (such as a loud noise or flashing light) that warns or alerts.

RFC 8632: An alarm signifies an undesirable state in a resource that requires corrective action.

SA5 IRP: An alarm signifies an undesired condition of a resource (e.g. network element, link) for which an operator action is required. It emphasizes a key requirement that operators (above Itf-N) should not be informed about an undesired condition unless it requires operator action. Use of this emphasis does not exclude this case: In certain context, it is not possible for alarm reporters (below Itf-N) to know whether a particular undesired condition requires operator action or not. In such context, the NM may receive alarms that do not require operator action.

Discussion: Common to all definitions is that an alarm requires operator attention. Except for that there are differences in the definitions. ITU-T X.733 and Merriam-Webster state that an alarm is something that brings some event to the operator attention. RFC 8632 and SA5 IRP state that an alarm signifies an undesired condition. This does not distinguish between the thing that is being reported and the way how the thing is being reported. An alarm, though, should always refer to how something is reported, for example with a notification or a flashing light or a loud noise.

Alarm correlation:

RFC 8632: Dependencies between alarms; several alarms can be grouped as relating to each other, for example, a streaming media alarm relating to a high-jitter alarm.

Root cause analysis:

RFC 8632: An alarm can indicate candidate root-cause resources, for example, a database issue alarm referring to a full-disk partition.

SA5 IRP: Root Cause Analysis is a process that can determine and identify the network condition (e.g. fault, mis configuration) causing the alarms.

Discussion: The SA5 definition is not compliant to what is done in the interface definition. The interface definition clearly refers to alarms as the result of the root cause analysis and not to network conditions.

SA5 needs to discuss the old definition of the term root cause analysis.

Service failure dection: This is not a terminology or definition currently. The following are described in [4] regarding threshold based warning:

Early detection of faults before significant effects have been felt by the user is a desirable requirement of communicating systems. Degradation of service may be detected by monitoring of error rates. Threshold mechanisms on counters and gauges are a method of detecting such trends and providing a warning to managers when the rate becomes high.

Service failure prediction: In [10], the requirement REQ-FAILURE_PRED_MDA-03 is as follows: MDA capability for failure prediction shall be able to provide the analytics output including predictions of potential service failures, as well as the possible recommendation actions to prevent failures.

Disccussion: From exiting specifications and requirements, it could be considered that “prediction” is a kind of capability which need to be introduced.

SA5 needs to discuss the definition of the term “Service failure prediction”. The Potential definition of service failure prediction is provided below:

Proposed new definition:

Editor's note: A new definition is not agreed yet. The following proposal will be subject to further modifications and is just provided here as baseline for further discussions.

Service failure prediction: Capability for providing the analytics output including predictions of potential service failures as well as the possible recommendation actions to prevent failures.

5.1.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

End of change