3GPP TSG-SA5 Meeting #146 	S5-226952

# Toulouse, France,  14-18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | draftCR | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 InputToDraftCR 28.533 on Access control for management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 17-11-2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Contribution is to elaborate the access control aspects towards the management services and aims to bring clarity on authentication and authorisation of the same |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Identity and access control, usecases and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No clarity and harmonisation of access control mechanisms in management systems |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-226544 |  |  |  |  |  |  |  |  |

First change

4.9.x	Identity and Access Control

Network Management systems are becoming a challenge to manage in terms of the users of the system as well as the access control that needs to be applied continuously on a need to know and least privilege basis

Flexibility and agility to adapt to these growing needs is the key for a sustainable identity and access control implementation.

The below depiction indicates some challenges to be considered in Management service(MnS) consumer identification, Management service(MnS) producer and the operational setup of authentication and access control.

Figure 1 - Areas of challenge in access control for management systems

The service based architecture needs to factor seamless integration to any system for authentication of Management Service(MnS) consumers as well as the most secure aspect of access towards the management service producer.

The need to make continuous user and access control related changes by security administrators or users of the system should be supported with ease and simplicity.

Today we have various human users, machine users and various resources that are continuously growing with complexities below:

- Human users seem to require various levels of access control.

- Machine type of communication in the direction towards automated systems require another type of access and may not necessarily be user name password based.

- Resources and modules themselves require various level of clearance depending on the sensitivity of the data being accessed

- Entities have to be centrally managed or have to be managed in a distributed manner

- Scopes and fine-grained granular permissions are very telecom specific and not IT system concerns

The identity and access control system should be designed for role based access control. This makes the possibility to provision with ease from an administrator point of view. Also the authorisation rules can support the fine grained permissions when associated with a role.

### 4.9.x.1    Usecases

#### 4.9.x.1.1 	Role based access control

This is based upon the concept of adding the required permissions and privileges to approved users. The combinations of the permissions and privileges make up a role. The users who belong to a role should be assigned to resources based on a least privilege principle and access rules associated to the resource. The principle of least privileges states that the users should be granted access only to the data and the operations that are required to perform the job. This minimises the possibility of a security breach.

The management system should be setup for access control by a system administrator who will have the know how of creating the required users and roles. Roles will use the various access rules. Additionally, the system administrator will also need to setup the access rules for a MnS producer.

Roles could be in various categories like full access and restricted access. The access could be the CRUD operations and combinations of the same on a MnS producer which contains the resources.

The access rules will be defined on a MnS producer which could be all or combinations of component A, component B and component C.

Figure 2 - role based access

The Figure 2 - role based access shows how a user is assigned to a role. The roles in turn are able to act upon a resource with the required operation on a MnS Producer. The actions that can be performed are defined by the access rules.

### 4.9.x.2    Requirements

UC-MSAC-0x - 3GPPP management system shall support role based access control.

End of changes