3GPP TSG-SA5 Meeting #146 	S5-226970

Toulouse, France, 14 -18 November 2022						revision of S5-226595

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0161 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and update of deallocation use case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case for network slice instance and network slice subnet instance deallocation is not aligned with the stage 2 solution. The goal of the deallocation is to deallocate a service, identified by service profile Id and the last step is to inform the consumer that the deallocation has been completed. The rest of the use case in internal to the producer. The goal is to deallocate a slice identified slice profile Id. The slice requirements are removed from an NSSI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Goal has been rephrased to be deallocation of a service. Final step has been rephrased to inform that deallocation has been completed. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The use case is not correct and and may cause confusion leading to incompatible implementations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3, 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on the procedures |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.3	Network slice instance deallocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To deallocate a network slice instance. |  |
| Actors and Roles | A network slice provisioning management service consumer. |  |
| Telecom resources | Network slice instance Network slice subnet instances A network slice provisioning management service provider. A network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice provisioning management service provider receives the request indicating that an existing NSI is no longer needed to support a particular service (identified by service profile Id). The NSI identification is included in the request. |  |
| Step 1 (M) | Based on the request, the network slice provisioning management service provider deletes the corresponding slice profile and checks if there are other services to be supported by the NSI. If there are none the network slice provisioning management service provider may decide to terminate the NSI; then proceed to Step 2. Otherwise, the network slice provisioning management service provider may decide to trigger to modify the NSI or to do nothing. The use case is completed; go to step 5. | NSI modification use case |
| Step 2 (M) | If the NSI to be terminated is active, the network slice provisioning management service provider de-activates the NSI. Then, the NSI to be terminated is inactive. | NSI de-activation use case |
| Step 3 (M) | The network slice provisioning management service provider identifies the network slice subnet instances used by the NSI, and for every such NSSI sends the request to the corresponding network slice subnet provisioning management service provider (s) indicating that the NSSI(s) are no longer needed for the NSI. The network slice subnet provisioning management service provider (s) may decide to terminate or modify the NSSI(s) based on the request and disassociates them with the NSI. |  |
| Step 4 (M) | The network slice provisioning management service provider receives the response from the network slice subnet provisioning management service provider (s) and terminates the NSI. |  |
| Step 5 (M) | The network slice provisioning management service provider notifies its consumer of the NSI deallocation. |  |
| Ends when | All the steps identified above are successfully completed or skipped per condition in the Step 1. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSI has been deallocated. |  |
| Traceability | REQ-PRO_NSI-FUN-3 |  |

### 5.1.4	Network slice subnet instance deallocation

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To deallocate a network slice subnet instance. |  |
| Actors and Roles | Network slice subnet provisioning management service consumer. |  |
| Telecom resources | Network slice subnet instance Network slice subnet provisioning management service provider. |  |
| Assumptions | N/A |  |
| Pre-conditions | N/A |  |
| Begins when | The network slice subnet provisioning management service provider receives network slice subnet related request from its authorized consumer indicating that an existing NSSI is no longer needed to support a particular set of network slice subnet requirements identified by a slice profile id). The NSSI identification is included in the request. |  |
| Step 1 (M) | Based on the request, the network slice subnet provisioning management service provider decides whether the NSSI should be terminated. If the decision is the NSSI should be terminated, go to the Step 2. If the decision is that the NSSI should not be terminated (e.g., the NSSI is shared or the network slice subnet provisioning management service provider decides to keep the NSSI for later use), the network slice subnet provisioning management service provider disassociates the NSSI from its consumer and provides feedback to the authorized consumer, maybe with removing its consumer’s configuration or not. Go to Step 5. |  |
| Step 2 (M) | If the NSSI consists of constituent NSSIs that are not managed directly by the network slice subnet provisioning management service provider, the network slice subnet provisioning management service provider sends a request to other network slice subnet provisioning management service provider(s) indicating that the constituent NSSIs are no longer needed for the NSSI. |  |
| Step 3 (M) | If the NSSI is associated with NSI, the network slice subnet provisioning management service provider disassociates the NSI from the NSSI to be terminated, and the network slice subnet provisioning management service provider may trigger request to NFVO for terminating or updating (e.g. scaling-in) the NS instance. . (see note). |  |
| Step 4 (M) | If there exists a transport network segment used by the NSSI, the network slice subnet provisioning management service provider may indicate that the transport network segment is no longer needed to support the NSSI. |  |
| Step 5 (M) | The network slice subnet provisioning management service provider sends response to its consumer. |  |
| Ends when | All the steps identified above are successfully completed. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The NSSI has been deallocated. |  |
| Traceability | REQ-PRO_NSSI-FUN-8, REQ-PRO_NSSI-FUN-11 |  |
| NOTE: In case where the NS instance is not dedicated for the NSSI, the network slice subnet provisioning management service provider does not terminate the NS instance. |  |  |

End of changes