3GPP TSG-SA5 Meeting #146 	S5-226978

# Toulouse, France, 14 -18 November 2022	revision of S5-226317

# Source:	Huawei

# Title:	Update VNF package update use case

# Document for:	Approval

Agenda Item:	6.8.5.1

1	Decision/action requested

For approval

2	References

[1]	3GPP TR 28.834 V0.3.0 Study on Management of Cloud Native Virtualized Network Functions

3	Rationale

UC#6 (VNF package update of the cloud-native VNF) only mentions interface between EM and VNFM (IFA008), the interface between 3GPP management system to NFVO (IFA013) also needs to be considered in this use case.

4	Detailed proposal

It is proposed to update scaling use case to support IFA013 interface in clause 5.6 of TR 28.834 V0.3.0 [1].

1st change

5.6	Use case# 6: VNF package update of the cloud-native VNF

5.6.1	Description

As a pre-condition to this use case, the network is running normally. The 3GPP management system has subscribed to VNF instance lifecycle notifications from ETSI NFV MANO.

The use case begins when the 3GPP management system decides to update VNF package for a VNF instance.

The 3GPP management system requests ETSI NFV MANO to change current VNF package (as defined in clause 7.2.24 ETSI GS NFV-IFA 008 [8] for the case where EM communicates with VNFM, and clause 7.3.5 in ETSI NFV-IFA013 V4.3.1 [9] for the case where OSS communicates with NFVO).

ETSI NFV MANO inspects the current and target VNF Descriptor (as defined in clause 7.1.15 ETSI GS NFV-IFA 011[7]) to differentiate possible changing values that concern to some VNF component (e.g. VDU, internal VLD, etc.) or property (e.g. a Scaling Aspect, etc.).

The ETSI NFV MANO will change of both the VNF software version and the VNF virtualised resources , including terminating the virtualised resource instances running the current software version and instantiating new virtualised resource instances with the target VNF software version.

When the VNF software version and the VNF virtualised resources have been changed, ETSI NFV MANO sends a notification to the 3GPP management system to notify that the VNF package has been updated. This notification includes information on the changes to VNF package.

5.6.2		Requirements

REQ-CVNF_LCM_CON-1  The 3GPP management system shall be able to send a request for VNF packager update of the cloud-native VNF to ETSI NFV MANO

REQ-CVNF_LCM_CON-2  The 3GPP management system shall be able to receive notification from ETSI NFV MANO about VNF package update of cloud-native VNF.

End of changes