3GPP TSG-SA5 Meeting #146	S5-227007

Toulouse, France, 14 -18 November 2022

# Source:	China Unicom, HUAWEI

# Title:	Add New Solution for Energy Efficiency of URLLC network slice- RAN based

# Document for:	Approval

Agenda Item:	6.9.2 Study on new aspects of Energy Efficiency (EE) for 5G Phase 2

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	SP-211440: "New Study on new aspects of EE for 5G networks Phase 2"

[2]	3GPP TR 28.913: “Study on new aspects of Energy Efficiency (EE) for 5G Phase 2”

[3]	3GPP TS 28.554: “Management and orchestration; 5G end to end Key Performance Indicators (KPI)”

3	Rationale

In SP-211440 [1], the second objective (“•	On the energy efficiency KPIs”) includes the following sub-objective: “Investigate on the definition of new EE KPIs which consider aspects such as e.g. coverage area, user experience, reliability of URLLC network slice”.

This pCR proposes to a new solution for EE (Energy Efficiency) KPI of URLLC slice based on NG-RAN into TR 28.913 [2].

4	Detailed proposal

This contribution proposes to make the following changes in [2].

1st Change

4.X	Key Issue #<A>: Energy Efficiency KPI of URLLC Network slice – RAN-based

4.X.2	Potential solutions

4.X.2.1	Potential solution: New KPIs for energy efficiency of URLLC network slice – RAN based

4.X.2.1.1	Introduction

In this potential solution, it’s proposed to add new KPIs for energy efficiency of URLLC network slice based on measurements made in NG-RAN. A new KPI is proposed in this solution to show the energy efficiency performance of URLLC network slice in NG-RAN.

4.X.2.1.2	Description

4.X.2.1.2.1	Energy efficiency of URLLC network slice based on NG-RAN with latency and reliability

a) A KPI that shows the energy efficiency of network slices of type URLLC based on NG-RAN with latency and reliability. The Pns for a network slice of type URLLC is the inverse of the average RAN User Plane (UP) latency of the network slice multiplied by the RAN packet transmission reliability of the network slice. In this KPI variant, latency and reliability are the two factors considered for evaluating the performance of network slice.

b)

Where,

PRANonlyURLLC,Latency,Reliability represents the performance of the network slice considering both latency and reliability for evaluating of URLLC.

RelibilityRANonlyURLLC is the reliability of URLLC network slice in RAN side obtained by UL reliability multiplied by DL reliability.

NOTE: The algorithm of downlink reliability and uplink reliability in RAN is FFS.

Where,

NetworkSliceMeanLatency_RAN is the averaged user plane latency of the network slice in RAN.

DRB.AirIfDelayDl is the average delay DL air-interface as defined in TS 28.552.

DRB.AirIfDelayUl is the Average delay UL on over-the-air interface as defined in TS 28.552.

c)

Where,

EERANonlyURLLC,Latency,Reliability is the energy efficiency of network slices of type URLLC based on RAN measurement taking latency and reliability into consideration.

ECRANonly is the energy consumption of the RAN-only network slice over the same observation period.

d) The KPI object is network slice.

PRANonlyURLLC,Latency,Reliability

RelibilityRANonlyURLLC

EERANonlyURLLC,Latency,Reliability

ECRANonly

End of changes