3GPP TSG-SA5 Meeting #146	S5-227008

Toulouse, France, 14 -18 November 2022

# Source:	China Unicom, HUAWEI

# Title:	Add solution for Energy Efficiency of a URLLC network slice based on reliability

# Document for:	Approval

Agenda Item:	6.9.2.1

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.913 v0.3.0: “Study on new aspects of Energy Efficiency (EE) for 5G Phase 2”

[2] 	3GPP TS 28.554 “Management and orchestration;5G end to end Key Performance Indicators (KPI)”

[3]            3GPP TR 28.832 v0.3.0: “Study on management aspects of URLLC”

3	Rationale

As presented in TR 28.913 [1], “Energy Efficiency KPI of URLLC Network Slice based on its Reliability” has been introduced as a key issue. It has been agreed that performance of a URLLC network slice can be its latency and reliability, while existing standardised solution (TS 28.554 [2], clause 6.7.2.3), determines Energy Efficiency (EE) for a URLLC slice only based on Latency performance, reliability performance is not considered. Reliability performance is also an integral part of URLLC slice by nature. Hence, it is important that a variant of Energy Efficiency KPI should also consider reliability performance of a URLLC slice for comprehensive assessment of EE.

This contribution is to propose a potential solution to add a network slice reliability factor to present variants of Energy Efficiency KPI of URLLC Network Slice in TS 28.554 [2].

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

4.6	Key Issue #6: Energy Efficiency KPI of URLLC Network Slice based on its Reliability

4.6.2	Potential solutions

4.6.2.X	Potential solution #X:

4.6.2.X.1	Introduction

In this potential solution #X, it is proposed to define two variants of Energy Efficiency KPI that consider reliability performance of a URLLC slice for comprehensive assessment of EE. Specifically, some of KPI and parameters refer to definitions in TS 28.554. A new KPI is proposed in this contribution.

4.6.2.X.2	         Description

4.6.2.X.2.1	Based on both latency and reliability of the network slice

a) A KPI that shows the energy efficiency of network slices of type URLLC based on both latency and reliability. The Pns for a network slice of type URLLC is the inverse of the average end-to-end User Plane (UP) latency of the network slice multiplied by the end-to-end packet transmission reliability of the network slice [2]. In this KPI variant, latency and reliability are the two factors considered for evaluating the performance of network slice.

b)

PURLLC, Latency,Reliability=Network slice E2E reliabilityNetwork slice mean latency

,where

‘Network slice mean latency’ is defined according to clause 6.7.2.3.2 in TS 28.554

‘Network slice E2E reliability’ is defined as the end-to-end packet transmission reliability of the network slice, and where the end-to-end packet transmission reliability of the network slice from for one S-NSSAI is defined by:

Network slice E2E reliability=DLRelPSR_Uu*DLRelPSR_N3*ULRelPSR_Uu*ULRelPSR_N3

Where DLRelPSR_N3 and ULRelPSR_N3 are defined according to clause 6.8.1 in TS 28.554. Specifically, DLRelPSR_UuULRelPSR_Uu the uplink reliability and downlink reliability for URLLC in RAN need further discussion according to Issue#3 in TR 28.832 [3].

c)

EEURLLC, Latency,Reliability=Network slice E2E  reliabilityNetwork slice mean latency*ECns

d) The KPI object is network slice.

PURLLC,DV, Latency,Reliability=(ωn3*DVN3+ωn9*DVN9)*Network slice E2E  reliabilityNetwork slice mean latency

DVN3DVN9

EEURLLC,DV, Latency,Reliability=(ωn3*DVN3+ωn9*DVN9)*Network slice E2E  reliabilityNetwork slice mean latency*ECNS

End of changes