3GPP TSG-SA5 Meeting #146 	S5-227033

Toulouse, France, 14 -18 November 2022

# Source:	Huawei

# Title:	pCR 28.824 Merge the use cases related to exposed MnS discovery

# Document for:	Approval

Agenda Item:	6.9.6.3

# 1	Decision/action requested

Approval

# 2	References

[1]	3GPP TR 28.824 V0.9.0 Study on network slice management capability exposure

# 3	Rationale

The use cases introduced in clause 5.3 and clause 5.4 of [1] are both related to exposed MnS discovery. The use case exposed MnS discovery service (clause 5.3) focuses on the discovery through BSS, while the use case Exposed MnS support to discovery systems (clause 5.4) emphasizes the involvement of an authorized and supported discovery system. It is proposed to give an updated use case to cover these two aspects of discovery service, no matter the discovery service goes through BSS or OSS and no matter the discovery service is provided by a discovery system or a discovery MnS producer.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

## 5.3	exposed MnS discovery service

### 5.3.1	Description

An exposed MnS should be allowed to register to a trusted discovery service entity (e.g. a discovery MnS producer or an authorized supported discovery system) such that interested authorized consumers (within or external to the operator) are able to discover it. This implies that the exposed MnS is only exposed to a discovery service entity where a pre-existing contract allows for such an exposure.

A use case of exposed MnS discovery service is described as follows:

1. MNO A provides exposed MnSs from 3GPP management system for external customers and these exposed MnSs go through BSS/OSS for exposure.

2. In order to provide such discovery service to external customer, MNO A registers its exposed MnS to a trusted discovery service entity. MNO A configures the exposed MnS with the discovery entity’s address and the exposed MnS is registered at the discovery service entity with the appropriate level of exposure.

3. MNO A performs changes in its management system that impacts the information exposed by the exposed MnS. The changed information is automatically updated in the discovery service entity.

4. In case the relationship between MNO A and a discovery service entity (especially for the discovery system) ends, which implies the discovery service entity is no longer trusted, the exposed MnS automatically requests deletion of its registration at the discovery service entity.

Editor’s Note: How the “is no longer trusted” would be handled by the authentication and authorization and system is FFS, since it is not clear how a non-trust party can be trusted to delete a registration entry.

### 5.3.2	Potential issues and gaps

#### 5.3.2.1	Issues

Study is needed whether the exposed MnS discovery service is to be provided by the MnS discovery service producer, by a dedicated exposed MnS discovery service producer (e.g. EGMF) or by a trusted discovery system.

Study is needed on whether the MnS data as defined in TS 28.533 [x] can also be re-used for exposed MnS data, or if any extensions are necessary.

Study is needed on figuring out differences if the discovery system is external or internal to the operator.

Discovery of service and consumption of a discovered service can be completely separated. However, there may be an issue with managing which consumers have access to the discovery system and could theoretically consume the management service when authorization rules are not respected.

There is an issue with authentication and authorization between the three parties i.e. MnS producer/operator, MnS consumer/customer and discovery system owner.

#### 5.3.2.2	Gaps

To limit issues the exposure from a discovery system of the operator may only provide “read” permissions (w.r.t the exposed MnS) without authentication and authorization. To execute the discovered exposed MnS the consumer still needs to be authenticated and authorized by the management system. Therefore, there is a gap in the difference in exposure for consumption, and exposure for discovery which needs to be solved.

Editor’s Note: Whether or not the issue with a third-party discovery system should be solved only for MnSs or for any 3GPP exposed service is FFS.

End of changes