3GPP TSG-SA5 Meeting #146 	S5-227075

Toulouse, France, 14 -18 November 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0022 | rev | 2 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Energy Saving compensation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE5GPLUS_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The energy saving procedures described in clause 6.2 does not specify the impact on the candidate cell(s) when energy saving is activated in the capacity booster cell in 5G. The concept of ES compensation and compensating for energy saving state are described in TS 32.551 for the legacy technologies. This contribution proposes Energy Saving compensation procedure for 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added to define the Energy Saving compensation procedure for 5G. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Energy Saving compensation procedure is not supported in 5G. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 5.1.x (new), 5.2.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Additional reference: This CR is derived from SA5#145e approved pCR S5-225864. Original S5-224348 rev4 was revised to S5-225864 post approval in #145e meeting. It is re-submitted for SA5#146 with tDoc S5-226059 since it was not implemented into TR28.913 in #145e post approval. S5-226059 is not under discussion for #146 because it was already approved in #145e. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - S5-226783 is a revision of S5-226293: Incorporation of review comments as described below: - The proposed new clause (Requirements for Energy saving compensation activation and deactivation procedures) is now a sub-clause under 5.2.3 instead of 5.2.1. - Updated requirements to remove "network elements" in clause 3.1, 5.2.3.y and 5.1.x.1. - Further incorporation of offline comments: - Comment: • Clause 3.1: I’d recommend not to bring in TS 28.310 the ambiguities that we left in 32.551 (regarding ‘network element’). Therefore, I’d like to suggest to define locally both ‘compensatingForEnergySaving state’ and ‘ES compensation’, by copy/pasting definitions from 32.551 but removing occurrences of ‘network element’ (but keeping ‘network function’ since we have a use case addressing energy saving for the UPF in 28.310) from these definitions. - Updated clause 3.1 to add the descripton for terms "compensatingForEnergySaving state" and "ES compensation". - Comment: Clause 3.1: for consistency, I’d propose to remove all remaining occurrences of ‘network element’ from already existing definitions, i.e. in energySaving state, ES activation, ES deactivation. - Updated terms energySaving state, ES activation, ES deactivation to remove occurrences of ‘network element’. |  |  |  |  |  |  |  |  |
|  |  | - Adding Huawei as co-signing company - S5-227075 is a revision of S5-226783 |  |  |  |  |  |  |  |  |

1st Change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Candidate cell: cell which can provide coverage when the original cell goes into energySaving state.

energySaving state: state in which some functions of a cell or network function are powered-down.

Note 1: 	In energySaving state, the cell or network function is still controllable.

Note 2: 	This is the state when the traffic goes below a certain threshold.

notEnergySaving state: state when no energy saving in progress.

Note 3:	This is the state when the traffic goes above a certain threshold.

ES activation: procedure to power down a cell or network function for energy saving purposes.

Note 4: 	As a result, the subject cell or network function goes into energySaving state.

ES deactivation: procedure to power up a cell or network function.

Note 5: 	As a result, the subject cell or network function goes into notEnergySaving state.

ES Probing procedure: procedure executed by an ES probing capable cell triggered by Cell Activation procedure or ES deactivation procedure.

NOTE 6:	The ES probing procedure [17] assists the decision whether the cell will transfer to notEnergySaving state or remain in energySaving state.

Note 7:	During the ES probing procedure the ES probing capable cell is not carrying traffic, while it can perform measurements and be visible to the UEs in its coverage.

compensatingForEnergySaving state: in an off-peak traffic situation, a cell is remaining powered on, e.g. taking over the coverage areas of neighbor cell in energySaving state..

ES compensation: the procedure to change a cell’s configuration to remain powered on for compensating energy saving activation on other cells, e.g. by increasing a cell’s coverage area. As a result, the cell is in compensatingForEnergySaving state.

2nd Change

### 5.1.x	Energy saving compensation activation and deactivation procedures

#### 5.1.x.1	Introduction

The MnS producer for Domain-centralized ES or the distributed ES function, that makes a decision for the NR capacity booster cell to enter or exit energySaving state, should be able to initiate energy saving compensation activation and/or deactivation on one or multiple cells.

#### 5.1.x.2	Description

For the energy saving use cases (defined in clause 5.1.3), when a NR capacity booster cell enters energySaving state, then the candidate cell(s) may transition to:

-	compensatingForEnergySaving.

Correspondingly, the use cases support the following procedures:

-	Energy saving compensation activation: the procedure to increase the coverage area for the candidate cell(s).

-	Energy saving compensation deactivation: the procedure to decrease a previously increased coverage area.

3rd Change

#### 5.2.3.y	Requirements for Energy saving compensation activation and deactivation procedures

REQ-ESCOL-FUN-y1: The Domain-centralized ES shall support the procedure to initiate energy saving compensation activation to one or multiple cells.

REQ-ESCOL-FUN-y2: The Domain-centralized ES shall support the procedure to initiate energy saving compensation deactivation to one or multiple cells.

REQ-ESCOL-FUN-y3: The distributed ES function shall support the procedure to initiate energy saving compensation activation to one or multiple cells.

REQ-ESCOL-FUN-y4: The distributed ES function shall support the procedure 	to initiate energy saving compensation deactivation to one or multiple cells.

End of change