3GPP TSG-SA5 Meeting #146Bis-e 	S5-231121

Electronic meeting, 16 - 19 January 2023

# Source:	Ericsson

# Title:	Correcting solution #1.11 in clause 7.1

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Changing the heading of the solution to also include that it is for QBC and adding an example.

# 4	Detailed proposal

First change

#### 7.1.4.11	Solution #1.11: QBC triggers common for all QoS flows

##### 7.1.4.11.1	General

A possible solution for key issue #1e covering requirements REQ-CH_VMNO-01, and REQ-CH_VMNO -02, about the QoS flow level trigger setting during the PDU session.

The QBC triggers mechanism is specified in the clause 5.2.1.6 TS 32.255 [4] and Roaming Charging Profile is specified in clause 5.2.1.7 of TS 32.255 [4]. The QBC triggers may include PDU session level triggers and QoS flow level triggers.

##### 7.1.4.11.2	Reference architecture

The reference architecture would be the same as in solution #1.3 clause 7.1.4.3.2.

##### 7.1.4.11.3	Message flows

The message flow is same with the PDU session establishment message flow in clause 5.2.2.12.2 or 5.2.2.18.2 of TS 32.255 [4]. In a Roaming Charging Profile negotiation, a QBC trigger is common for all QFIs and represent the total value of all QoS flows combined for thresholds and limits e.g., if a limit is set to 100 then it represents to the sum of all QoS flows values and will trigger when the sum of all QoS flows (QFIs) reach the value.

End of changes