3GPP TSG-SA5 Meeting #146Bis-e 	S5-231100

Electronic meeting, 16 - 19 January 2023

# Source:	MATRIXX Software

# Title:	pCR TR 32.847 Add evaluation and conclusion for Key Issue #x

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

This pCR is to add evaluation and conclusion for Key Issue #x on SBI between UE CHF and Tenant CHF

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

# 3	Rationale

This pCR is to add evaluation and conclusion for Key Issue #x on SBI between UE CHF and Tenant CHF.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TR 32.847 [1]

First change

### 6.x.4	Evaluation

Both solutions #x.1 and #x.2 can solve Key Issue #x, however with some differences in high level principles.

The solution #x.1 introduces a new paradigm with CHF having two roles regarding the Nchf_ConvergedCharging service: as a "NF service consumer" of the Nchf_ConvergedCharging service (i.e. UE CHF) and as a "NF service provider" of the Nchf_ConvergedCharging service (i.e. NS Tenant CHF).

In the solution #x.1, the Nchf_ConvergedCharging service mixes different levels of charging information (i.e. UE level and NS level of charging information), compared to existing behavior where only one level (either UE level or NS Tenant level) is handled.

The solution #x.2 allows to preserve the current principles of Nchf_ConvergedCharging service, and paves the way for new charging services to be introduced, improving their scope visibility.

Next change

### 6.x.5	Conclusion

It is concluded on solution #x.2 to be normative with a new charging service API between UE CHF and Tenant CHF.

End of change