3GPP TSG-SA5 Meeting #146Bis-e 	S5-231118

Electronic meeting, 16 - 19 January 2023

# Source:	Ericsson

# Title:	Adding solution in clause 7.1 for QBC triggers configured in the V-CHF

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding solution on QBC triggers configured in the V-CHF in clause 7.1 for key issue #1e covering requirement REQ-CH_VMNO-07.

# 4	Detailed proposal

First change

#### 7.1.4.x	Solution #1.x: QBC triggers configured in the V-CHF

##### 7.1.4.x.1	General

A possible solution for key issue #1e covering requirement REQ-CH_VMNO-07, about the FBC and QBC negotiation between visited MNO and home MNO.

For FBC the V-CHF sets the FBC triggers independent of HPLMN, this since the triggers for FBC is not used for wholesale and settlement between visited MNO and home MNO. For the QBC there is configuration done in the V-CHF for the triggers used for wholesale and settlement between visited MNO and home MNO. This configuration is done based on the agreements negotiated between the visited MNO and home MNO and may therefore be configured on a per home MNO basis.

##### 7.1.4.x.2	Reference architecture

The reference architecture would be the same as in solution #1.1 clause 7.1.4.1.2.

##### 7.1.4.x.3	Message flows

Not applicable since the triggers would be part of the configuration of the CHF.

End of changes