3GPP TSG-SA5 Meeting #147	S5-232074

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0397 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modify the criteria for incrementing counters of Intra-GNB Handover Preparation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RadiSys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Handover preparation in the case of Inter-gNB-DU flavor of Intra-gNB-CU handover occurs by triggering UE CONTEXT SETUP procedure. In current specification Intra-GNB Handover Preparation counter gets pegged only when UE Context Modification procedure is triggered. UE Context Modification procedure will be applicable for handover preparation consideration for Intra-gNB-DU handover only. This omits Inter-gNB-DU Mobility scenario within Intra-gNB-CU Mobility as in case of Inter-gNB-DU Mobility scenario handover preparation is triggered by initiating UE Context Setup procedure instead of UE Context Modification procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Criteria for pegging the Intra-gNB handover counters needs to be modified to include UE Context Setup Procedure for Handover Preparation during Inter-DU mobility within Intra-gNB Handover scenario. This change will cover all Intra-gNB Handover Preparation scenarios including both the Intra-gNB-CU with Inter-gNB-DU and the Intra-gNB-CU with Intra-gNB-DU mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Intra-gNB handover KPI counter will partially cover Intra-gNB Handover Preparation scenarios by including only the Intra-gNB-DU mobility during Intra-gNB handover scenarios and omitting the Inter-gNB-DU mobility during Intra-gNB handover scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.7.1.1 to 5.1.3.7.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### Start of changes

###### 5.1.3.7.1.1	Number of requested legacy handover preparations

a)	This measurement provides the number of outgoing intra-gNB legacy handover preparations requested by the source NRCellCU for split gNB deployment.

b)	CC.

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP is sending UE CONTEXT MODIFICATION REQUEST message (see TS 38.473 [6] clause 8.3.4) to gNB-DU or UE CONTEXT SETUP REQUEST message (see TS 38.473 [6] clause 8.3.1) to target gNB-DU to initiate an intra-gNB legacy handover.

d)	A single integer value.

e)	MM.HoPrepIntraReq.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.2	Number of successful legacy handover preparations

a)	This measurement provides the number of successful intra-gNB legacy handover preparations received by the source NRCellCU, for split gNB deployment.

b)	CC

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP receives UE CONTEXT MODIFICATION RESPONSE message (see TS 38.473 [6] clause 8.3.4) from gNB-DU or UE CONTEXT SETUP RESPONSE message (see TS 38.473 [6] clause 8.3.1) from target gNB-DU to initiate a successful intra-gNB legacy handover.

d)	A single integer value.

e)	MM.HoPrepIntraSucc.

f)	NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.3	Number of requested conditional handover preparations

a)	This measurement provides the number of outgoing intra-gNB conditional handover preparations requested by the source NRCellCU for a split gNB deployment.

b)	CC.

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP is sending a UE CONTEXT MODIFICATION REQUEST message (see TS 38.473 [6] clause 8.3.4) to gNB-DU or UE CONTEXT SETUP REQUEST message (see TS 38.473 [6] clause 8.3.1) to target gNB-DU to request resources for an intra-gNB conditional handover.

d)	A single integer value.

e)	MM.ChoPrepIntraReq

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.4	Number of successful conditional handover preparations

a)	This measurement provides the number of successful intra-gNB conditional handover preparations received by the source NRCellCU, for a split gNB deployment.

b)	CC

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP receives a UE CONTEXT MODIFICATION RESPONSE message (see TS 38.473 [6] clause 8.3.4) from gNB-DU or UE CONTEXT SETUP RESPONSE message (see TS 38.473 [6] clause 8.3.1) from target gNB-DU to initiate a successful intra-gNB conditional handover.

d)	A single integer value.

e)	MM.ChoPrepIntraSucc

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.5	Number of requested DAPS handover preparations

a)	This measurement provides the number of outgoing intra-gNB DAPS handover preparations requested by the source NRCellCU for a split gNB deployment.

b)	CC.

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP is sending a UE CONTEXT MODIFICATION REQUEST message (see TS 38.473 [6] clause 8.3.4) to gNB-DU or UE CONTEXT SETUP REQUEST message (see TS 38.473 [6] clause 8.3.1) to target gNB-DU to request resources for an intra-gNB DAPS handover.

d)	A single integer value.

e)	MM.DapsHoPrepIntraReq

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.6	Number of successful DAPS handover preparations

a)	This measurement provides the number of successful intra-gNB DAPS handover preparations received by the source NRCellCU, for a split gNB deployment.

b)	CC

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP receives a UE CONTEXT MODIFICATION RESPONSE message (see TS 38.473 [6] clause 8.3.4) from gNB-DU or UE CONTEXT SETUP RESPONSE message (see TS 38.473 [6] clause 8.3.1) from target gNB-DU to initiate a successful intra-gNB DAPS handover.

d)	A single integer value.

e)	MM.DapsHoPrepIntraSucc

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.7	Number of UEs for which conditional handover preparations are requested

a)	This measurement provides the number of UEs for which outgoing intra-gNB conditional handover preparations are requested by the source NRCellCU for a split gNB deployment.

b)	CC.

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP is sending a UE CONTEXT MODIFICATION REQUEST message (see TS 38.473 [6] clause 8.3.4) to gNB-DU or UE CONTEXT SETUP REQUEST message (see TS 38.473 [6] clause 8.3.1) to target gNB-DU to request resources for an intra-gNB conditional handover. The counter is incremented by 1 for each UE, even if UE CONTEXT MODIFICATION REQUEST or UE CONTEXT SETUP REQUEST messages were sent for several cells.

d)	A single integer value.

e)	MM.ChoPrepIntraReqUes

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

###### 5.1.3.7.1.8	Number of UEs for which conditional handover preparations are successful

a)	This measurement provides the number of UEs for which intra-gNB conditional handover preparations received by the source NRCellCU are successful, for a split gNB deployment.

b)	CC

c)	For split gNB deployment the measurement is triggered and stepped by 1 when gNB-CUCP receives a UE CONTEXT MODIFICATION RESPONSE message (see TS 38.473 [6] clause 8.3.4) from gNB-DU or UE CONTEXT SETUP RESPONSE message (see TS 38.473 [6] clause 8.3.1) from target gNB-DU to initiate a successful intra-gNB conditional handover. The counter is incremented by 1 for each UE, even if UE CONTEXT MODIFICATION RESPONSE or UE CONTEXT SETUP RESPONSE messages were received for several cells.

d)	A single integer value.

e)	MM.ChoPrepIntraSuccUes

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

i)	One usage of this performance measurement is for performance assurance.

#### End of changes