3GPP TSG-SA5 Meeting #147	S5-232566

# Athens, Greece, 27th Feb – 3rd March 2023

# Source:	Alibaba Group, AsiaInfo

# Title:	Add Conclusion for network slice management capability exposure via CAPIF

# Document for:	Approval

Agenda Item:	6.9.6.4

1	Decision/action requested

Include the proposed text in TR 28.824

2	References

[1]		3GPP TR 28.824: "Study on network slice management capability exposure"

3	Rationale

In 23.824, solution 7.9 suggests to conduct network slice management capability exposure via CAPIF. 3 alternatives are proposed considering different ways for role mapping between 3GPP management system and the component of CAPIF. Discussion has been taken in SA5, which is to consider network slice management capability exposure via CAPIF as the direction for normative work. Therefore, this pCR proposes to add Conclusion for network slice management capability exposure via CAPIF.

4	Detailed proposal

First change

8	Conclusion and Recommendation

8.X	Network slice management capability exposure via CAPIF

8.X.1	Conclusion

3 alternatives are given in solution 7.9.

For alternative 1, API provider domain acts as MnS consumer which interacts with the MnS producer within SA5. SA5 jurisdiction ends at MnS Consumer. What MnS consumers does to expose the MnS further to API Invoker (external entity) is out–of-scope of SA5.How the external MnS consumer conduct authentication, authentication, discover the MnS and consume the MnS is not specified in this alternative.

No SA5 standard work is needed for alternative 1 since SA5 jurisdiction ends at the service API domain of CAPIF. However, the requirement in clause 6 are not addressed in a standardized manner.

For alternative 2, MnS producer embeds API provider domain functions capabilities and has direct interaction with CAPIF core function. External MnS consumer can conduct authentication and authorization with CCF. After that, the external MnS consumer can consume the MnS via the service API provided by API provider domain. Besides, CCF can be used as one single exposure proxy for the Network Operator to support exposure related service to different layers of capabilities (e.g. SA5 and SA2).

With certain extension for CAPIF interface, solution on how external MnS consumer discover and consume MnS can be specified by using alternative 2. In order to achieve that, extension of CAPIF interfaces (e.g. CAPIF-1e, CAPIF-2e, CAPIF-3, CAPIF-4, CAPIF-5) is needed.

For alternative 3, MnS producer embeds both CAPIF Core Function and API provider domain functions capabilities. CCF and API provider domain interact with each other via internal interfaces.

With certain extension for CAPIF interface, solution on how external MnS consumer discover and consume MnS can be specified. In order to achieve that, Extension of CAPIF interface (e.g. CAPIF CAPIF-1e, CAPIF-2e) is needed.

So far, there are several gaps regarding, MnS publishing, discovery, which are captured in TR 28.824:

- Whether and how to publish a MnS that can be discovered by external customer is not speficied in existing 3GPP management system.

- If there is a need to publish MnS, then the exposure of MnS (e.g. performance MnS regarding NR and 5GC) is not specified in 3GPP management system.

- To limit issues the exposure from a discovery system of the operator may only provide “read” permissions (w.r.t the exposed MnS) without authentication and authorization. To execute the discovered exposed MnS the consumer still needs to be authenticated and authorized by the management system. Therefore, there is a gap in the difference in exposure for consumption, and exposure for discovery which needs to be solved.

All these gap can be resolved by alternative 2 and 3 since all the related interfaces are within the scope of alternative 2 and 3. Alternative 1 can not solve the gaps mentioned above.

Since alternative 2 can be used to cover more layers of exposure capabilities, it is suggested to use alternative 2 as the baseline for the normative work.

8.X.2	Recommendation

It is suggested to recommend alternative 2 as the baseline for the normative work, which includes:

- Mapping MnS Producers and MnS Consumers with CAPIF entities.

- Defining procedures enabling external consumer to access management services.

-  Mapping MnS with ServiceAPIDefinition in CAPIF.

End of changes