3GPP TSG-SA5 Meeting #147 	S5-232600

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Additional evaluation of solutions for clause 7.2

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding evaluations of solutions for clause 7.2 covering convey charging information from visited MNO to home MNO.

# 4	Detailed proposal

First change

### 7.2.5	Evaluation

Solutions #2.1 and #2.2 both solves key issues #2a and #2b.

-	Solution #2.1 uses a new CHF-to-CHF communication and solves more key issues.

-	Solution #2.2 uses V-SMF to both V-CHF and H-CHF communication and is supported in the scope of 5G connectivity charging.

Allowing the CHF not just to be a producer of the Nchf_ConvergedCharging service but also a consumer would allow for more integration possibilities.

Solutions #2.8 and 2.9 both solves key issues #2c.

-	Solution #2.8 uses NRF to find the H-CHF.

-	Solution #2.9 uses SUPI to find the H-CHF and can supported the case when there is limited H-NRF access.

Both can be supported at the same time depending on the visitor MNO and agreement with home MNO.

Solutions #2.6 and #2.7 both solves key issues #2d.

-	Solution #2.6 reuses the Nchf_ConvergedCharging between CHFs and may add additional charging information to better support CHF-to-CHF communication.

-	Solution #2.7 uses a new Nchf service and would have different messages as well as triggers.

The H-CHF will still do converged charging for the UE in the current scenario, meaning that a new service isn’t required.

Solutions #2.10, #2.11 and #2.12 all solves key issues #2e.

-	Solution #2.10 the trigger for the CHF-to-CHF communication is based on the UE. This means that the V-CHF will have a new session towards the H-CHF that may be independent of the session between the V-SMF and the V-CHF. If the same UE have several sessions towards the same V-CHF this could save some requests, but this would require a completely new way of handling the sessions in both H-CHF and V-CHF.

-	Solution #2.11 the trigger for the CHF-to-CHF communication is based on the reception of a charging data request, this will mean that from the H-CHF point of view the triggers will happen like it would be directly from the V-SMF. The V-CHF would forward some sessions to the H-CHF and would not necessarily have a separate session handling.

-	Solution #2.12 handles how the roaming charging profile could be used in the CHF-to-CHF scenario.

The number of charging sessions triggered simultaneously from the same UE while roaming is limited, this together with that this will limit the way the V-CHF is selected, means that the solution #2.10 isn’t required but a simpler trigger mechanism like in #2.11 can be used.

Solutions #2.1, #2.4, #2.16 all solves key issues #2f and #2g.

-	Solution #2.1 uses a new CHF-to-CHF communication and solves more key issues.

-	Solution #2.4 uses a new SMSF to both V-CHF and H-CHF communication.

-	Solution #2.16 uses Diameter communication between SMS-SC and charging system and is supported in the scope of SMS charging.

Allowing the CHF not just to be a producer of the Nchf_ConvergedCharging service but also a consumer would allow for more integration possibilities.

Solutions #2.1 and #2.5 both solves key issues #2h and #2i.

-	Solution #2.1 uses a new CHF-to-CHF communication and solves more key issues.

-	Solution #2.5 uses AMF to both V-CHF and H-CHF communication and is supported in the scope of 5G connection and mobility charging as is.

Allowing the CHF not just to be a producer of the Nchf_ConvergedCharging service but also a consumer would allow for more integration possibilities.

Editor’s note:	Further evaluations are FFS.

End of changes