3GPP TSG-SA5 Meeting #147 	S5-232599

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Adding solution in clause 7.1 for QoS flow session triggers

# Document for:	Approval

Agenda Item:	7.5.3

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding a solution on QBC with QoS flow session triggers in clause 7.1 for key issue #1f covering requirements REQ-CH_VMNO-08.

# 4	Detailed proposal

First change

#### 7.1.4.y	Solution #1.y: QBC with separate QBC PDU session triggers

##### 7.1.4.y.1	General

A possible solution for key issue #1f covering requirements REQ-CH_VMNO-08 about the PDU session level setting for QBC triggers.

The introduce specific QBC triggers that are common for all QoS flows, corresponding to the PDU session level triggers for FBC, this means that there will be PDU session level triggers for FBC and PDU session level triggers for QBC. Which triggers to have on the QBC PDU session level would be based on requirements.

##### 7.1.4.y.2	Reference architecture

The reference architecture would be the same as in solution #1.3 clause 7.1.4.3.2 or figure 4.2.2 of TS 32.255 [4].

##### 7.1.4.y.3	Message flows

For home routed scenario, the message flow is same with the PDU session establishment message flow in clause 5.2.2.12.2 of TS 32.255 [4]. In step 11ch-d of Figure 5.2.2.12.2.1 in TS 32.255 [4], the H-CHF acknowledges by sending Charging Data Response to the H-SMF and optionally includes a "Roaming Charging Profile". The "Roaming Charging Profile" may include both QoS flow and QBC PDU session level triggers (e.g., both QoS change and QoS flow session limit) for QBC, for FBC both rating group and FBC PDU session level triggers may be included in the trigger element of Charging Data Response.

For local breakout scenario, the message flow is same with the PDU session establishment message flow in clause 5.2.2.18.2 of TS 32.255 [4]. In step 9ch-d2 of Figure 5.2.2.18.2-1 in TS 32.255 [4], the H-CHF acknowledges by sending Charging Data Response to the V-SMF and optionally includes a "Roaming Charging Profile". The "Roaming Charging Profile" may include both QoS flow and QBC PDU session level triggers (e.g., both QoS change and QoS flow session limit) for QBC, for FBC both rating group and FBC PDU session level triggers may be included in the trigger element of Charging Data Response.

For example, a FBC PDU Session level "Expiry of data time limit per PDU session" for FBC is set to 600 seconds while a QBC PDU session level "Expiry of data time limit per QoS flow session" trigger for QBC is set to 900 seconds, then the 600 seconds value will be used for FBC while the 900 seconds value will be used for QBC.

End of changes