3GPP TSG-SA5 Meeting #147	S5-232357

# 27 February - 03 March 2023, Athens, GR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Input to Draft CR] Add recommended actions for non-3GPP operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | MDA may involve operations related to non-3GPP systems. In some cases for MDA output, including e.g., recommendations and/or predictions the 3GPP managmement system may need to take some actions by interacting with a non-3GPP management system, e.g., in case of network slice TN management related issue or to request ETSI NFVO MANO system to scale in or out a VNF instance. Therefore, the MDA MnS producer may include the recommended actions for the non-3GPP operations to the consumer in the MDA output. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the attribute for recommended actions for non-3GPP operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The interaction with non-3GPP system is not supported by MDA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.1, 8.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 8.5.1	RecommendedAction <<dataType>>

#### 8.5.1.1	Definition

This data type specifies the type of recommended action in the analytics output.

#### 8.5.1.2	Information elements

Table 8.5.1.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommended3GPPActions | It contains the recommendations actions concerning 3GPP defined operations on MOIs. | O | type: Recommended3GPPAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedNon3GPPActions | It contains the recommendations actions for non-3GPP operations for interacting with non-3GPP management system. | O | type: RecommendedNon3GPPAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedHumanReadableActions | It contains the recommendations on human readable actions. NOTE: Further details of recommended human readable actions are not specified. | O | type: string multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

Next modified section

### 8.5.x	RecommendedNon3gppAction <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the type of recommended action for non-3GPP operation.

#### 8.5.x.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| OperationName | It specifies the name of the non-3GPP operation in the recommended action. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Parameters | It specifies the parameter names and values for the non-3GPP operation in the recommended action. | M | type: AttributeValuePair (see 32.156 [18]) multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

#### 8.5.x.3	Constraints

None.

End of modified sections