3GPP TSG-SA5 Meeting #147	S5-232695

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0116 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.554 Energy Efficiency KPI of a URLLC network slice based on reliability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BEIJING SAMSUNG TELECOM R&D |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE5GPLUS_Ph2 |  |  |  |  |  | Date: |  |  | 17-02-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The performance of a URLLC network slice can be both , its latency and reliability. Only EE KPI based on latency of URLLC slice as performance is defined in clause 6.7.2.3 of TS 28.554 but the KPI for determining EE of URLLC network slice based on its Reliability as a performance is not defined. In SA5#146 , the solution for defining EE KPI of URLLC slice based on reliability was agreed via pCR S5-227009 which was later included in TR 28.913. So, this CR is for the normative work of same. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It adds a new sub-clause 6.7.2.3.x for defining EE KPI of URLLC network slice based on its reliability. It uses the already existing KPI of reliability in TS 28.554 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be no mean to determine EE KPI of URLLC slice based on its reliability performance and it will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.7.2.3	Energy efficiency of URLLC network slice

##### 6.7.2.3.1	Introduction

This KPI is defined with three variants.

Second change

##### 6.7.2.3.x	Based on Reliability of the network slice

a) EEURLLC,Reliability

b) Energy Efficiency KPI of a URLLC network slice based on its reliability performance is represented as EEURLLC,Reliability. Since generic EE KPI formula of a slice is the ratio of Performance of network slice to the Energy Consumption of network slice hence EEURLLC,Reliability is given as below:

Where, PNS is the performance of a network slice. Here, performance of Network Slice is in terms of its reliability and hence PNS is actually denoted as PURLLC,Reliability. It is calculated for a considered time duration T1. ECNS is the Energy Consumption of the whole slice as specified in TS 28.554 [2] clause 6.7.3.3 and is calculated for the same time duration T1.

Reliability performance of URLLC slice i.e. PURLLC,Reliability can be judged by “Packet Success Rate percentage” (PSR%). The definition of reliability in TS 22.261 [15] says “Reliability is defined in the context of network layer packet transmissions, as percentage value of the packets successfully delivered to a given system entity within the time constraint required by the targeted service out of all the packets transmitted.”.

So, in this case, PURLLC,Reliability is denoted by PURLLC,Reliability,PSR and is defined as:

where:

# PSR% is Packet Success Rate percentage and is calculated over different interfaces and direction (UL/DL) which is explained in sections below;

# X is the total number of packets sent over an interface in a URLLC slice, within the considered time frame T1.

If PURLLC,Reliability,PSR is divided by Energy Consumption of network slice (ECNS) which is measured for the same time period T1, then we get EE KPI i.e. EEURLLC,Reliability. It tells that “with an evaluated reliability (PSR%), how many packets can be successfully sent per Joule of energy in a URLLC slice in a given time frame constraint”.

So, in this case, EE KPI is given by:

PURLLC,Reliability,PSR can be calculated independently for DL and UL directions. With PSR% based approach the EEURLLC,Reliability of a URLLC slice has the unit of packets or bits per Joule. EEURLLC,Reliability can be calculated per interface and per DL and UL direction.

Throughout the slice, the same or different PSR% might exist on different interfaces.

# If it is same, the PSR % (thus reliability) of a slice can be calculated at any one segment of network i.e between UE and gNB or between gNB and UPF;

# In case, if it is not same, the implementations may choose to calculate the PSR% of a URLLC slice at any interface deemed appropriate for the operator e.g. N3 or can consider combined reliability of all the interfaces (e.g. multiplying all interfaces reliability).

c) # Reliability calculation for uplink over Uu interface: PURLLC,Reliability,PSR is obtained for Uu interface by using PSR% calculated in equation below:

PSRUL,Uu = ULRelPSR_Uu.SNSSAI

Where, PSRUL,Uu is equal to ULRelPSR_Uu.SNSSAI which is PSR% in UL for Uu interface per SNSSAI as defined in TS 28.554 [2] clause 6.8.1.2.

# Reliability calculation for downlink over Uu interface: PURLLC,Reliability,PSR is obtained for Uu interface by using corresponding PSR% as calculated in equation below.

PSRDL,Uu = DLRelPSR_Uu.SNSSAI ST1,drbid.SNSSAI

Where, PSRDL,Uu  is equal to DLRelPSR_Uu.SNSSAI which is PSR% in DL for Uu interface per SNSSAI as defined in TS 28.554 [2] clause 6.8.1.1.

# Reliability calculation over N3 interface in uplink: PURLLC,Reliability,PSR is obtained for N3 interface by using PSR%  calculated in equation below. It is based on number of GTP data packets measurement.

PSRUL,N3 = ULRelPSR_N3.SNSSAI

Where, PSRUL,N3 is equal to ULRelPSR_N3.SNSSAI which is PSR% in UL for N3 interface per SNSSAI as defined in TS 28.554 [2] clause 6.8.1.4.

# Reliability calculation over N3 interface in downlink: PURLLC,Reliability,PSR is obtained for N3 interface by using PSR% calculated in equation below. It is based on number of GTP data packets measurement.

PSRDL,N3 = DLRelPSR_N3.SNSSAI

Where, PSRDL,N3 is equal to DLRelPSR_N3.SNSSAI which is PSR% in DL for N3 interface per SNSSAI as defined in TS 28.554 [2] clause 6.8.1.3.

d) NetworkSlice

e) Consider that from above equations, in uplink, RAN domain and Core domain reliability, i.e. PSRUL,Uu  & PSRUL,N3 is known to the operator.

Now operator wants to know if X number of packets are transmitted by UE then out of X, how many packets can be successfully received in UPF i.e. what is its End to End Reliability.

This can be calculated as in equation below:

End to End PSR % = [{(X×PSRUL,Uu) × (PSRUL,N3)} ÷ X] × 100

Suppose X = 1000000 packets, PSRUL,Uu  = 99.98% , PSRUL,N3 = 99.99% , then

End to End PSR % = [{(1000000 × .9998) × (.9999)} ÷ 1000000] × 100

= [{(999800) × (.9999)} ÷ 1000000] × 100

= [{999700} ÷ 1000000] × 100

= [.999700] × 100

= 99.97 %

Explanation of above calculation: out of 1 million packets sent by UE, 999800 packets are successfully received in gNB because reliability between UE and gNB is 99.98%.

Now these 999800 packets will be sent over N3 interface as GTP-U packets towards UPF. Since PSR% between gNB and UPF is 99.99% so based on that, out of 999800 packets sent over N3 interface, 999700 packets are successfully received in UPF. So out of 1 million packets sent by UE, finally 999700 packets are received successfully in UPF, hence E2E PSR% is 99.97 %.

End of changes