3GPP TSG-SA5 Meeting #147	S5-232860

# Athens, Greece, 27th Fe 2023 – 3rd Mar 2023

# Source:	Samsung

# Title:	pCR 28.903 E/WBI related Key Issues.docx

# Document for:	Approval

Agenda Item:	6.9.7.2

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides E/WBI relayed Key Issues and requirements.

# 4	Detailed proposal

First modification

6.b Key Issue#2: EDN Sharing

Once the federation relationship is established, the OPs can share the EDN available in their network with each other. An OP can select some of the EDN from the offered list and register that with the other OP. This will imply that the OP may use the edge resources available in the registered EDN only.

This is crucial to investigate in the method and solutions used for EDN sharing management. It is to be discussed whether it require enhancement to edge NRM and the procedures defined in TS 28.538.

6.b.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall enable sharing of available EDN with other ECSP management system.

REQ-FUN-NBI-1: ECSP management system shall disable sharing of available EDN with other ECSP management system.

6.b.2	 Potential Solutions

TBD

6.c Key Issue#3: Federated EAS resource reservation management

This is intended for an OP to reserve resources for an application provider, with other OP, when the application provider initiate the reservation using NBI. The application provider shall be able to request reservation of resources with partner OP on per location basis. The partner OP shall be able to reserve resources for a given application provider from the allocated quota for the leading OP. The partner OP may have allocated the pool of resources for the leading OP during the EDN registration process. Once the resource reservation request is approved by the partner OP, a resource pool identifier is provided to the leading OP to refer to specific resource pool for application provider. Application provider can use the identifier while instantiating the application to indicate which resource are to be used when deploying applications in partner OP zones.

This is crucial to investigate in the method and solutions used for resource reservation management. It is to be discussed whether it require enhancement to edge NRM and the procedures defined in TS 28.538.

6.c.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability enabling ASP to request reservation of resources with other ECSP management system on per EDN basis.

REQ-FUN-NBI-1: ECSP management system shall have a capability enabling ASP to request deletion and updation of the reservation made with other ECSP management system.

6.c.2 	Potential Solutions

TBD

6.d Key Issue#4: Federated EAS deployment and termination

E/WBI shall control the launch and termination of applications on a partner OP. This will be use by a leading OP to instantiate an application to edge clouds (EDN) of partner OP as requested by application provider over NBI. A leading OP shall make the application instantiation result available on the NBI interface. Partner OP shall also provide the application instance status over E/WBI to leading OP which leading OP may expose to application providers on NBI.

This is crucial to investigate in the method and solutions used for application management on other OPs. It is to be discussed whether it require enhancement to edge NRM and the procedures defined in TS 28.538.

6.d.1 	Requirements

REQ-FUN-NBI-1: ECSP management system shall have a capability enabling ASP to request deployment of EAS on the edge network of other ECSP Management system.

6.d.2 	Potential Solutions

TBD