3GPP TSG-SA5 Meeting #147 	S5-232889

Athens, Greece, 27 February – 3 March 2023

# Source:	Huawei

# Title:	pCR 28.865 Add solultion of network capability preparation

# Document for:	Approval

Agenda Item:	6.9.5.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	3GPP TR 28.865: "Study on deterministic communication service assurance"; v0.3.1

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3"; v18.2.2

[4]	TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; stage 1"; v18.3.0

[5]	TS 22.261: "Service requirements for cyber-physical control applications in vertical domains; stage 1"; v18.8.0

[6]	SP-220981: "New Rel-18 WID Management Data Analytics phase 2"

[7]	TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"; v18.2.0

[8]	TS 28.535: "Management services for communication service assurance; Requirements"; v17.5.0

[9]	TS 28.536: "Management services for communication service assurance; Stage 2 and stage 3"; v17.4.1

[10]	NG.116: "Generic Network Slice Template"; v7.0; 17 June 2022.

3	Rationale

This tdoc addresses the network capability preparation related to DCSA [1, 2]. It contains activities for provisioning of network functions which support the deterministic communication services. For example, to identify what network functions are needed, providing the required network functions and resources. In this tdoc, the reliability aspects are mainly discussed.

To improve reliability of deterministic communication service, the provisioning of redundant transmissions should be supported by the 3GPP management system. Two potential attributes for reliability support are provided in Table 1. NRM of UPF may be impacted to include the attributes in Table 1. Existing performance measurements should be performed for all redundant transmissions.

Table 1

|  | Attribute name | Description |
| --- | --- | --- |
|  | numOfAdditionalLinks | The number of additional transmission links. Redundant transmissions should be configured for high reliability. |
|  | typeOfAdditionalLink | The following types of additional transmission links are supported: e2eand N3/N9: - e2e: Dual Connectivity based end to end Redundant User Plane Paths; - N3/N9: Support of redundant transmission on N3/N9 interfaces; |

Proposal 1: 3GPP management system should provide the reliability supportfor deterministic communication services.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

Editor's note: this clause will contain the issues and potential solutions for deterministic communication service assurance. Relation and potential enhancements to eCOSLA will also be studied for the related issues.

5.X.2	Potential solutions

5.X.2.a	Potential solution #2: Provisioning of redundant transmissions

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for issue#1 at a high-level.

To improve reliability of deterministic communication service, the provisioning of redundant transmissions should be supported by the 3GPP management system.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for issue#1.

Provisioning of network functions for high reliability contains configuration of the types and the corresponding number of redundant links, as depicted in Table 1.

Table 1

|  | numOfAdditionalLinks | The number of additional transmission links. Redundant transmissions should be configured for high reliability. |
| --- | --- | --- |
|  | typeOfAdditionalLink | The following types of additional transmission links are supported: e2e and N3/N9: - e2e: Dual Connectivity based end to end Redundant User Plane Paths; - N3/N9: Support of redundant transmission on N3/N9 interfaces; |

Editor's Note: These attributes will be revisited in the future.

End of change